Alpha Miniloader Howto

David A. Rusling, david.rusling@reo.mts.dec.com

v0.3, 17th October 1995

This document describes the Miniloader, a program for Alpha based sys-

tems that can be used to initialize the machine and load Linux.  The

Alpha Linux Miniloader (to give it it's full name) is also known as

Milo.

1.  What is Milo?

On Intel based PC systems, the BIOS firmware sets up the system and

then loads the image to be run from the boot block of a DOS file

system.  This is more or less what Milo does on an Alpha based system,

however there are several interesting differences between BIOS

firmware and Milo, not least of which is that Milo includes and uses

standard Linux device drivers unmodified.  Milo is firmware, unlike

LILO, which relies on the BIOS firmware to get itself loaded.	 The

main functional parts of Milo are:

1. PALcode,

2. Memory set up code (builds page tables and turns on virtual

addressing),

3. VGA initialisation code (simple ISA VGA or BIOS emulation code),

4. Linux kernel code.	 This includes real Linux kernel code (for

example, the interrupt handling) and ersatz or mock Linux kernel,

5. Linux block device drivers (for example, the floppy driver),

6. File system support (ext2),

7. User inteface code (MILO),

8. Kernel interface code (sets up the HWRPB and memory map for linux),

9. Optional x86 emulator to initialize graphics cards.

The following paragraphs describe these functional parts in more

detail.

PALcode can be thought of as a tiny software layer that tailors the

chip to a particular operating system.  It runs in a special mode

(PALmode) which has certain restrictions but it uses the standard

Alpha instruction set with just five extra instructions.  In this way,

the Alpha chip can run such diverse operating systems as Windows NT,

OpenVMS, Digital Unix and, of course, Linux.	The PALcode that Milo

uses (and therefore Linux itself) is, like the rest of Milo, freeware.

It is derived from Digital's Evaluation Board software example Digital

Unix PALcode..  The differences between the different PALcodes are

because of differences in address mapping and interrupt handling that

exist between the Alpha chips (21066 based systems have a different

I/O map to 21064+2107x systems) and different Alpha based systems.

For Milo to operate properly it needs to know what memory is

available, where Linux will eventually be running from and it must be

able to allocate temporary memory for the Linux device drivers.  The

code maintains a memory map that has entries for permanent and

temporary allocated pages.  It effectively discards itself as it

passes control to Linux and tells Linux just what memory is available.

This means that Milo is very frugal with memory and only reserves

enough memory for the PALcode (8 pages) and the HWRPB and page tables

(3 pages).  This leaves *most* of the memory in the system for Linux

itself.

The final act of the memory code is to set up and turn on virtual

addressing so that the data structures that Linux expects to see are

at the correct place in virtual memory.

You have the choice of two forms of VGA initialisation.  There is

standard, very simple ISA VGA initialisation code which is somewhat

restrictive.	Milo also contains BIOS emulation code which allows it

to run the on-board BIOS from video cards in order to properly

initialise them.

The x86 emulator that Milo uses to execute the BIOS initialization

code found on graphics cards is basically an 8086 emulator with just

enough 386 support to get some of the graphics cards initialized.

Linux device drivers live within the kernel and expect certain

services from the kernel.  Some of these services are provided

directly by Linux kernel code, for example the interrupt handling and

some is provided by kernel look-alike routines.

Milo's most powerful feature is that you can embed unaltered Linux

device drivers into it.  This gives it the potential to support every

device that Linux does.  Milo includes all of the block devices that

are configured into the Linux kernel that it is built against as well

as a lot of the block device code (for example, ll_rw_blk()).

The only supported file system at the moment is the EXT2 filesystem.

This means that Milo can load Linux (or flash update utilities)

directly off of real file systems rather than from boot blocks and

other strange places.	 Gzip'd files are supported and these are

recommended, particularly if you are loading from floppy which is a

rather slow medium.

Built into Milo is a simple keyboard driver which, together with an

equally simple video driver allows it to have a simple user interface.

That interface allows you to list file systems on configured devices,

boot Linux or run flash update utilities and set environment variables

that control the system's booting.  Like LILO, you can pass arguments

to the Kernel.

Milo must tell the Linux kernel what sort of system this is, how much

memory there is and which of that memory is free.  It does this using

the HWRPB (Hardware Restart Parameter Block) data structure and

associated memory cluster descriptions.  These are placed at the

appropriate place in virtual memory just before control is passed to

the Linux kernel.

2.  Pre-Built Standard Milo Images.

If you are planning to run Linux on a standard Alpha based system,

then there are pre-built "standard" Milo images that you might use.

These (along with the sources and other interesting stuff) can be

found on gatekeeper.dec.com in pub/Digital/Linux-Alpha/Miniloader.

The images subdirectory contains a directory per standard system with

Milo images starting with milo and having the following naming

convention:

1. bios - includes BIOS emulation code,

2. floppy - contains only floppy drivers (explicitly *not* SCSI),

3. arc - Windows NT ARC firmware loadable Milo image.

The .floppy images are particularly useful when first powering up a

non-standard system, for example to test a different SCSI driver.  You

could use this Milo image to load Linux and see that the device

drivers work correctly before embedding those drivers into Milo.

Updateflash utilities are all called updateflash and use the same

naming convention as Milo images.

Files ending in .dd are full floppy disk images and should be written

using rawrite.exe on DOS or dd on Linux.  Files ending in .gz are

gzip'd to save space.

3.  How To Build Milo.

You build Milo seperately from the Kernel.  As Milo requires parts of

the kernel to function (for example interrupt handling) you must first

configure and build the kernel that matches with Milo that you want to

build.  Mostly this means building the kernel with the same version

number.  So, miniboot-1.3.31.tgz will build against

linux-1.3.31.tar.gz.	Milo *may* build against a higher version of the

kernel, but there again it may not.  In the remainder of this

discussion, I assume that your kernel sources and object files are

stored in the subtree at /usr/src/linux.

Before you try to build Milo, you should note that you can build Milo

in two basic ways, the first is with vmlinux embedded in it and the

second is with Linux device drivers in it.  I suggest that you read

the section on loading Milo before you answer this question, but

embedding vmlinux into Milo makes the image too large for most

flash/roms.

To build Milo, change your working directory to the milo source

directory and invoke make with:

make KSRC=/usr/src/linux config

Just like the Linux kernel, you will be asked a series of questions

Make a WNT ARC loadable image (MINI_ARC) [n]

Take the default and answer "n" as this option is not yet fully

working and released (it will be).  The next question is:

Include BIOS emulation code (MINI_BIOSEMU) [y]

You should answer yes if you have a video card that is of any type

other than simple ISA VGA.  See the list at the end of this howto to

see if your video card is known to be supported.  The next question

asks you if you want to see Milo output (and input) echoed to the

serial port of your system:

Echo output to the serial port' MINI_SERIAL_ECHO [y]

It's a good idea to echo kernel printk to /dev/ttyS0 if you can.  If

you can (and want to), then type "y", otherwise "n".

You are now free to build Milo.  So go ahead and type:

make KSRC=/usr/src/linux.noname

All being well, all of the images that this target requires will be

built.  The following files are of interest:

milo

The main item on the menu.

milo.rom

A ROMable or FLASHable version of Milo.

updateflash.gz

This is a utility that can be loaded by Milo.  It can update the

flash with a Milo image so that you don't have to boot from

floppy or the network.	This contains a copy of Milo.

mboot, bootm

Used in building an SRM boot block floppy for systems that

support them, such as Noname.

4.  Supported Systems.

Milo has been built and tested on the following systems (if you know

of others than please do mail me):

1. AXPpci33 (aka Noname)

2. EB66+, 266Mhz 21066 based Alpha Evaluation Board,

3. AlphaPC64 (aka Cabriolet), 275Mhz, APECS (2107x) based board and

clones.

The system configurations that I have access to (and therefore tested

with) include the following devices (and if you know of more, please

let me know):

1. Number 9 GXE ISA Video cards and the Number 9 GXE PCI video cards,

2. NCR 8100 SCSI,

3. IDE disks,

4. Standard 14" VGA monitors, Digital VRC16 Monitor,

5. Digital LK450-AA keyboards

The BIOS emulation code is known to work with the following cards:

bus card type:		       chip:   BIOS version:		       Note:

--- --------------------------- ------  ---------------------------     -----

ISA #9 GXE Level 12	       S3 928  #9GXE BIOS Version 1.13.03S

PCI #9 GXE 64 PCI	       S3 864  #9-864 BIOS Version 1.02.09u

PCI Diamond Stealth 64 DRAM PCI S3 864  BIOS Version 1.14	       1

PCI Diamond Video 64 DRAM PCI   S3 868  BIOS Version 1.02	       1

Notes:

------

#1:     Reported by pgwduane@mail.atnet.net.

If you know of any other graphics cards that work, please report your

result to davidm@azstarnet.com (mailto:davidm@azstarnet.com).	 If your

card doesn't work, you may want to report this also but there no

guarantee that the problem will be fixed quickly.

I'm not saying that it will not work with other video cards, however

it does contain basic VGA initialisation code that *should* work with

most ISA cards.

In general, you are advised to pick an S3 based video card as

currently the only Alpha X display driver is for the S3 chipset

(unless you want to run character cell only).

5.  How To Load Milo.

How you do this depends on what system you are trying to load.

Generally, it is a two step process.	First load and test Milo, then

program Milo into flash.  There are several ways of loading Milo, from

a failsafe boot block floppy, from the Windows NT ARC firmware, from

an Alpha Evaluation Board Debug Monitor and from flash or ROM.  Which

of these methods is supported and just how you do it depends on what

system you're running.

5.1.	System Specific Information

5.1.1.  AxpPCI33 (Noname)

The Noname board can load from a failsafe boot block floppy and from

the Windows NT ARC firmware.	A flash update utility, runnable from

Milo is available so that once Milo is running, it can be blown into

flash.  However, be warned that once you have done this you will lose

the previous image held there as there is only room for one image.

The way that Noname boots is controlled by a set of jumpers on the

board, j29 and j28.  These look like:

4

J29     2 x x x 6

1 x x x 5

J28     2 x x x 6

1 x x x 5

3

The two options that we're interested in are J28, pins 1-3 which boots

the console/loader from flash and J29, pins 1-3 which boots the

console/loader from a boot block floppy.  The second option is the one

that you need to first boot Milo on the Noname board.

Once you've selected the boot from floppy option via the jumpers, put

the SRM boot block floppy containing Milo into the floppy and reboot.

In a few seconds (after the floppy light goes out) you should see the

screen blank to white and Milo telling you what's going on.

If you are really interested in technical stuff, the Noname loads

images off of the floppy into physical address 0x104000 and images

from flash into 0x100000.  For this reason, Milo is built with it's

PALcode starting at 0x200000.	 When it is first loaded, it moves

itself to the correct location (see relocate.S).

5.1.2.  EB66+

The EB66+, like all of the Alpha Evaluation Boards built by Digital

contain the Evaluation Board Debug Monitor and so this is available to

load Milo.  Quite often (although not always) boards whose design is

derived from these include the Debug Monitor also.  Usually, the

boards include the Windows NT ARC firmware.

These systems have two boot images in flash controlled by jumpers.  I

usually wire the jumper to the turbo switch (who needs a turbo switch

when you have an Alpha?).

Blowing an image into flash is exactly the same proceedure as for the

AlphaPC64.

5.1.3.  AlphaPC64 (Cabriolet)

The AlphaPC64, like the EB66+, includes two images in flash, the first

is the Evaluation Board Debug Monitor and the second is the Windows NT

ARC firmware.	 You select between these boot options (and Milo when

it's been put into flash) using a combination of jumpers and a boot

option which is saved in the NVRAM of the TOY clock.

The jumper is J2, SP bits 6 and 7 have the following meanings:

o  SP bit 6 should always be out.  If this jumper is set then the SROM

mini-debugger gets booted,

o  SP bit 7 in is boot image selected by the boot option byte in the

TOY clock,

o  SP bit 7 out is boot first image in flash.

So, with bit 7 out, the Debug Monitor will be booted as it is *always*

the first image in flash.  With bit 7 in, the image selected by the

boot option in the TOY clock will be selected.  The Debug Monitor, the

Windows NT ARC firmware and Milo all support setting this boot option

byte but you must be very careful using it.  In particular, you cannot

set the boot option so that next time the system boots Milo when you

are running the Windows NT ARC firmware, it only allows you to set

Debug Monitor or Windows NT ARC as boot options.

To get Milo into flash, you'll need a flashable image.  The build

proceedures make milo.rom by default for systems that support this,

but you can make a rom image using the makerom tool in the Debug

Monitor software that comes with the board:

> makerom -v -i7 -l200000 Milo -o mini.flash

(type makerom to find out what the arguments mean, but 7 is a flash

image id used by the srom and -l200000 gives the load address for the

image as 0x200000).

Load that image into memory (via the Debug Monitor commands flload,

netload, and so on) at 0x200000 and then blow the image into flash:

AlphaPC64> flash 200000 8

(200000 is where the image to be blown is in memory and 8 is the

segment number where you put the image.   There are 16 1024*64 byte

segments in the flash and the Debug Monitor is at seg 0 and the

Windows NT ARC firmware is at seg 4).

Set up the image that the srom will boot by writing the number of the

image into the TOY clock.

AlphaPC64> bootopt 131

(131 means boot the 3rd image, 129 = 1st, 130 = 2nd and so on).

Power off, put jumper 7 on and power on and you should see the Milo

burst into life.  If you don't then take jumper 7 back off and reboot

the Debug Monitor.

5.2.	Loading Milo from the Evaluation Board Debug Monitor

Before you consider this method, you should note that the current

versions of the Evaluation Board Debug Monitor do not include video or

keyboard drivers and so you must be prepared to connect another system

via the serial port so that you can use the Debug Monitor.  It's

interface is very simple and typing help shows a whole heap of

commands.  The ones that are most interesting include the word boot or

load in them.

The Evaluation Board Debug Monitor can load an image either via the

network (netboot) or via a floppy (flboot).  In either case, set the

boot address to 0x200000 (> bootadr 200000) before booting the image.

If the image is on floppy (and note that only DOS formatted floppies

are supported), then you will need to type the following command:

AlphaPC64> flboot <Milo-image-name>

5.3.	Loading Milo from a Failsafe Boot Block Floppy

If you do not have a standard pre-built Milo .dd image, then you may

need to build an SRM boot block floppy.  Once you have built Milo, you

need to do the following on Digital Unix box:

fddisk -fmt /dev/rfd0a

cat mboot bootm > /dev/rfd0a

disklabel -rw rfd0a 'rx23' mboot bootm

Or on a Linux box:

cat mboot bootm > /dev/fd0

If you have a standard Milo image available (say milo.bios.dd) then

you would build a boot block floppy using the following command:

dd if=mini.bios.dd of=/dev/fd0

As well as the boot block floppy you will need the following:

1. vmlinux on an EXT2 format floppy, SCSI or IDE disk (gzip'd is

better),

2. updatefloppy.gz on an EXT2 format floppy disk (if you want to write

Milo into flash).

5.4.	Loading Milo from Flash

This is always system specific.

5.5.	Loading Milo from the Windows NT ARC firmware

The Windows NT ARC firmware is an environment in which programs can

run and make callbacks into the firmware to achieve actions.	The

Windows NT OSLoader is a program that does exactly this.   Linload.exe

is a much simpler program which does just enough to load and execute

the Alpha Linux Miniloader (MILO).  It loads the appropriate image

file into memory at 0x00000000 and then makes a swap-PAL PALcall to

it.  Milo relocates itself to 0x200000 and continues on through the

PALcode reset entry point as before.

By default (and design) the ARC loadable Milo built in the standard

distribution tree contains no video or keyboard initialisation code.

This is because this has already been carried out by the Windows NT

ARC console and doesn't need doing yet again.

Before you add a Linux boot option, you will need to copy linload.exe

and the ARC loadable Milo that you wish to load to someplace that the

Windows NT ARC firmware can read from.  In my example, I assume that

you're booting from a DOS format floppy disk:

1. At the boot menu, select "Supplementary menu..."

2. At the "Supplementary menu", select "Set up the system..."

3. At the "Setup menu", select "Manage boot selection menu..."

4. In the "Boot selections menu", choose "Add a boot selection"

5. Choose "Floppy Disk 0"

6. Enter "linload.exe" as the osloader directory and name

7. Say "yes" to the operating system being on the same partition as

the osloader

8. Enter "

9. I usually enter "Linux" as the name for this boot selection

10.

Say "No" you do not want to initialise the debugger at boot time

11.

You should now be back in the "Boot selections menu", choose the

"Change a boot selection option" and pick the selection you just

created as the one to edit

12.

Use the down arrow to get "OSLOADFILENAME" up and then type in the

name of the Milo ARC image that you wish to use, for example

"noname.arc" followed by return.

13.

Press ESC to get back to the "Boot Selections menu"

14.

Choose "Setup Menu" (or hit ESC again) and choose "Supplementary

menu, and save changes" option

15.

ESC will get you back to the "Boot menu" and you can attempt to

boot Milo.	 If you do not want Linux as the first boot option, then

you can alter the order of the boot options in the "Boot selections

menu".

At the end of all this, you should have a boot selection that looks

something like:

LOADIDENTIFIER=Linux

SYSTEMPARTITION=multi(0)disk(0)fdisk(0)

OSLOADER=multi(0)disk(0)fdisk(0)\linload.exe

OSLOADPARTITION=multi(0)disk(0)fdisk(0)

OSLOADFILENAME=\noname.arc

OSLOADOPTIONS=

You can now boot Milo (and then Linux).  No doubt you can do this from

a file system that Windows NT understands such as NTFS or DOS on a

hard disk, but I've not yet tried this.

6.  Milo's User Interface.

Once you have correctly installed/loaded/run Milo you will see the

MILO (for MIniLOader) prompt displayed on your screen.  There is a

very simple interface that you must use in order to boot a particular

Linux kernel image.  Typing "help" is a good idea as it gives a useful

summary of the commands:

MILO command summary:

ls [dev:[dir]]   - List files in the specified dir on the specified device

boot [dev:file] [boot string]

- Boot Linux from the specified device and file

- and pass the boot string supplied

run dev:file     - Run the standalone program in dev:file

bootopt num	     - Select firmware type to use on next power up

resetenv	     - Delete all environment variables

setenv VAR VALUE - Set the variable VAR to the specified VALUE

unsetenv VAR     - Delete the specified environment variable

printenv	     - Display the current environment variable settings

show	     - Display all known devices

help	     - Print this text

Devices are specified as: fd0, hda, sda...

Environment variables that MILO cares about:

MEMORY_SIZE	     - System memory size in megabytes

AUTOBOOT	     - If set, MILO attempts to boot on powerup

and enters command loop only on failure

AUTOBOOT_TIMEOUT - Seconds to wait before auto-booting on powerup

BOOT_DEV	     - Specifies the default boot device

BOOT_FILE	     - Specifies the default boot file

BOOT_STRING	     - Specifies the boot string to pass to the kernel

SCSIn_HOSTID     - Specifies the host id of the n-th SCSI controller.

Note that the bootopt command only appears on AlphaPC64 (and similar)

systems.  Refer to the board's dcoumentation to find out just what it

means.

6.1.	Booting Linux

To boot a Linux kernel you will need to have an EXT2 formated disk

(either SCSI or floppy) available to Milo.  You can gzip the image, in

this case Milo will recognise that it is gzip'd by the .gz suffix.

You should note that the version of Milo does not usually have to

match the version of the Linux kernel that you are loading.  You can

boot using the following commands:

MILO> boot fd0:vmlinux.gz

MILO> boot sdb2:vmlinux.gz

If you're installing BLADE, then you'll need to specify a root device

and so on.  So you'd use:

MILO> boot fd0:vmlinux.gz root=/dev/fd0 ramdisk=1440

Milo will automatically contain the block devices that you configure

into your vmlinux.  I've tested the floppy driver, the IDE driver and

the NCR SCSI driver, these work fine.	 Others may or may not.	 One

word of warning about the NCR SCSI driver.  I've found that so long as

you build the driver with OPTION_IO_MAPPED defined, then all works

well.	 Also, it is important to set the host id of the SCSI controller

to a reasonable value.  By default, MILO will initialize it to the

highest possible value (7) which should normally work just fine.

However, if you wish, you can explicitly set the host id of the n-th

SCSI controller in the system by setting environment variable

SCSIn_HOSTID to the appropriate value.  For example, to set the hostid

of the first SCSI controller to 7, you can issue the following command

at the MILO prompt:

setenv SCSI0_HOSTID 7

6.2.	Running the Flash Update Utility

At the moment, the flash update utility is only available for the

Noname board, but over time others will be supported.	 Before you

start you'll need a device available to Milo that contains the

updateflash program.	This (like vmlinux) can be gzip'd.  You need to

run the update flash utility	program from the Milo using the (>run)

command:

MILO> run fd0:updateflash.gz

Wait until it has completed before powering off your system.

7.  Some Restrictions.

Unfortunately this is not a perfect world and there, as always, some

restrictions that you should be aware of.

Milo is not meant to load operating systems other than Linux, although

it can load images linked to run at the same place in memory as Linux

(which is 0xFFFFFC0000310000).  This is how the flash update utilities

can be run.

The PALcode sources included in miniboot/palcode/blah are correct,

however there are problems when they are built using the latest gas.

They do build if you use the ancient a.out gas that's supplied in the

Alpha Evaluation Board toolset (and that's how they were built).  I'm

trying to get someone to fix the new gas.  Meanwhile, as a workaround,

I've provided pre-built PALcode for the supported boards and David

Mosberger-Tang has a fixed gas on his ftp site.

I've not yet written/tested the flash update stuff for the EB66+ and

AlphaPC64, but I have given instructions on how to load the Milo into

flash on those systems using the Alpha Evaluation Board Debug Monitor

which ships with those systems.

Several people reported having problems reading files from the floppy

disk using the Evaluation Board Debug Monitor supplied with Digital

Alpha Evaluation Boards such as the AlphaPC64.  Usually, the Debug

Monitor can load the first few sectors all right, but then goes into

an endless loop complaining about "bad sectors."  Apparently, there is

an incompatibility between the DOS file system as expected by the

Debug Monitor and the Linux implementation of DOSFS.	To make the long

story short: if you run into this problem, try using DOS to write the

floppy disk.	For example, if loading the file milo.cab doesn't work,

use a DOS machine, insert the floppy and then do:

copy a:milo.cab c:

copy c:milo.cab a:

del c:milo.cab

Then try booting from that floppy again.  This normally solves the

problem.

The x86 emulator that Milo uses to execute the BIOS initialization

code found on graphics cards is basically an 8086 emulator with just

enough 386 support to get some of the graphics cards initialized.  At

present, are currently known not to work:

bus card type:		       chip:   BIOS version:		       Note:

--- --------------------------- ------  ---------------------------     -----

PCI ATI Graphics Pro Turbo      ???     ???			       1

Notes:

------

#1:     Reported by cruff@niwot.scd.ucar.EDU.

A word of warning: if you do try using MILO with the x86 emulator

enabled and it doesn't work with the graphics card in your system, it

may damage your monitor.  Thus, please be careful before doing this on

a system with an untested graphics card.

When loading Milo via the Windows NT ARC firmware, Windows NT seems to

store time in a different format from Linux and so the Windows NT ARC

firmware complains every time the system boots and the date is wrong

on Linux every time it boots.	 I use date and clock to put this right

by hand each time I boot.  This makes life hard if you want to co-

exist with Windows NT, but if you're using Milo to blow another image

into flash then there's no problem at all.

8.  Acknowledgements.

I would like to thank Eric Rasmussen and Eilleen Samberg, the original

authors of the PALcode, Jim Paradis for the keyboard driver and the

original MILO interface and David Mosberger-Tang for the BIOS

emulation as well as his support and encouragement.

There are a number of things that still need doing to Milo, amongst

which are proper flash support for the AlphaPC64 and ISO9660 file

system support.  If you want to add something yourself, then do let me

know so that we do not duplicate our efforts.

Finally, a big thank you to Digital for producing such a wonderful

chip (and paying me to do this).

###################################################################################################################################################################################################################################################################################################################################################################################################################################################################