DRIVER FEATURES

o	Independent control of reader and punch (i.e., reader and

punch can work at same time, can even be assigned to

different users).

o	Punch generates header/trailer tape of null bytes (may be

disabled if not desired, length of header/trailer can be

set by user).  A control-Z character (decimal 26) is punched

before the trailer null bytes to indicate end-of-file (control-Z

byte punching may be suppressed by user).

o	Driver performs error logging; the following errors are

reported: reader/punch timeout, reader/punch hardware

error.

o	Powerfail recovery during punching of user's first QIO

request, or during punching of trailer tape; powerfailure

at other times is unrecoverable (user told how many bytes

read/punched before powerfailure occurred).

o	Reader off-line error message sent to console (every 30

seconds) until reader turned on-line (if reader turned

off-line before QIO request, or between QIO requests; if

reader turned off while reading tape, condition inter-

preted as end-of-file, and user told how many bytes read

before end-of-file condition recognized). The tape must be in

the reader before a read request is issued (regardless of

whether the reader is on or off line).

o	If any error condition occurs, user informed in second word

of IOSB how many bytes read/punched before error occurred.

o	Can use DCL COPY command to read/punch data/files.  When

punching a file (i.e., $COPY file_name PC1:) no line separator

information is sent (e.g., no CR, LF, or SOS line numbers).

Therefore, if you ask for a file to be punched, and then read

it back in, it will not look exactly the same.  You may need to

write your own copy program that sends carriage returns, line

feeds, and line numbers.  (Note that this problem is not caused

by the driver, since it only reads/punches what/how much it is

told; the system just isn't giving the driver the line separator

information.)  You may even want to write byte counts preceding

data that is punched, so that it can be easily restored (on a

line by line basis).

SUPPORTED FUNCTION CODES

IO$_READVBLK,IO$_READLBLK,IO$_READPBLK

Restrictions:

Reader only.

Tape must be in reader before read QIO is issued.

Logical and physical codes require appropriate privilege.

If the first non-zero length read QIO issued by the user is

a virtual or logical read, characters

are read until the first non-null character is found (i.e.,

the header is skipped); a physical QIO returns header null

bytes as real data.  As soon as an EOF byte (decimal 26)

is encountered, an end-of-file condition is signalled

(unless user has disabled searching for an EOF byte).  If the

first non-zero length read QIO issued by the user was a

physical read, the default is not to look for an EOF byte.

Note that if you are not looking for an EOF byte, the reader

reads until the end-of-tape is encountered; if the tape end

is not torn evenly, you could read some garbage data at the

end of the file.  Also note that if you request a read after

receiving an end-of-file error message, you will receive

another end-of-file error message (with 0 bytes read); this

is done so the DCL $COPY command can work.  If you issue a

second read, then the tape will be read normally.

Parameters:

P1 = address of buffer into which data is to be read

P2 = number of bytes to read (<65536)

________________________________________________________________________

IO$_WRITEVBLK,IO$_WRITELBLK,IO$_WRITEPBLK,IO$_WRITEOF

Restrictions:

Punch only.

Logical and physical codes require appropriate privilege.

If the first non-zero length write QIO issued by the

user is not a physical write, a header of null bytes

is punched (default is 50 null bytes; can be changed by

IO$_MODIFY QIO); no header is punched for physical code.

The default is for an EOF byte (control-Z) to be punched

preceding the trailer, if the first non-zero length write

QIO was not a write physical QIO.

IO$_WRITEOF only applies if paper tape ACP (PCACP) is

running; ACP translates this function code into

IO$_WRITEVBLK code.  Not really useful, used to illustrate

how ACPs are written.  Requires logical I/O privilege.

Parameters:

P1 = address of buffer containing data to be punched

P2 = number of bytes to be punched (<65536)

IO$_MODIFY

Restrictions:

Punch only.

Parameters:

P2 = number of null bytes to be punched in header/trailer

(<65536).

________________________________________________________________________

IO$_DELETE

Restrictions:

Punch only.

Parameters:

None	Resets default (50) number of bytes to punch in header/

trailer.

________________________________________________________________________

IO$_DEACCESS

Restrictions:

Punch only.

Parameters:

IOSB should be specified.  Second longword of IOSB will contain

number of null bytes to be punched in header/trailer.

________________________________________________________________________

IO$_MOUNT

Restrictions:

Issued only by ACPMNT image to create paper tape ACP (PCACP).

Requires MOUNT privilege.

Parameters:

None	Note that the ACP (PCACP) is not required for the

driver to work. In fact, the ACP will rarely be in

the system, since its only value is as an instructional

tool (used to show how ACPs are written).

IO$_ACPCONTROL

Restrictions:

Issued only by PCDMT image to cause the ACP (PCACP) to be

dismounted.

Parameters:

None

________________________________________________________________________

IO$_CREATE

Restrictions:

None

Parametere:

None

For reader, enables searching for end-of-file byte.

Not needed if first read QIO was virtual/logical read.

For punch, enables punching of end-of-file byte.  Not

needed if first write QIO was write virtual/logical QIO.

________________________________________________________________________

IO$_READPROMPT

Restrictions:

None

Parameters:

None

For reader, used to disable searching for end-of-file

byte.  Not needed if first read QIO is physical read.

For punch, used to disable punching of end-of-file byte.

Not needed if first QIO is a write physical QIO.

DRIVER-SPECIFIC ERROR CODES

In addition to errors detected by the operating system, the

driver returns the following error codes:

SS$_CANCEL	User issued $CANCEL system service.

SS$_DEVNOTMOUNT	User issued MOUNT QIO from program other than ACPMNT,

or user issued WRITEOF function code and PCACP is not

running.

SS$_DRVERR	Device hardware error, no tape in punch, or no

power to punch.

SS$_ENDOFFILE	User specified more bytes to be read than there were

on the tape; could also be caused if reader turned

off-line during the middle of a read request, or if

there is no power supplied to the reader.  This error

code used when an EOF byte is found (control-Z).  The

first read following an EOF read also returns this

error code, with 0 bytes read in the IOSB.

SS$_ILLIOFUNC	User tried to read from punch, write to reader, or

issued an unsupported function code.

SS$_NOPRIV	User tried to mount PCACP without MOUNT privilege.

SS$_TAPEPOSLOST	Unrecoverable powerfail error.

SS$_TIMEOUT	Device timed out (probably a hardware error, or a very

very very busy system).

DRIVER INSTALLATION

The following commands should be used to load the paper tape

reader/punch driver (PCDRIVER).  Note that the reader is ALWAYS PC0:,

and the punch is always PC1:.

$MCR SYSGEN

LOAD SYS$SYSTEM:PCDRIVER		! may have to change filespec

CONNECT PCA0/ADAPTER=3-			! depending on where you keep

/NUMVEC=2-				! file PCDRIVER.EXE

/VECTOR=%O70-

/CSR=%O777550-

/DRIVER=PCDRIVER

CONNECT PCA1/ADAPTER=3-

/NUMVEC=1-

/VECTOR=%O74-

/CSR=%O777554-

/DRIVER=PCDRIVER

EXIT

Notice that the paper tape reader punch ACP (PCACP) is not required

in order for the driver to work properly (in fact, PCACP is only used

as an illustration of how to write ACPs).  If you want the ACP in your

system, you may have to modify the filespecs in programs ACPMNT and

PCDMT to find file PCACP.EXE (the default is SYS$SYSTEM:).

To mount PCACP, type $MCR ACPMNT (you will need CMKRNL, MOUNT, and

DETACH privileges).  To dismount PCACP, type $MCR PCDMT (you will

need CMKRNL privilege).  (This assumes you have ACPMNT.EXE and

PCDMT.EXE in the SYS$SYSTEM: directory.)