This directory contains various simple magtape handling utilities

for use under VAX/VMS.  VMS does fine with ANSI labeled tapes,

but provides no facilities for dealing with other types of tapes.

All the programs included here are very simple, and somewhat

special purpose.  It is often easier to make another program

than to make one monster general purpose program.

All programs are supplied in one .FOR file, and do not require

any other files to link.

All assume that they are dealing with MTA0:, because our system

has only one drive.

Watch record lengths and block sizes; tapes blocked with more than

one record per block will often need minor program patches.

ASCRD	- read an unlabeled ASCII magtape into an ASCII disk file

ASCWRT	- write an unlabeled ASCII tape from an ASCII disk file

BINREAD	- read an unlabeled tape into a binary (fortran 'unformatted')

disk file

BINWRT	- write an unlabeled tape from a binary disk file

EBCREAD	- read an unlabeled EBCDIC tape into an ASCII disk file

MISCREAD- reads various formats; see its internal instructions

RTTAPE	- reads labeled ASCII tapes with almost-ANSI headers.

Some PDP-11 wrote a tape which would DIRECTORY under ANSI

format, but would not copy in files.  This will.

Note that some tricks can be done by using combinations of these,

such as BINREAD and BINWRT to do an image copy of a tape without

interpreting it.

SKIP	- simple way to position the tape to the desired file.

Optionally rewinds the tape, then advances any desired

number of files.

Tapes from some systems are encoded in funny ways, and the above

programs do no decoding of files.  The following programs do not

access the magtape, but instead take their input from files

created by the tape utilities above.

Some UNIVAC 1108 systems can write EBCDIC tapes.  Each Univac file

(equivalent to a VAX directory) appears as one file on the tape.

DEUNIVAC reads such a file, after it has been read in by MISCREAD,

and writes one VAX file for each Univax 'element'.

Tapes from the UNIX operating system (mainly on PDP-11s and VAXs)

will often be in TAR format.  After using BINREAD to copy the tape

to disk, DETAR splits the master file up into the original

separate files.  Note that the Unix directory name information

is thrown away, only the last part of the file name is used.

Binary files are ignored because of serious differences in

record delimiters etc. between UNIX and VMS.

Even VAX's own ANSI format is odd looking unless decoded by the VMS

utilities.  If an ANSI tape must be read as foreign (if, for example,

a tape has bad spots causing VMS to refuse to read it),

use DEANSI to decode it into a normal file.  DEANSI assumes it is

dealing with the contents of one file, it does not understand

file headers and trailers.

The author accepts no responsibility for these programs.

Most are so short as to be self-explanatory, but in the

unlikely event that there are questions I will be happy to

try to answer them.

Martin Pensak

Research Computing Laboratory

601 Olin Building

Case Western Reserve University

Cleveland, Ohio 44106

216-368-4202.