4-December-1981

DCL Patch for PUSH / POP Commands                     ___ _____ ___ ____ _ ___ ________

Gary L. Grebus

Battelle Columbus Labs

505 King Avenue

Columbus, Ohio  43201

This is a patch to DCL to simulate TOPS-20  style  PUSH  and

POP  commands  on  the VAX.  These commands work as follows:

suppose you are editing and need to know the name of a file.

Type  Control-Y  to get out of the editor.  Type PUSH.  When

you get  the  next  "$"  prompt,  you  are  executing  in  a

subprocess  with  the same default directory, logical names,

and DCL symbols as in your original session.  You can do any

DCL  commands  at  this  point,  including another PUSH.  To

return to the interrupted program give the command POP.  The

subprocess  is  deleted and the image in the PUSH'ed process

is continued.  These commands can be used  with  any  image,

except  one  INSTALL'ed  with  privilege.   This  is because

Control-Y causes image rundown for such images.  Also,  PUSH

may not work properly with programs which enable a Control-Y

AST.

This modification consists of  a  patch  to  DCL  and  three

programs.   This  is  not  a project for the faint of heart.

However, these commands have been very useful to  me.   They

are   particularly   useful  if  you  are  constantly  being

interrupted by users with  questions.   You  can  PUSH  your

work, look at a user problem, and POP back.  Also, since the

name of the CLI is specified in the User Authorization File,

you  can  make  a  special  version of DCL and not alter the

standard one.  The  components  of  this  change  are  given

below:

DCL.COM - This file is an input file to  PATCH  to  put  the

hooks  into  DCL.   This mod requires sacrificing one of the

DCL internally  executed  commands.   These  are  listed  in

section 4.2.3 of the Command Language User's Guide.  I chose

to replace EOD, although DEPOSIT might be a  better  choice.

I suppose it would be possible to incorporate this change as

a completely new command but that looked to hairy to  do  at

the time.

There are two pieces to the patch.  The first  replaces  the

code  for the EOD command with a call to the image activator

to merge in DCLHOOK.  This image is  merged  into  P1  space

without  bothering the currently executing P0 image and then

called.  Following DCLHOOK's execution, a branch is made  to

the  DCL  code for the CONTINUE command to restart any image

that was interrupted.

The second piece of the patch is in the  intialization  code

for  DCL.   Whenever  DCL is started within a subprocess, it

calls the image activator to do a P0 merge of FLOGHOOK.

You will probably want to change  the  file  specs  for  the

images or define logical names FLOGHOOK and DCLHOOK.

DCLHOOK.MAR - This file is the source for the DCLHOOK image.

It saves relevant information about the current process such

as default directory, DCL symbols,  process  logical  names,

etc.   onto a file FORK.ENV in the user's default directory.

It then creates a  subprocess  running  DCL  with  the  same

process  permanent  files  as  the  main  process.  The main

process is placed  into  hibernation  until  the  subprocess

completes.

FLOGHOOK.MAR - This file is  the  source  for  the  FLOGHOOK

image.   This  image  is  called  when  DCL  activates  in a

subprocess.  It  reads  the  file  written  by  DCLHOOK  and

restores  the  information  found there.  Thus it creates in

the subprocess an environent very similar to  the  one  that

existed in the main process.

POP.MAR - This file is the source for the POP  command.   It

returns  to  the  next  higher level process by deleting the

process it is run in.  A check is made for a main process to

prevent the user from deleting himself completely.

Also included are files to compile  and  link  all  of  this

stuff.   I  cheat  with  the image files and force the entry

point to be at the very beginning the  image.   Then  I  can

simply  call  the image at the point at which it was loaded.

The right way would be to use the information from the image

activator  call to determine the actual address of the entry

point.

To install all this stuff use the  PATCH  command  to  apply

DCL.COM  to  a  copy  of  your DCL image.  Execute the three

command procedures to generate  the  images  required.   The

patch  to  DCL  assumes that the DCLHOOK and FLOGHOOK images

reside in a directory with  the  logical  name  BAT$SYSTEM:.

You  can  either  create  such a logical name, or modify the

patch (this is a bit tricky).  Alternately, you  can  define

the  logical  names DCLHOOK and FLOGHOOK to the the complete

image file specs for these two files.  The  file  spec  must

include  a  device  name.   Finally, define the PUSH and POP

commands as:

$PUSH :== EOD (or whatever DCL command you borrowed)

$POP :== RUN somewhere:POP.EXE

This whole thing was done as a quick hack,  but  works  well

here  (under  VMS  2.3).  It is not the ultimate solution to

making subprocesses share main process information, but that

would  take some rewriting of VMS.  The comments in the code

and patch are generally correct but the terminology may be a

bit inconsistant (this whole thing sort of evolved).  If you

have any questions about this mod, you  can  contact  me  at

(614)424-4664.