.lm 5

.rm 75

.p

This directory contains two programs written at the Washington University

School of Medicine to evaluate the need for additional MASSbus adapters

on a VAX 11/780.  The first is a real time display program called MBAUSE

which displays the activity on all of your MASSbus adapters as well as

the IO rates on all of your disk drives (assuming that you have less than

a dozen or so).  The second program is a discrete time event simulation

of MASSbus traffic called BUSMODEL.  It allows you to calculate the IO delays

due to heavy disk IO traffic under various load conditions.

.p

A command file is included to build each program.  Since MBAUSE has its

fingers in the executive, it will have to be rebuilt for version 3 of VMS.

In fact, if you don't rebuild MBAUSE each time you move to a new major

release of VMS, then it will probably crash your system.

.p

BUSMODEL is written in PASCAL so you will need that compiler to rebuild

it.  Otherwise, it is self contained.  Because it burns so much CPU time,

we usually do a bunch of runs at once in batch.  To this end, a command

file which runs it is included.

.p

In case you are curious, the conclusion of the study which motivated these

programs was that we didn't need an additional MBA.  We had an RM03, and

an RM80 on a single MBA and we wanted to know if we should get an additional

MBA when we added another RM80.  The result of the simulations showed that

the advantages of having another MBA were only slight and that the money

could be much better spent on additional memory to reduce paging.  In fact,

it is difficult to construct a situation in a 3 disk system where the MBA

becomes the bottleneck.  After we installed the third disk, the MBAUSE

program demonstrated that our predictions were correct.

.literal

Richard F. Wrenn

Washington University School of Medicine

Department of Biological Chemistry

660 S. Euclid

St. Louis, MO, 63110

.end literal