HGRAPH Documentation - July 11, 1983

Written by

H. L. McMaken

Applied Mathematical Sciences

Ames Laboratory - USDOE

Iowa State University

All rights (and wrongs) reserved

The following documents the line  drawing  routines  developed

for  use  on  the  LSI  11/23.   These routines will work with

either the Houston Instruments DMP-3  or  DMP-29  pen  plotter

(HIPLOT), the Grinnell Systems GMR 271 image processing system

(GRINNELL), or the Visual Products 550 (VISUAL).

PAGE 2

I. TERMINOLOGY

In order to understand many of the routines included  in  this

document,  one  must  understand the terminology that is used.

The  reader   will   encounter   such   phrases   as   "screen

coordinates", "world coordinates", "nominal inches", "plotting

frame", "plotting window", and "plotting page".  To understand

these, consider the following diagram:

The plotting window contains  the  data  to  be  plotted  (the

dashed  line in the above).  This data is "clipped" so that it

will only appear inside the  plotting  window.   The  plotting

frame is a region outside of which no plotting may take place.

The plotting window and the labels for the plotting window are

contained  within  the plotting frame.  The plotting window is

defined relative to the lower left hand corner of the plotting

frame.   The  plotting  page is the maximum allowable plotting

area.  This is dependent on the device used.  For  the  HIPLOT

this is 10.25 by 7.25 units, for the GRINNELL this is 10.22 by

10.22 units, and for the VISUAL this is 10.24  by  7.8  units.

These  units are expressed in "nominal inches".  On the HIPLOT

these are actual  inches.   These  nominal  inches  provide  a

global  coordinate  system  independent  of  the given device.

World coordinates are the coordinates associated with the data

to  be  plotted.   Screen coordinates are the coordinates used

internally by the plotting device.

PAGE 3

II. GETTING STARTED.

For the casual user, there are several higher  level  routines

which  should  be  of  interest.  These are divided into three

catagories:  initializing and terminating plots,  drawing  two

dimensional  plots,  and drawing three dimensional plots.  For

more advanced users, additional categories entitled Additional

Routines and Auxiliary Routines are included.

To get started, consider the following simple example program.

It  first  generates  the  data to be plotted into the x and y

arrays.  The  next  six  statements  initialize  the  plotting

device and plotting routines, plot the data, and terminate the

plot.  These six routines are in general the only routines one

needs  to  use  to  create  a  two-dimensional  plot.  A brief

description  of  each  routine  follows.   A   more   detailed

description follows in subsequent sections.

DIMENSION X(41), Y(41)

C

DO 10 I = 1, 41

X(I) = .05 * ( I - 1 )

Y(I) = SIN( 3.141592654 * X(I) )

10    CONTINUE

C

CALL INIPLT( 99, 10., 7. )

CALL WINDOW( 2., 8., 2., 6. )

CALL VUPORT( 0., 2., -1., 1. )

CALL AXIS( .5, .5, 'x', 1, 2, 1, 'sin', 3, 2, 1 )

CALL LINE( X, Y, 41, 0, 0, 0, 0 )

CALL ENDPLT

C

END

The above produces the following plot.

PAGE 4

Subroutine INIPLT initializes the plotting device and plotting

routines.   The  arguments  indicate  that  the  data is to be

written to FTN99.DAT, and that the plotting page is  10  by  7

nominal   inches.   Subroutine  WINDOW  defines  the  plotting

window.  The plotting window defined in the program is 6 by  4

nominal  inches starting at location (2,2).  Subroutine VUPORT

sets up the correspondence between the world  coordinates  and

the screen coordinates.  For this program, the horizontal axis

runs from 0 units to 2 units, while  the  vertical  axis  runs

from  -1 units to 1 units.  Subroutine AXIS draws a box around

the plotting window, draws tick marks every .5  units  on  the

horizontal and vertical axes, and labels these tick marks.  It

centers a lower case 'x' under  the  horizontal  axis,  and  a

lower case 'sin' to the left of the vertical axis.  Subroutine

LINE connects the points  to  be  plotted  by  line  segments.

Subroutine ENDPLT terminates the plot.

PAGE 5

III. INITIALIZING AND TERMINATING PLOTS.

The following statements initialize and terminate plotting:

SUBROUTINE INIPLT

SUBROUTINE DMPPLT

SUBROUTINE ENDPLT

To start the plot, the user must first initialize the plotting

routines.   This  requires  that  the  following  statement be

executed PRIOR to calling any other plotting routine:

CALL INIPLT(iunit, xsize, ysize)

This routine initializes the plot.  The  variables  xsize  and

ysize  denote the size of the plotting page in nominal inches.

For the HIPLOT , this is the size of the paper.  HIPLOT has  a

maximum  length  of  10.25 inches and a height of 7.25 inches.

Setting xsize larger than 10.25, or ysize larger than 7.25 may

cause  the  limit  switches  to  be activated during the plot.

This  will  cause  unpredictable  results.   No  plotting   is

permitted  outside this area.  The maximum size of the plot on

the GRINNELL is 10.22 by 10.22.  The maximum size of the  plot

on  the  VISUAL  is  10.24  by 7.8.  For compatability between

devices, bounds  within  the  common  intersection  should  be

chosen.

The variable iunit is the  logical  unit  number  selected  to

contain  the plot commands.  When plotting on the GRINNELL the

variable iunit is ignored.  The following table indicates  the

conventions used on the 11/23 for the HIPLOT and the VISUAL.

PAGE 6

The logical unit number equal to  7  is  associated  with  the

user's  terminal.   When  iunit is equal to 7 the plot is sent

directly to the terminal.  Hence, the HIPLOT must be connected

to  the printer port if you are to see the plot.  The software

will automatically enable the printer port  prior  to  sending

the  data, and disable the printer port upon completion of the

plot.  Since the terminal  for  the  VISUAL  is  the  plotting

device,  setting  iunit = 7 will display the plot immediately.

The plotting instructions generated by  the  program  are  not

stored in a file and are lost upon termination of the program.

Setting iunit = 9, sends  the  plot  directly  to  the  queued

plotting  device.   This  is  a  HIPLOT  device  and  will not

understand the data  generated  by  a  plot  for  the  VISUAL.

Hence,  DO  NOT USE iunit = 9 with the VISUAL.  To save a copy

of the  plot  use  iunit  =  10  to  99.   It  is  the  user's

responsibility to see that saved copies of the plot are backed

up.

To dump the buffer so that the entire plot may be seen,  place

the following statement in your program:

CALL DMPPLT

Subroutine DMPPLT is useful for maintaining  synchronized  I/O

for  interactive  plotting,  without  the  hassle of recalling

INIPLT and ENDPLT.

To terminate a plot, place the  following  statement  in  your

program:

CALL ENDPLT

This routine terminates the plotting.  It  must  be  the  last

plotting  routine  called  if  the  entire  plot file is to be

viewed.  No other plotting routine may  follow  without  first

calling INIPLT.

PAGE 7

IV. TWO DIMENSIONAL PLOTS.

For two dimensional plots the  following  routines  should  be

used.

CALL FRAME

CALL WINDOW

CALL VUPORT

CALL AXIS

CALL DASHLN

CALL LINE

These are described below.

SUBROUTINE FRAME(xmin, xmax, ymin, ymax)

This routine allows the user to select the plotting frame.  No

plotting  may  take  place outside of this region.  Subroutine

FRAME maps the  origin  into  the  point  (xmin,  ymin).   The

usefulness of this routine is in producing multiple plots on a

single page.  In this case one would like to  make  sure  that

parts  of  one plot do not overwrite parts of other plots.  By

specifying limits, one can control  where  plotting  can  take

place.   The  variables  xmin  and  xmax  are  the minimum and

maximum horizontal  dimensions  in  inches  of  the  allowable

plotting area, and the variables ymin and ymax are the minimum

and maximum vertical dimensions.  Subroutine FRAME need not be

called  unless desired.  However, the first call to FRAME must

follow  INIPLT  and  preceed  calls  to  WINDOW  and   VUPORT.

Subsequent calls to subroutine FRAME will automatically update

the transformation from world to screen coordinates using  the

arguments of the last call to WINDOW and VUPORT.  The position

of the plotting window will be in the same place  relative  to

the  point (xmin, ymin) as the previous plot.  If FRAME is not

called, the origin is taken to be the lower left  hand  corner

of the plotting page.

SUBROUTINE WINDOW(xwin0, xwin1, ywin0, ywin1)

This routine sets up the dimensions of  the  plotting  window.

The variables xwin0, xwin1, ywin0, and ywin1 are the bounds in

inches of this window.  The values  xwin0,  xwin1,  ywin0  and

ywin1 are given relative to the last call to FRAME.

SUBROUTINE VUPORT (xmin, xmax, ymin, ymax)

This routine sets up  the  correspondence  between  the  world

coordinates and the screen coordinates.  World coordinates are

the coordinates  associated  with  the  data  to  be  plotted.

Screen  coordinates  are  the  coordinates use by the plotting

device.  The variables xmin and xmax  represent  respectively,

the minimum and maximum x-values to be plotted.  The variables

PAGE 8

ymin and ymax represent respectively, the minimum and  maximum

y-values  to  be  plotted.   The  point  (xmin,  ymin) will be

plotted at (xwin0, ywin0).  The point  (xmax,  ymax)  will  be

plotted  at  (xwin1,  ywin1).   Values  falling  outside  this

viewport will be clipped.  The default values for (xmin, ymin)

and  (xmax, ymax) are (0, 0) and (1, 1), respectively.  A call

to VUPORT is not needed after a call to WINDOW.  This  feature

is useful for multiple plots in which the scaling of the x and

y axes do not change.  To draw a second plot on the same  page

(  or  screen  )  the user need only issue a call to WINDOW to

move the plotting window.  This  will  adjust  the  coordinate

transformation  between  world  and screen coordinates so that

the previous values supplied by VUPORT are now plotted at  the

positions  indicated  by  the arguments of the call to WINDOW.

Note, however, that merely changing WINDOW will  not  guaranty

that  one  plot  will  not  overwrite another plot.  To insure

this, the user should use FRAME.

SUBROUTINE AXIS(xtic, ytic, ixtit, ixlen, ixsiz, ixfmt,

iytit, iylen, iysiz, iyfmt)

This routine draws the axes for two  dimensional  plots.   The

variables xtic and ytic are the distances in world coordinates

between tick marks on the x and y axes.  The variable ixtit is

the label for the x-axis.  The variable ixlen is the number of

characters in ixtit.  The variable ixsiz is  the  size  (value

between 1 and 5) of the x label.  For the HIPLOT the following

sizes are available:

For ixsiz = 1 the characters are 0.07 inch high.

For ixsiz = 2 the characters are 0.14 inch high.

For ixsiz = 3 the characters are 0.28 inch high.

For ixsiz = 4 the characters are 0.56 inch high.

For ixsiz = 5 the characters are 1.12 inch high.

For the GRINNELL the following sizes are avialable:

For ixsiz = 1 the characters are 0.16 inch high.

For ixsiz = 2 the characters are 0.16 inch high.

For ixsiz = 3 the characters are 0.32 inch high.

For ixsiz = 4 the characters are 0.32 inch high.

For ixsiz = 5 the characters are 0.32 inch high.

For the VISUAL the following sizes are avialable:

For ixsiz = 1 the characters are 0.11 inch high.

For ixsiz = 2 the characters are 0.15 inch high.

For ixsiz = 3 the characters are 0.29 inch high.

For ixsiz = 4 the characters are 0.44 inch high.

For ixsiz = 5 the characters are 0.59 inch high.

If ixsiz is outside of this range, ixsiz is reset to  2.   The

variable ixfmt defines the format of the values on the x-axis.

PAGE 9

For ixfmt = 0 to 3, the format is taken to be  F9.ixfmt.   For

ixfmt  =  4,  the format is E9.2.  The variables iytit, iylen,

iysiz and iyfmt are defined in a  corresponding  way  for  the

y-axis.

SUBROUTINE DASHLN(x, y, n, icol, isym, isize, inum, lintyp)

This subroutine will draw dashed lines of  a  user  controlled

color through the n coordinate points (x, y).  The variables x

and y are n-dimensional arrays.   The  variable  isym  is  the

symbol  to  be plotted at the points.  The GRINNELL and VISUAL

currently do not support symbol plots.  The value of isym must

be  between 0 and 5.  For isym = 0 a plus sign is plotted, for

isym = 1 a cross is plotted, for isym = 2 a square is plotted,

for  isym  = 3 a circle is plotted, for isym = 4 a triangle is

plotted, and for isym  =  5  an  hourglass  is  plotted.   The

variable  isize is the size of the symbol.  The value of isize

must be between 1 and 5.  The size of the symbol is  given  in

inches  by  .035  *  2  **  isize.   A symbol is plotted every

abs(inum) points.  If  inum  is  negative,  only  symbols  are

plotted.   If  any  of  the following are out of range, only a

line  is  drawn:   isym,  isize,  inum.   The  variable   icol

determines  the  color  of  the  line.   For  the GRINNELL the

folowing colors are avialable:

For icol = 0, a white line is drawn.

For icol = 1, a red line is drawn.

For icol = 2, a orange line is drawn.

For icol = 3, a yellow line is drawn.

For icol = 4, a green line is drawn.

For icol = 5, a blue line is drawn.

For icol = 6, a purple line is drawn.

For the HIPLOT

the following association between icol and color is suggested:

For icol = 0, a black line is drawn.

For icol = 1, a red line is drawn.

For icol = 2, a orange line is drawn.

For icol = 3, a lime-green line is drawn.

For icol = 4, a green line is drawn.

For icol = 5, a blue line is drawn.

For icol = 6, a lavender line is drawn.

For icol = 7, a brown line is drawn.

The parameter lintyp controls the type of dashed  line  drawn.

For  lintyp  equal  to  0,  a solid line is drawn;  for lintyp

equal to 1, a dotted line is drawn, for lintyp between  2  and

6,  a  dashed  line  of  increasing  dash length is drawn, for

lintyp equal to 7 a dash-dot line is  drawn,  and  for  lintyp

equal to 8, a dash-dot-dot line is drawn.

SUBROUTINE LINE(x, y, n, icol, isym, isize, inum)

PAGE 10

This  routine  draws  a  solid  line  through  the   n   world

coordinates,  (x,  y).   The arguments of LINE are the same as

described above for DASHLN.

The following details the relation between INIPLT,  FRAME  and

WINDOW.

Subroutine INIPLT  sets  up  the  maximum  size  of  the  plot

(plotting  page).   This  is  either  the physical size of the

paper for the HIPLOT (i.e., 10.25 x 7.25 inches), the size  of

the  GRINNELL  screen  (i.e., 10.22 x 10.22 nominal inches) or

the size of the VISUAL  screen  (i.e.,  10.24  x  7.8  nominal

inches).   Within  this  region  you  may  futher  divide  the

plotting page by framing certain portions of it.  Within  each

plotting  frame  you can specify a plotting window.  The lines

drawn as part of the plot of x and y coordinate pairs will  be

contained within the plotting window.  The labels for the axes

will be contained within the  plotting  frame.   The  plotting

frame  will  be  contained  within  the plotting page (see the

section on TERMINOLOGY).  As an example, the following program

will  divide  the  plotting page into four separate regions in

which a copy of a plot of sin(x) and cos(x) from 0 to 6.28  is

drawn.  The plot produced by this program is shown below.

PAGE 11

DIMENSION X(41), S(41), C(41)

C

C	CALCULATE X AND Y VALUES

C

DO 10 I = 1, 41

X(I) = .157 * ( I - 1 )

C(I) = COS( X(I) )

S(I) = SIN( X(I) )

10	CONTINUE

C

C	SET PHYSICAL PAGE SIZE TO BE 10 X 7 INCHES

C

CALL INIPLT( 99, 10., 7. )

C

C	SELECT LOWER LEFT HAND QUARTER AS PLOTTING FRAME

C

CALL FRAME ( 0., 5., 0., 3.5 )

CALL WINDOW( 1.5, 4.5, 1., 3. )

CALL VUPORT( 0., 6.28, -1., 1. )

CALL AXIS ( 1., .5, 'x', 1, 2, 0, 'y(x)', 4, 2, 1 )

CALL DASHLN ( X, C, 41, 0, 0, 0, 0, 1 )

CALL DASHLN ( X, S, 41, 1, 0, 0, 0, 2 )

C

C	DRAW SECOND PLOT IN UPPER LEFT

C	NOTE THAT FRAME WILL AUTOMATICALLY ADJUST WINDOW AND VUPORT

C	FOR YOU

C

CALL FRAME ( 0., 5., 3.5, 7. )

CALL AXIS ( 1., .5, 'x', 1, 2, 0, 'y(x)', 4, 2, 1 )

CALL DASHLN ( X, C, 41, 2, 0, 0, 0, 3 )

CALL DASHLN ( X, S, 41, 3, 0, 0, 0, 4 )

C

C	DRAW THIRD PLOT IN UPPER RIGHT

C

CALL FRAME ( 5., 10., 3.5, 7. )

CALL AXIS ( 1., .5, 'x', 1, 2, 0, 'y(x)', 4, 2, 1 )

CALL DASHLN ( X, C, 41, 4, 0, 0, 0, 5 )

CALL DASHLN ( X, S, 41, 5, 0, 0, 0, 6 )

C

C	DRAW FOURTH PLOT IN LOWER RIGHT

C

CALL FRAME ( 5., 10., 0., 3.5 )

CALL AXIS ( 1., .5, 'x', 1, 2, 0, 'y(x)', 4, 2, 1 )

CALL DASHLN ( X, C, 41, 6, 0, 0, 0, 7 )

CALL DASHLN ( X, S, 41, 6, 0, 0, 0, 8 )

C

C	TERMINATE PLOTTING

C

CALL ENDPLT

C

END

PAGE 12

V. THREE DIMENSIONAL PLOTS.

Three dimensional plots with hidden line removal  are  handled

by the following routine:

CALL PLOT3D(f,x,y,nx,ny,xw,yw,xview,yview,zview,line,scale)

The variable f is an nx by ny array containing the  values  of

the  surface to be plotted.  The variables x and y are also nx

by ny arrays.  They are used internally by PLOT3D for  storing

the x and y plot coordinates.  The variables nx and ny are the

dimensions of the f, x, and y arrays.  The variables xw and yw

are  the  dimensions  of  the  plot  in inches.  The variables

xview, yview and zview are the location in inches of the point

of  observation  relative  to the grid.  The grid is xw by yw.

The variable line chooses  which  direction(s)  the  projected

grid  lines  are  drawn.   If  line  =  1, only lines in the x

direction are drawn.  If  line  =  2,  only  lines  in  the  y

direction  are  drawn.   For line = 0, lines are drawn in both

the x and y directions.   The  variable  scale  is  a  scaling

factor  for  the  array  f.  For scale = 0, a suitable scaling

factor is chosen by PLOT3D.

A sample program is given below.

DIMENSION F(21, 11), X(21, 11), Y(21, 11)

C

PI = 3.141592654

TYPE 2

ACCEPT *, IUNIT

C

CALL INIPLT(IUNIT, 10., 7.)

C

DO 20 J = 1, 11

XX = .1 * (J - 1)

DO 10 I = 1, 21

YY = .1 * (I - 1)

F(I, J) = COS(PI * XX) * SIN(PI * YY)

10	     CONTINUE

20	CONTINUE

C

TYPE 1

ACCEPT *, LINE, XV, YV, ZV

C

CALL PLOT3D(F,X,Y,21,11,10.,5.,XV,YV,ZV,LINE,0.)

CALL ENDPLT

C

STOP

1	FORMAT('$INPUT LINE, XV, YV, ZV? ')

2	FORMAT('$INPUT IUNIT? ')

END

PAGE 13

With line = 0, XV = 20, YV = 30 and ZV = 40, the above program

produces the following plot.

With line = 0, XV = -10, YV = 30 and  ZV  =  40,  the  program

produces this plot.

PAGE 14

VI. ADDITIONAL ROUTINES.

SUBROUTINE ERASE

Subroutine ERASE has been added to  the  software  package  in

order to erase the GRINNELL or VISUAL screen without reissuing

a call to INIPLT.  This has the advantage of not resetting all

of the internal saved parameters generated by calls to WINDOW,

VUPORT, etc..  This routine has no effect  for  plots  on  the

HIPLOT.

SUBROUTINE PLTOFF

Subroutine PLTOFF will disable the printer port when  plotting

on   the   HIPLOT   using  iunit  =  7,  or  will  enable  the

alphanumerics screen on the VISUAL.  When used with PLTON,  it

allows  the  user  to  enter  data  to the program while it is

plotting data.  Innocently typing in data  while  the  printer

port  is  enabled can cause erratic behavior on the HIPLOT.  A

similar transgression on the VISUAL results in the plot  being

overwritten.

SUBROUTINE PLTON

Subroutine PLTON will enable the printer port when plotting on

the  HIPLOT  using iunit = 7, or will enable the alphagraphics

screen on the VISUAL.

SUBROUTINE MOVETO( xwrld, ywrld, iud, lintyp )

Subroutine MOVETO will move from the current  cursor  position

to  the  world  coordinate  (xwrld, ywrld) if it is inside the

plotting window, otherwise it will move to the intersection of

the plotting window.  MOVETO will draw a line only when iud is

equal to one.  The type of line drawn is determined by lintyp.

FUNCTION ISCRX( xwrld )

Function ISCRX converts the x  world  coordinate  into  the  x

screen coordinate.

FUNCTION ISCRY( ywrld )

Function ISCRY converts the y  world  coordinate  into  the  y

screen coordinate.

FUNCTION XWORLD( ix )

Function XWORLD converts the x  screen  coordinate  to  the  x

world coordinate.

PAGE 15

FUNCTION YWORLD( iy )

Function YWORLD converts the y  screen  coordinate  to  the  y

world coordinate.

SUBROUTINE CURPOS( icurx, icury )

Subroutine CURPOS returns the current screen coordinates  into

icurx and icury.

SUBROUTINE PLTSTR( ix, iy, str, len, irot, isize )

Subroutine PLTSTR will write an alphanumeric string  'str'  of

length 'len' at screen location given by (ix, iy).  The height

of the string  is  determined  by  isize  (see  AXIS  for  the

relevant   sizes   of  the  characters).   The  argument  irot

determines the rotation of the  string.   For  irot  =  1  the

string  is  written  right side up, for irot = 2 the string is

rotated clockwise through 90 degrees, for irot = 3 the  string

is rotated 180 degrees, and for irot = 4 the string is rotated

through 270 degress.

PAGE 16

VII. RUNNING A PROGRAM.

To compile, link and run a program  which  will  plot  on  the

HIPLOT,  the GRINNELL or the VISUAL, the following sequence is

used.  For the HIPLOT use,

FORT filespec

LINK filespec,SY:HIPLOT

RUN filespec

or

HIX filespec

For the GRINNELL use,

FORT filespec

LINK filespec,SY:GRPLOT,SY:GRLIB

RUN filespec

or

GRX filespec

For the VISUAL use,

FORT filespec

LINK filespec,SY:VIPLOT

RUN filespec

or

VIX filespec

The files  SY:HIPLOT,  SY:GRPLOT,  and  SY:VIPLOT  are  object

modules  containing  the  device  dependent plotting routines.

The file SY:GRLIB is an object library  of  routines  for  the

GRINNELL.   Listings  of the routines contained in each of the

object modules are located on the system disk as  SY:name.LST.

The  FORTRAN source is also contained on the system disk.  The

FORTRAN source and listing file  for  the  device  independent

routines   are   on  the  system  disk  as  SY:HGRAPH.FOR  and

SY:HGRAPH.LST, respectively.

PAGE 17

VIII. DISPLAYING A DISK PLOT FILE.

To display a disk plot file containing the device instructions

for either the HIPLOT or the VISUAL, one should use one of the

following.  To display a plot on the VISUAL type,

TYPE filspec

To display a plot on the HIPLOT connected to  a  printer  port

type,

TYPLOT filspec

To display a plot on the queued HIPLOT device type,

COPY filspec PT:

SPOOL PT FORM=STD

The letters STD must be capitalized.  As an  example,  suppose

you  generated a plot using the VISUAL software with iunit=99.

This created a disk file named  FTN99.DAT.   To  display  this

file on the VISUAL type the following:

TYPE FTN99.DAT

PAGE 18

IX. AUXILIARY ROUTINES.

The following lower level  routines  have  been  included  for

completness.    These  routines  should  not  be  used  unless

absolutely  needed,  as  their  function  may  change  without

notice.

SUBROUTINE COLTYP(icol)

This routine sets the line color.  It is inoperative  for  the

VISUAL and the single pen HIPLOT.

SUBROUTINE DASH( ix, iy, iud, lintyp )

This routine will draw a dashed line of type lintyp  from  the

current position to the screen coordinate (ix, iy).

SUBROUTINE INSECT(i1, j1, iplot, iplot0, x, y, nx, ny, l)

This routine computes the intersection point of  a  grid  line

which is partially hidden.  It is used by PLOT3D.

SUBROUTINE MARKER(mrknum, isize)

This routine places a marker at the current position.   It  is

nonfunctional for the VISUAL and GRINNELL

SUBROUTINE MOV1ST( xwrld, ywrld, lintyp )

Subroutine  MOV1ST  moves  to  the  world  coordinate  (xwrld,

ywrld).   The argument lintyp refers to the type of line to be

initialized (see DASHLN for details  on  the  types  of  lines

available).

SUBROUTINE PENDWN

This routine puts the pen down so it will draw a visible line.

It is applicable only for HIPLOT and VISUAL.

SUBROUTINE PENUP

This routine puts the pen up so  no  line  is  drawn.   It  is

applicable only for HIPLOT and VISUAL.

SUBROUTINE PLOT(ix, iy, i)

This routine moves to the screen coordinates (ix, iy).  If i =

0,  the  pen is put up before moving, if i = 1, the pen is put

down before moving.  It checks to see if within page boundary.

PAGE 19

SUBROUTINE PLOTIN(x, y, i)

This routine moves to the point (x, y) where x and y are given

in  nominal  inches.   If  i=0, pen is put up before plotting,

otherwise the pen is put down.

SUBROUTINE TRIML( str, n, len )

This routine trims leading  blanks  from  the  string  STR  of

length  n.  It returns in LEN the length of the trimmed string

and in STR the string left justified with all blanks removed.

SUBROUTINE WRITCH(ch)

This routine is used by the VISUAL  to  output  characters  to

either  the  terminal  or  a  disk file.  The argument ch is a

single byte.

SUBROUTINE WRTSTR(str, len, irot, isize)

This routine writes a string at  the  current  location.   The

argument len is the length of the string;  irot determines the

rotation of the string (1 - 4), 1  is  right  side  up,  2  is

rotated  90 degrees clockwise, etc.;  and isize is the size of

the string (1 <= isize <= 5).  The string is checked to see if

in is in bounds.

APPENDIX A

DEVICE INDEPENDENT ROUTINES

0001        SUBROUTINE FRAME (xmin, xmax, ymin, ymax)

c

c     this routine frames a plot, i.e., no plotting of any kind

c     may take place outside of this frame without issuing a new

c     call to frame

c

0002        common /zgraph/ lunplt, iascr, ibscr, icscr, idscr,

1 xm, ym, dx, dy, dxb, dyb, ixcur, iycur, xmid, ymid,

1 xslope, yslope, xconst, yconst, scrx, scry, ichar(5),

1 ixorig, iyorig

c

0003        common /pagsiz/ xbond, ybond

c

0004        xn = amax1 ( 0., xmin )

0005        xx = amin1 ( float(xbond), xmax )

0006        yn = amax1 ( 0., ymin )

0007        yx = amin1 ( float(ybond), ymax )

0008        xmo = xmid

0009        ymo = ymid

0010        xmid = .5 * scrx * ( xx + xn )

0011        ymid = .5 * scry * ( yx + yn )

0012        dxb = .5 * scrx * ( xx - xn )

0013        dyb = .5 * scry * ( yx - yn )

c

0014        dxo = xmid - xmo

0015        dyo = ymid - ymo

0016        ixorig = scrx * xn

0017        iyorig = scry * yn

c

c     move window into frame

c

0018        iascr = iascr + dxo

0019        ibscr = ibscr + dxo

0020        icscr = icscr + dyo

0021        idscr = idscr + dyo

0022        xm = xm + dxo

0023        ym = ym + dyo

c

c     move view port into frame

c

DEVICE INDEPENDENT ROUTINES                                             PAGE A-2

0024        xconst = xconst + dxo

0025        yconst = yconst + dyo

c

0026        return

0027        end

DEVICE INDEPENDENT ROUTINES                                             PAGE A-3

0001        SUBROUTINE WINDOW(xwin0, xwin1, ywin0, ywin1)

c

c     this subroutine sets the dimensions of the plotting window.

c     xwin0, xwin1, ywin0, and ywin1 are the bounds in inches of

c     the plotting window. the actual plot less labels are contained

c     within this window. scaling is performed by subroutine setwin.

c

0002        common /zgraph/ lunplt, iascr, ibscr, icscr, idscr,

1 xm, ym, dx, dy, dxb, dyb, ixcur, iycur, xmid, ymid,

1 xslope, yslope, xconst, yconst, scrx, scry, ichar(5),

1 ixorig, iyorig

c

0003        common /world/ xmin, xmax, ymin, ymax

c

c     convert from inches to plot coordinates

c

0004        iascr = scrx * xwin0 + ixorig

0005        ibscr = scrx * xwin1 + ixorig

0006        icscr = scry * ywin0 + iyorig

0007        idscr = scry * ywin1 + iyorig

c

0008        dx = .5 * (ibscr - iascr)

0009        dy = .5 * (idscr - icscr)

0010        xm = .5 * (ibscr + iascr)

0011        ym = .5 * (idscr + icscr)

c

0012        call vuport ( xmin, xmax, ymin, ymax )

0013        return

0014        end

DEVICE INDEPENDENT ROUTINES                                             PAGE A-4

0001        SUBROUTINE VUPORT(awrld, bwrld, cwrld, dwrld)

c

c     this subroutine sets up the correspondence between the

c     world coordinates and the screen coordinates by initializing

c     the variables xslope, yslope, xconst and yconst which

c     implicitly define the affine map from world to screen

c     coordinates through the formulas

c

c     	xscr = xwrld * xslope + xconst

c     	yscr = ywrld * yslope + yconst

c

c

0002        common /zgraph/ lunplt, iascr, ibscr, icscr, idscr,

1 xm, ym, dx, dy, dxb, dyb, ixcur, iycur, xmid, ymid,

1 xslope, yslope, xconst, yconst, scrx, scry, ichar(5),

1 ixorig, iyorig

c

0003        common /world/ xmin, xmax, ymin, ymax

c

0004        xmin = awrld

0005        xmax = bwrld

0006        ymin = cwrld

0007        ymax = dwrld

c

0008        xslope = (ibscr - iascr) / (bwrld - awrld)

0009        yslope = (idscr - icscr) / (dwrld - cwrld)

c

0010        xconst = - xslope * awrld + float(iascr)

0011        yconst = - yslope * cwrld + float(icscr)

c

0012        return

0013        end

DEVICE INDEPENDENT ROUTINES                                             PAGE A-5

0001        SUBROUTINE AXIS(xtic, ytic, ixtit, ixlen, ixsiz, ixfmt,

1			    iytit, iylen, iysiz, iyfmt)

c

c  draws axes for two dimensional plots. xtic and ytic are the

c  distances between tick marks on the x and y axes. ixtit is the

c  label for the x axis. ixlen is the length of the x label. ixsiz

c  is the size (1 - 5) of the x label. iytit, iylen and iysiz have

c  obvious cooresponding definitions.

c

0002        byte ixtit(ixlen), iytit(iylen), label(9)

0003        integer fmt(2) , emt, num(5), form(3)

0004        common /zgraph/ lunplt, iascr, ibscr, icscr, idscr,

1 xm, ym, dx, dy, dxb, dyb, ixcur, iycur, xmid, ymid,

1 xslope, yslope, xconst, yconst, scrx, scry, ichar(5),

1 ixorig, iyorig

c

0005        data fmt / '(F', '9.' /

0006        data emt / '(E' /

0007        data num / '0)', '1)', '2)', '3)', '2)' /

c

c  draw window (plot) boundary

c

0008        call coltyp(0)

0009        call plot(iascr, icscr, 0)

0010        call plot(ibscr, icscr, 1)

0011        call plot(ibscr, idscr, 1)

0012        call plot(iascr, idscr, 1)

0013        call plot(iascr, icscr, 1)

c

0014        idx = ifix(xtic * xslope)

0015        idy = ifix(ytic * yslope)

0016        iy = icscr

0017        n = (ibscr - iascr) / idx - 1

ccc

c

c  The following code puts tick marks and labels on the horizontal axis

c

0018        is = iascr

c

c  set up format of labels for horizontal axis

c

0019        iform = max0 ( 0 , ixfmt )

0020        form(1) = fmt(1)

0021        form(2) = fmt(2)

0022        form(3) = num( min0( 5 , iform + 1) )

0023        if ( iform .gt. 3 ) form(1) = emt

0024  c

c  label tick mark on left end of horizontal axis

c

c    compute value, translate and trim leading blanks

c

0025        xmin = xworld( iascr )

0026        encode ( 9 , form, label ) xmin

DEVICE INDEPENDENT ROUTINES                                             PAGE A-6

0027        call triml(label, 9, len)

c

c    remove decimal point for integer label

c

0028        if ( iform .eq. 0 ) len = len - 1

0029  c

c    determine size of label and center

c

0030        ixl = len * ichar(ixsiz)

0031        ix = is - ixl / 2

0032        iy = icscr - (ichar(ixsiz) * 7) / 3

c

c    write label

c

0033        call pltstr(ix, iy, label , len , 1 , ixsiz)

c

c  draw ticks and write labels on the interior of horizontal axis

c

0034        lentic = scrx / 4

c

c  if n < = 0 then there will be no tick marks drawn nor any labels

c  written on the horizontal axis

c

0035        if(n .le. 0)goto 20

0037        do 10 i = 1, n

0038        	is = is + idx

c

c    draw tick mark

c

0039        	call plot(is, icscr, 0)

0040        	call plot(is, icscr + lentic, 1)

c

c    label tick mark

c

0041        	xmin = xmin + xtic

0042        	encode ( 9 , form, label ) xmin

0043        	call triml(label, 9, len)

0044        	if ( iform .eq. 0 ) len = len - 1

0046        	ixl = len * ichar(ixsiz)

0047        	ix = is - ixl / 2

c

0048        	call pltstr(ix, iy, label , len , 1 , ixsiz)

c

0049  10    continue

c

c  label tick mark on right end of horizontal axis

c

0050  20    continue

0051        is = is + idx

0052        xmin = xmin + xtic

0053        encode ( 9 , form, label ) xmin

0054        call triml(label , 9, len)

0055        if ( iform .eq. 0 ) len = len - 1

DEVICE INDEPENDENT ROUTINES                                             PAGE A-7

0057        ixl = len * ichar(ixsiz)

0058        ix = is - ixl / 2

c

0059        call pltstr(ix, iy, label , len , 1 , ixsiz)

c

c     trim leading and trailing blanks

c

0060        do 23 i = 1, ixlen

0061        if ( ixtit(i) .ne. ' ' ) goto 25

0063  23    continue

0064        i = ixlen

c

0065  25    continue

c

0066        do 27 j = ixlen, i, -1

0067        if ( ixtit(j) .ne. ' ' ) goto 29

0069  27    continue

0070        j = i

c

0071  29    continue

c

c  center horizontal title and plot

c

0072        ixln = j + 1 - i

0073        ixl = ixln * ichar(ixsiz)

0074        ix = (iascr + ibscr - ixl) / 2

0075        iy = iy - (7 * ichar(ixsiz)) / 3

c

0076        call pltstr(ix, iy, ixtit(i), ixln, 1, ixsiz)

c

0077        ix = iascr

0078        n = ( idscr - icscr ) / idy - 1

ccc

c

c  The following code puts tick marks and labels on the vertical axis

c

0079        call plot(iascr, icscr, 0)

0080        is = icscr

c

c  set up format of labels for vertical axis

c

0081        iform = max0 ( 0 , iyfmt )

0082        form(1) = fmt(1)

0083        form(2) = fmt(2)

0084        form(3) = num( min0( 5 , iform + 1) )

0085        if ( iform .gt. 3 ) form(1) = emt

0086  c

c  label tick mark on bottom end of vertical axis

c

c    compute value, translate and trim leading blanks

c

0087        ymin = yworld( icscr )

0088        encode ( 9 , form, label ) ymin

DEVICE INDEPENDENT ROUTINES                                             PAGE A-8

0089        call triml(label, 9, len)

c

c    remove decimal point for integer label

c

0090        if ( iform .eq. 0 ) len = len - 1

0091  c

c    determine size of label and center

c

0092        iyl = (len + 1) * ichar(iysiz )

0093        ix = iascr - iyl

0094        ixmin = ix

0095        ihgt = (7 * ichar(iysiz )) / 12

0096        iy = is - ihgt

c

c    write label

c

0097        call pltstr(ix, iy, label , len , 1 , iysiz)

c

c  draw ticks and write labels on the interior of vertical axis

c

0098        lentic = scry / 4

c

c  if n < = 0 then there will be no tick marks drawn nor any labels

c  written on the vertical axis

c

0099        if(n .le. 0)goto 40

0101        do 30 i = 1, n

0102        	is = is + idy

c

c    draw tick mark

c

0103        	call plot(iascr, is, 0)

0104        	call plot(iascr + lentic, is, 1)

c

c    label tick mark

c

0105        	ymin = ymin + ytic

0106        	encode ( 9 , form, label ) ymin

0107        	call triml ( label, 9, len )

0108        	if ( iform .eq. 0 ) len = len - 1

0110        	ix = iascr - ( len + 1 ) * ichar (iysiz)

0111        	ixmin = min0 ( ixmin, ix )

0112        	iy = is - ihgt

c

0113        	call pltstr(ix, iy, label , len , 1 , iysiz)

c

0114  30    continue

c

c  label tick mark on top end of vertical axis

c

0115  40    continue

0116        is = is + idy

0117        ymin = ymin + ytic

DEVICE INDEPENDENT ROUTINES                                             PAGE A-9

0118        encode ( 9 , form, label ) ymin

0119        call triml ( label, 9, len )

0120        if ( iform .eq. 0 ) len = len - 1

0122        ix = iascr - ( len + 1 ) * ichar (iysiz)

0123        ixmin = min0 ( ixmin, ix )

0124        iy = is - ihgt

c

0125        call pltstr(ix, iy, label , len , 1 , iysiz)

c

c  center vertical title and plot

c

0126        ix = ixmin - (7 * ichar(ixsiz)) / 6

0127        iyl = iylen * ichar(iysiz)

0128        iy = (icscr + idscr - iyl) / 2

c

0129        call pltstr(ix, iy, iytit, iylen, 4, iysiz)

c

0130        return

0131        end

DEVICE INDEPENDENT ROUTINES                                            PAGE A-10

0001        SUBROUTINE LINE ( x, y, n, icol, isym, isize, inum )

0002        dimension x(n), y(n)

0003        call dashln(x, y, n, icol, isym, isize, inum, 0 )

0004        return

0005        end

DEVICE INDEPENDENT ROUTINES                                            PAGE A-11

0001        SUBROUTINE DASHLN(x, y, n, icol, isym, isize, inum, ityp)

c

c     this routine plots the n world coordinates, (x, y).

c     icol is the color of line which connects the points.

c     isym is the symbol to be plotted at the points.

c     isize is the size of the symbol.

c     ityp is the type of line which connects the points.

c     a symbol is plotted every abs(inum) points. if inum is

c     negative, only symbols are plotted.

c     if any of the following are out of range, only a line is

c     drawn: isym, isize, inum

c

0002        byte mark

0003        dimension x(n), y(n)

0004        common/chkbon/ibond, istrt, linntp

c

0005        mark = isize .le. 0.or.isize .gt. 5

0006        mark = mark.or.isym .lt. 0.or.isym .gt. 5

0007        mark = mark.or.inum .eq. 0

0008        ibond = 0

c

0009        if(inum .lt. 0)goto 20

0010  c

c     draw line

c

0011        call coltyp(icol)

0012        istrt = 1

0013        call moveto(x(1), y(1), 0, ityp)

c

0014        do 10 i = 2, n

0015        	call moveto(x(i), y(i), 1, ityp)

0016  10    continue

c

0017  20    if(mark)return

0018  c

c     draw symbols

c

0019        ibond = 0

0020        itemp = iabs(inum)

0021        do 30 i = 1, n, itemp

0022        	call moveto(x(i), y(i), 0, ityp)

0023        	if(ibond .eq. 0)call marker(isym, isize)

0025  30    continue

c

0026        return

0027        end

DEVICE INDEPENDENT ROUTINES                                            PAGE A-12

0001        SUBROUTINE MOVETO (xwrld, ywrld, iud, lintyp)

c

c     moves the pen to the point with world coordinates (xwrld, ywrld)

c     draws a line only if iud = 1

c     the type of line drawn is set by the lintyp

c     move is made only to window boundary

c

0002        dimension ix1(2), iy1(2)

0003        common /zgraph/ lunplt, iascr, ibscr, icscr, idscr,

1 xm, ym, dx, dy, dxb, dyb, ixcur, iycur, xmid, ymid,

1 xslope, yslope, xconst, yconst, scrx, scry, ichar(5),

1 ixorig, iyorig

0004        common/chkbon/ibond, istrt, linntp

c

0005        if ( istrt .eq. 0 .and. linntp .eq. lintyp ) goto 500

0007        call mov1st(xwrld, ywrld, lintyp)

0008        return

c

0009  500   ix = iscrx( xwrld )

0010        iy = iscry( ywrld )

c

c    (ix,iy) is screen coordinate inside window

c    (ixo,iyo) is current screen coordinate

c    (ixcur,iycur) is previous screen coordinate

c    (ix0,iy0) is screen coordinate outside window

c

0011        ixo = ix

0012        iyo = iy

c

c     check to see if in window

c

0013        inx = (abs(ix - xm) .le. dx)

0014        iny = (abs(iy - ym) .le. dy)

0015        in = inx * iny

c

c     in is one if inside window, zero if outside window

c     ibond is zero if previous point inside window, minus one if outside

c     in + ibond will be positive only if both previous point and current

c     	point are inside window

c     in + ibond will be zero only if one point is inside and one point

c     	is outside

c     in + ibond will be negative only if both points are outside

c

0016        if ( in + ibond ) 300, 100, 200

ccc

c     this section finds the intersection with the window boundary for

c     consecutive points on opposite sides of the boundary

c

0017  100   ix0 = ixcur

0018        iy0 = iycur

0019        if ( in ) goto 120

0020  c

c     current point outside window

DEVICE INDEPENDENT ROUTINES                                            PAGE A-13

c

0021        ibond = - 1

c

c     find intersection (if any) with horizontal boundaries

c

0022        if ( inx ) goto 110

0024        xsl = (iy - iy0) / float(ix - ix0)

0025        xsc = xm + sign(dx, ix - xm)

0026        iy = xsl * (xsc - ix) + iy

0027        ix = xsc

c

c     find intersection (if any) with vertical boundaries

c

0028  110   if ( abs( iy - ym ) .le. dy ) goto 200

0030        ysl = (ix - ixcur) / float(iy - iycur)

0031        ysc = ym + sign(dy, iy - ym)

0032        ix = ysl * (ysc - iy) + ix

0033        iy = ysc

0034        goto 200

c

c     current point inside window

c

0035  120   ibond = 0

c

c     find intersection (if any) with horizontal boundaries

c

0036        if ( abs( ix0 - xm ) .le. dx ) goto 130

0038        xsl = (iy - iy0) / float(ix - ix0)

0039        xsc = xm + sign(dx, ix0 - xm)

0040        iy0 = xsl * (xsc - ix0) + iy0

0041        ix0 = xsc

c

c     find intersection (if any) with vertical boundaries

c

0042  130   if ( abs( iy0 - ym ) .le. dy ) goto 140

0044        ysl = (ix - ix0) / float(iy - iy0)

0045        ysc = ym + sign(dy, iy0 - ym)

0046        ix0 = ysl * (ysc - iy0) + ix0

0047        iy0 = ysc

c

c     move pen to boundary

c

0048  140   call dash(ix0, iy0, 0, lintyp)

ccc

c     this section plots points within the window

c

0049  200   call dash(ix, iy, iud, lintyp)

c

c     save current coordinates

c

0050        ixcur = ixo

0051        iycur = iyo

c

DEVICE INDEPENDENT ROUTINES                                            PAGE A-14

0052        return

ccc

c     this section handels points outside window

c

0053  300   insect = 0

0054        if ( ix .eq. ixcur ) goto 320

0056        if( abs(xm - dx - .5*(ix+ixcur)) .gt. .5*iabs(ix-ixcur) )goto 310

0058        xsl = ( iy - iycur ) / float( ix - ixcur )

0059        y0 = xsl * ( xm - dx - ix ) + iy

0060        if ( ( ( y0 - iy ) * ( y0 - iycur ) .gt. 0. )

1    .or. ( abs( y0 - ym ) .gt. dy ) ) goto 310

0062        insect = 1

0063        ix1(insect) = xm - dx

0064        iy1(insect) = y0

c

0065  310   if( abs(xm + dx - .5*(ix+ixcur)) .gt. .5*iabs(ix-ixcur) )goto 320

0067        y0 = xsl * ( xm + dx - ix ) + iy

0068        if ( ( ( y0 - iy ) * ( y0 - iycur ) .gt. 0. )

1    .or. ( abs( y0 - ym ) .gt. dy ) ) goto 320

0070        insect = insect + 1

0071        ix1(insect) = xm + dx

0072        iy1(insect) = y0

0073        if ( insect .eq. 2 ) goto 350

0074  c

0075  320   if ( iy .eq. iycur ) goto 390

0077        if( abs(ym - dy - .5*(iy+iycur)) .gt. .5*iabs(iy-iycur) )goto 330

0079        ysl = ( ix - ixcur ) / float( iy - iycur )

0080        x0 = ysl * ( ym - dy - iy ) + ix

0081        if ( ( ( x0 - ix ) * ( x0 - ixcur ) .gt. 0. )

1    .or. ( abs( x0 - xm ) .gt. dx ) ) goto 330

0083        insect = insect + 1

0084        ix1(insect) = x0

0085        iy1(insect) = ym - dy

0086        if ( insect .eq. 2 .and. ix1(2) .ne. ix1(1) ) goto 350

0088        insect = 1

c

0089  330   if( abs(ym + dy - .5*(iy+iycur)) .gt. .5*iabs(iy-iycur) )goto 390

0091        x0 = ysl * ( ym + dy - iy ) + ix

0092        if ( ( ( x0 - ix ) * ( x0 - ixcur ) .gt. 0. )

1    .or. ( abs( x0 - xm ) .gt. dx ) ) goto 390

0094        insect = insect + 1

0095        ix1(insect) = x0

0096        iy1(insect) = ym + dy

c

0097        if ( insect .lt. 2 ) goto 390

0099  350   call dash ( ix1(1), iy1(1), 0, lintyp)

0100        call dash ( ix1(2), iy1(2), iud, lintyp)

0101  390   ibond = -1

0102        ixcur = ixo

0103        iycur = iyo

0104        return

0105        end

DEVICE INDEPENDENT ROUTINES                                            PAGE A-15

0001        SUBROUTINE MOV1ST (xwrld, ywrld, lintyp)

c

c     moves the pen to the point with world coordinates (xwrld, ywrld)

c     draws a line only if iud = 1

c     the type of line drawn is set by the lintyp

c     move is made only to window boundary

c

0002        dimension ix1(2), iy1(2)

0003        common /zgraph/ lunplt, iascr, ibscr, icscr, idscr,

1 xm, ym, dx, dy, dxb, dyb, ixcur, iycur, xmid, ymid,

1 xslope, yslope, xconst, yconst, scrx, scry, ichar(5),

1 ixorig, iyorig

0004        common/chkbon/ibond, istrt, linntp

c

0005        ix = iscrx( xwrld )

0006        iy = iscry( ywrld )

0007        ibond = 0

0008        istrt = 1

c

c    (ix,iy) is screen coordinate

c     check to see if in window

c

0009        inx = (abs(ix - xm) .le. dx)

0010        iny = (abs(iy - ym) .le. dy)

0011        in = inx * iny

c

c     in is one if inside window, zero if outside window

c

0012        if ( in ) goto 100

0013  c

c     initial point outside of plotting window

0014        ibond = -1

0015        ixcur = ix

0016        iycur = iy

c

0017  100   call dash(ix, iy, 0, lintyp)

c

c     save current coordinates

c

0018        ixcur = ix

0019        iycur = iy

0020        istrt = 0

c

0021        return

0022        end

DEVICE INDEPENDENT ROUTINES                                            PAGE A-16

0001        SUBROUTINE DASH( ix, iy, iud, lintyp )

c

0002        integer lind(11)

0003        real idash(11), ispace(11)

0004        data lind / 1, 2, 3, 4, 5, 6, 9, 10, 7, 11, 8 /

0005        data idash/.02, .04, .08, .16, .24, .32, .16, .16, .04, .04, .04/

0006        data ispace/.08, .08, .08, .08, .12, .16, .08, .08, .08, .08, .08/

c

0007        common /zgraph/ lunplt, iascr, ibscr, icscr, idscr,

1 xm, ym, dx, dy, dxb, dyb, ixcur, iycur, xmid, ymid,

1 xslope, yslope, xconst, yconst, scrx, scry, ichar(5),

1 ixorig, iyorig

c

0008        common /chkbon/ ibond, istrt, lin

0009        data lin, lin1 / 0, 0 /

c

0010        if ( istrt .ne. 1 ) goto 50

0012        if ( lintyp .gt. 0 ) goto 60

0014        lin = 0

0015        goto 50

0016  60    lin = max0 ( lintyp, 1 )

0017        lin = min0 ( lin, 8 )

0018        lin1 = lin

0019        dash = scrx * idash(lin1)

0020        space = 0

0021  50    if ( lin .le. 0 ) goto 300

0023        if ( iud .ne. 1 ) goto 300

0025        dx1 = ix - ixcur

0026        dy1 = iy - iycur

0027        ds = sqrt ( dx1 * dx1 + dy1 * dy1 )

0028        if ( ds .eq. 0. ) return

0030        c = dx1 / ds

0031        s = dy1 / ds

0032        x0 = ixcur

0033        y0 = iycur

0034        if ( dash .eq. 0. ) goto 200

0036  100   if ( ds .ge. dash ) goto 150

0038        dash = dash - ds

0039        ds = 0.

0040        call plot ( ix, iy, 1 )

0041        return

0042  150   ds = ds - dash

0043        x0 = dash * c + x0

0044        y0 = dash * s + y0

0045        ix0 = x0 +.5

0046        iy0 = y0 +.5

0047        call plot ( ix0, iy0, 1 )

0048        dash = 0.

0049        space = scrx * ispace(lin1)

0050        if ( space .eq. 0. ) goto 270

0052  200   if ( ds .ge. space ) goto 250

0054        space = space - ds

0055        ds = 0.

DEVICE INDEPENDENT ROUTINES                                            PAGE A-17

0056        call plot ( ix, iy, 0 )

0057        return

0058  250   ds = ds - space

0059        x0 = space * c + x0

0060        y0 = space * s + y0

0061        ix0 = x0 +.5

0062        iy0 = y0 +.5

0063        call plot ( ix0, iy0, 0 )

0064        space = 0.

0065        lin1 = lind(lin1)

0066  270   dash = scrx * idash(lin1)

0067        goto 100

0068  300   call plot ( ix, iy, iud )

0069        return

0070        end

DEVICE INDEPENDENT ROUTINES                                            PAGE A-18

0001        SUBROUTINE PLTSTR( ix, iy, str, len, irot, isize )

c

c     this routine moves to the point (ix,iy) and writes a string

c     only if the entire string is in the boundary

c

0002        byte str(len)

c

0003        common /zgraph/ lunplt, iascr, ibscr, icscr, idscr,

1 xm, ym, dx, dy, dxb, dyb, ixcur, iycur, xmid, ymid,

1 xslope, yslope, xconst, yconst, scrx, scry, ichar(5),

1 ixorig, iyorig

c

c     attempt move to point and write string

c

0004        call plot ( ix, iy, 0 )

0005        if ( (ix .eq. ixcur) .and. (iy .eq. iycur) )

1	call wrtstr ( str, len, irot, isize )

0007        return

0008        end

DEVICE INDEPENDENT ROUTINES                                            PAGE A-19

0001        SUBROUTINE TRIML( label, n, len )

0002        byte label(n)

c

0003        do 10 i = 1, n

0004        if ( label(i) .ne. ' ' ) goto 20

0006  10    continue

c

0007  20    len = 0

0008        do 30 j = i, n

0009        len = len + 1

0010        label(len) = label(j)

0011  30    continue

c

0012        return

0013        end

DEVICE INDEPENDENT ROUTINES                                            PAGE A-20

0001        FUNCTION ISCRX( xwrld )

0002        common /zgraph/ lunplt, iascr, ibscr, icscr, idscr,

1 xm, ym, dx, dy, dxb, dyb, ixcur, iycur, xmid, ymid,

1 xslope, yslope, xconst, yconst, scrx, scry, ichar(5),

1 ixorig, iyorig

c

0003        x = amin1( xwrld * xslope + xconst, 32767. )

0004        x = amax1 ( x, -32767. )

0005        iscrx = ifix( x )

c

0006        return

0007        end

DEVICE INDEPENDENT ROUTINES                                            PAGE A-21

0001        FUNCTION ISCRY( ywrld )

0002        common /zgraph/ lunplt, iascr, ibscr, icscr, idscr,

1 xm, ym, dx, dy, dxb, dyb, ixcur, iycur, xmid, ymid,

1 xslope, yslope, xconst, yconst, scrx, scry, ichar(5),

1 ixorig, iyorig

c

0003        y = amin1 ( ywrld * yslope + yconst, 32767. )

0004        y = amax1 ( y, -32767. )

0005        iscry = ifix ( y )

c

0006        return

0007        end

DEVICE INDEPENDENT ROUTINES                                            PAGE A-22

0001        FUNCTION XWORLD( ix )

0002        common /zgraph/ lunplt, iascr, ibscr, icscr, idscr,

1 xm, ym, dx, dy, dxb, dyb, ixcur, iycur, xmid, ymid,

1 xslope, yslope, xconst, yconst, scrx, scry, ichar(5),

1 ixorig, iyorig

c

0003        xworld = ( ix - xconst ) / xslope

c

0004        return

0005        end

DEVICE INDEPENDENT ROUTINES                                            PAGE A-23

0001        FUNCTION YWORLD( iy )

0002        common /zgraph/ lunplt, iascr, ibscr, icscr, idscr,

1 xm, ym, dx, dy, dxb, dyb, ixcur, iycur, xmid, ymid,

1 xslope, yslope, xconst, yconst, scrx, scry, ichar(5),

1 ixorig, iyorig

c

0003        yworld = ( iy - yconst ) / yslope

c

0004        return

0005        end

DEVICE INDEPENDENT ROUTINES                                            PAGE A-24

0001        SUBROUTINE CURPOS( icurx, icury )

0002        common /zgraph/ lunplt, iascr, ibscr, icscr, idscr,

1 xm, ym, dx, dy, dxb, dyb, ixcur, iycur, xmid, ymid,

1 xslope, yslope, xconst, yconst, scrx, scry, ichar(5),

1 ixorig, iyorig

c

0003        icurx = ixcur

0004        icury = iycur

0005        return

0006        end

DEVICE INDEPENDENT ROUTINES                                            PAGE A-25

ccc

c  SUBROUTINE PLOT3D

c

c  This routine plots three dimensional surfaces on a rectangular

c  grid with hidden line removal. The array f(nx,ny) contains the

c  values of the surface above a given grid point. The arrays

c  x(nx,ny) and y(nx,ny) are internal work arrays containing the x

c  and y coordinate of the projected surface. The variables nx and

c  ny are the dimensions of the arrays. The variables xw and yw are

c  the size of the projected plot in inches. The variables xview,

c  yview and zview are the position at which the surface is to be

c  viewed in which the grid lies in a rectangle bounded by the

c  diagonal vertices (0,0) and (xw,yw). The variable line determines

c  which directions the grid lines are plotted. For line = 1, only

c  those grid lines parallel to the x-axis are plotted. For line =

c  2, only those grid lines parallel to the y-axis are plotted. For

c  line = 0 all grid lines are plotted. The variable scale

c  determines the scaling factor for the surface to be plotted. If

c  scale = 0, the routine will choose a suitable scaling factor.

c

c

0001        SUBROUTINE PLOT3D(f,x,y,nx,ny,xw,yw,xview,yview,zview,line,scale)

c

0002        dimension f(nx,ny),x(nx,ny),y(nx,ny)

c

0003        common /zgraph/ lunplt, iascr, ibscr, icscr, idscr,

1 xm, ym, pdx, pdy, dxb, dyb, ixcur, iycur, xmido, ymido,

1 xslope, yslope, xconst, yconst, scrx, scry, ichar(5),

1 ixorig, iyorig

0004        common/pscale/xv,yv,dx,dy,ca,sa,ly,lx,xscale,xn,yscale,yn

0005        common/color/isubch

c

0006        data small,pt5/1.e-06,.5/

c

0007        if(line.le.2)goto 10

0009        line=line-3

0010        goto 305

c

c  normalize constants

c

0011  10    xb=1.

0012        yb=yw/xw

0013        xv=xview/xw

0014        yv=yview/xw

0015        h=zview/xw

0016        ir=1

0017        nxx=nx+1

0018        nyy=ny+1

c

c  determine projection plane

c

0019        a=atan2(.5*xb-xv,.5*yb-yv)

0020        sa=sin(a)

DEVICE INDEPENDENT ROUTINES                                            PAGE A-26

0021        ca=cos(a)

c

c  y component of the vertices of the projected grid

c

0022        y2=xb*sa

0023        y3=yb*ca

0024        y4=y2+y3

0025        yc=xv*sa+yv*ca

0026        yn=amin1(0.,y2,y3,y4)

0027        if(yc.ge.yn)yc=yn-.01*abs(yb)

0029        yc=-yc

0030        xc=xv*ca-yv*sa

0031        dx=xb/(nx-1)

0032        dxx=1./dx

0033        vx=dxx*xv+1.00001

0034        lx=vx

0035        dy=yb/(ny-1)

0036        dyy=1./dy

0037        vy=dyy*yv+1.00001

0038        ly=vy

0039        yn=0.

0040        yx=yn

0041        d=.5*y4

0042        yd=yc+d

0043        yp=0.

c

c  determine minimum and maximum of the surface

c

0044        tn=f(1,1)

0045        tx=tn

0046        do 50 j=1,ny

0047        do 50 i=1,nx

0048        tn=amin1(tn,f(i,j))

0049  50    tx=amax1(tx,f(i,j))

c

c  compute scaling factor

c

0050        s=tx-tn

0051        if(s.eq.0.)s=tx

0053        s=amax1(xb,yb)/s

0054        if(scale.ne.0.)s=scale

0055  c

c  compute projected x and y coordinates of surface

c

0056        do 120 j=1,ny

0057        xp=0.

0058        c=yp*sa+xc

0059        do 100 i=1,nx

0060        t=f(i,j)*s

0061        hl=xp*sa+yp*ca

0062        x(i,j)=(xp*ca-c)*yd/(hl+yc)+small

0063        y(i,j)=((hl-d)*h+t*yd)/(hl+yc)+small

0064        yn=amin1(yn,y(i,j))

DEVICE INDEPENDENT ROUTINES                                            PAGE A-27

0065        yx=amax1(yx,y(i,j))

0066  100   xp=xp+dx

0067  120   yp=yp+dy

c

c  x component of the vertices of the grid

c

0068        x1=x(1,1)

0069        x2=x(nx,1)

0070        x3=x(1,ny)

0071        x4=x(nx,ny)

0072        tn=tn*yd*s*1.1

c

c  y component of the vertices of the projected grid

c

0073        y1=(tn-d*h)/yc

0074        y2=((y2-d)*h+tn)/(y2+yc)

0075        y3=((y3-d)*h+tn)/(y3+yc)

0076        y4=((y4-d)*h+tn)/(y4+yc)

c

0077        xn=amin1(x1,x2,x3,x4)

0078        xx=amax1(x1,x2,x3,x4)

0079        yn=amin1(yn,y1,y2,y3,y4)

0080        xscale=scrx*xw/(xx-xn)

0081        yscale=scry*yw/(yx-yn)

0082        xn=xscale*xn

0083        yn=yscale*yn

c

c  convert to screen cordinates

c

0084        do 200 j=1,ny

0085        do 200 i=1,nx

0086        x(i,j)=aint(xscale*x(i,j)-xn+pt5)

0087  200   y(i,j)=aint(yscale*y(i,j)-yn+pt5)

c

0088        x1=aint(xscale*x1-xn+pt5)

0089        y1=aint(yscale*y1-yn+pt5)

0090        x2=aint(xscale*x2-xn+pt5)

0091        y2=aint(yscale*y2-yn+pt5)

0092        x3=aint(xscale*x3-xn+pt5)

0093        y3=aint(yscale*y3-yn+pt5)

0094        x4=aint(xscale*x4-xn+pt5)

0095        y4=aint(yscale*y4-yn+pt5)

0096        xscale=xscale*yd

c

c  begin hidden line removal

c

0097        do 300 j=1,ny

0098        do 300 i=1,nx

0099        ix=isign(1,lx-i)

0100        iy=isign(1,ly-j)

0101        ih=i

0102        iplot=1

0103        if(0.eq.lx-i)goto 250

DEVICE INDEPENDENT ROUTINES                                            PAGE A-28

0105  215   ih=ih+ix

0106        if(ih.lt.1.or.ih.gt.nx)goto 250

0108        xp=(ih-1)*dx

0109        xm=dx/dy*(dy*(j-1)-yv)/(dx*(i-1)-xv)

0110        yp=xm*(ih-i)+j

0111        jj=ifix(yp)

0112        jh=jj+1

0113        if(jj.lt.1.or.jh.gt.ny)goto 250

0115        yp=(yp-1)*dy

0116        hl=yc+xp*sa+yp*ca

0117        xi=aint(xscale*(xp*ca-yp*sa-xc)/hl-xn+pt5)

0118        yi=(y(ih,jj)-y(ih,jh))*(xi-aint(x(ih,jh)))/

1(aint(x(ih,jj))-aint(x(ih,jh)))+y(ih,jh)

0119        if(aint(yi+pt5).lt.y(i,j))goto 215

0121        iplot=0

0122        goto 290

0123  250   if(ly-j.eq.0)goto 290

0125        jh=j

0126  260   jh=jh+iy

0127        if(jh.lt.1.or.jh.gt.ny)goto 290

0129        yp=(jh-1)*dy

0130        ym=dy/dx*(dx*(i-1)-xv)/(dy*(j-1)-yv)

0131        xp=ym*(jh-j)+i

0132        ii=ifix(xp)

0133        ih=ii+1

0134        if(ii.lt.1.or.ih.gt.nx)goto 290

0136        xp=(xp-1)*dx

0137        hl=yc+xp*sa+yp*ca

0138        xi=aint(xscale*(xp*ca-yp*sa-xc)/hl-xn+pt5)

0139        yi=(y(ii,jh)-y(ih,jh))*(xi-aint(x(ih,jh)))/

1(aint(x(ii,jh))-aint(x(ih,jh)))+y(ih,jh)

0140        if(aint(yi+pt5).lt.y(i,j))goto 260

0142        iplot=0

0143  290   x(i,j)=x(i,j)+.101*iplot

0144  300   continue

c

c  plot x grid lines

c

0145  305   if(line.eq.2)goto 350

0147        isubch=1

0148        do 310 j=1,ny

0149        iplot0=ifix(10*(x(1,j)-ifix(x(1,j))))

0150        if(iplot0.eq.1)call plot(ifix(x(1,j)),ifix(y(1,j)),0)

0152        do 310 i=2,nx

0153        iplot=ifix(10*(x(i,j)-aint(x(i,j))))

0154        call insect(i,j,iplot,iplot0,x,y,nx,ny,0)

0155  310   iplot0=iplot

c

c  plot visible verticle bars

c

0156        if(lx.ge.1.and.lx.le.nx)goto 350

0158        if(lx.lt.1)goto 320

0160        ii=nx

DEVICE INDEPENDENT ROUTINES                                            PAGE A-29

0161        aa=(y2-y4)/(x2-x4)

0162        bb=y4-x4*aa

0163        ix=x2

0164        iy=y2

0165        goto 330

0166  320   ii=1

0167        aa=(y3-y1)/(x3-x1)

0168        bb=y1-x1*aa

0169        ix=x1

0170        iy=y1

0171  330   do 340 j=1,ny

0172        call plot(ifix(x(ii,j)),ifix(y(ii,j)),0)

0173  340   call plot(ifix(x(ii,j)),ifix(aa*x(ii,j)+bb),1)

0174        call plot(ix,iy,1)

c

c  plot y grid lines

c

0175  350   if(line.eq.1)return

0177        isubch=2

0178        do 400 i=1,nx

0179        iplot0=ifix(10*(x(i,1)-ifix(x(i,1))))

0180        if(iplot0.eq.1)call plot(ifix(x(i,1)),ifix(y(i,1)),0)

0182        do 400 j=2,ny

0183        iplot=ifix(10*(x(i,j)-aint(x(i,j))))

0184        call insect(i,j,iplot,iplot0,x,y,nx,ny,1)

0185  400   iplot0=iplot

c

c  plot visible verticle bars

c

0186        if(ly.ge.1.and.ly.le.ny)return

0188        if(ly.lt.1)goto 420

0190        jj=ny

0191        aa=(y3-y4)/(x3-x4)

0192        bb=y4-x4*aa

0193        ix=x3

0194        iy=y3

0195        goto 430

0196  420   jj=1

0197        aa=(y2-y1)/(x2-x1)

0198        bb=y1-x1*aa

0199        ix=x1

0200        iy=y1

0201  430   do 440 i=1,nx

0202        call plot(ifix(x(i,jj)),ifix(y(i,jj)),0)

0203  440   call plot(ifix(x(i,jj)),ifix(aa*x(i,jj)+bb),1)

0204        call plot(ix,iy,1)

0205        return

0206        end

DEVICE INDEPENDENT ROUTINES                                            PAGE A-30

ccc

c  SUBROUTINE INSECT

c

c  This routine computes the intersection point of a grid line which is

c  partially hidden.

c

c

0001        SUBROUTINE INSECT(i1,j1,iplot,iplot0,x,y,nx,ny,l)

c

0002        dimension x(nx,ny),y(nx,ny)

0003        byte eq1,eq2

0004        common/pscale/xv,yv,dx,dy,ca,sa,ly,lx,xscale,xn,yscale,yn

0005        data pt5/.5/

0006        if(iplot.eq.iplot0)goto 290

0008        i=i1

0009        j=j1

0010        i0=i1

0011        j0=j1

0012        ix=isign(1,lx-i)

0013        iy=isign(1,ly-j)

0014        insct=0

0015        if(l.eq.0)goto 10

0017        j=j1-iplot0

0018        j0=j1-iplot

0019        xf=aint(x(i0,j0))

0020        yf=y(i0,j0)

0021        xl=aint(x(i,j))

0022        yl=y(i,j)

0023        imin=i1

0024        jmin=iy*min0(iy*j,iy*j0)

0025        if(jmin.eq.j0.and.(y(i+ix,j)-yl)/(x(i+ix,j)-xl).lt.

1(yf-yl)/(xf-xl))goto 270

0027        goto 20

0028  10    i=i1-iplot0

0029        i0=i1-iplot

0030        xf=aint(x(i0,j0))

0031        yf=y(i0,j0)

0032        xl=aint(x(i,j))

0033        yl=y(i,j)

0034        imin=ix*min0(ix*i,ix*i0)

0035        jmin=j1

0036        if(imin.eq.i0.and.(y(i,j+iy)-yl)/(x(i,j+iy)-xl).gt.

1(yf-yl)/(xf-xl))goto 270

0038  20    xint=xf-xl

0039        yint=yf-yl

0040        ih=imin

0041        if(lx-i.eq.0)goto 250

0043        xm1=dx/dy*(dy*(j-1)-yv)/(dx*(i-1)-xv)

0044        xm2=dx/dy*(dy*(j0-1)-yv)/(dx*(i0-1)-xv)

0045  215   ih=ih+ix

0046        if(ih.lt.1.or.ih.gt.nx)goto 250

0048        yp1=xm1*(ih-i)+j

0049        if(i.eq.ih)yp1=yp1+iy

DEVICE INDEPENDENT ROUTINES                                            PAGE A-31

0051        yp2=xm2*(ih-i0)+j0

0052        if(i0.eq.ih)yp2=yp2+iy

0054        jn=ifix(amin1(yp1,yp2))

0055        jx=ifix(amax1(yp1,yp2))

0056        do 220 jj=jn,jx

0057        jh=jj+1

0058        if(jj.lt.1.or.jh.gt.ny)goto 220

0060        if((x(ih,jh)-aint(x(ih,jh)))+(x(ih,jj)-aint(x(ih,jj))).eq.0)

1goto 220

0062        s1=(yf-yl)/(xf-xl)

0063        s2=(y(ih,jj)-y(ih,jh))/(aint(x(ih,jj))-aint(x(ih,jh)))

0064        if(s1.eq.s2)goto 220

0066        xi=(y(ih,jh)-yl+xl*s1-aint(x(ih,jh))*s2)/(s1-s2)

0067        if(abs(aint(xi+pt5)-.5*(xl+xf)).gt..5*abs(xf-xl))goto 220

0069        xmid=.5*aint(x(ih,jj)+x(ih,jh))

0070        xdif=.5*abs(aint(x(ih,jj))-aint(x(ih,jh)))

0071        if(abs(aint(xi+pt5)-xmid).gt.xdif)goto 220

0073        jh=(jj+jh+iy*(jj-jh))/2

0074        yi=s1*(xi-xl)+yl

0075        if(xint*(y(ih,jh+iy)-y(ih,jh))*ix*iy.lt.

1yint*(aint(x(ih,jh+iy))-aint(x(ih,jh)))*ix*iy)goto 220

0077        xf=aint(xi+pt5)

0078        yf=aint(yi+pt5)

0079        insct=1

0080        if(xf-xl.eq.0)goto 270

0082  220   continue

0083        goto 215

0084  250   if(ly-j.eq.0)goto 270

0086        ym1=dy/dx*(dx*(i-1)-xv)/(dy*(j-1)-yv)

0087        ym2=dy/dx*(dx*(i0-1)-xv)/(dy*(j0-1)-yv)

0088        jh=jmin

0089  260   jh=jh+iy

0090        if(jh.lt.1.or.jh.gt.ny)goto 270

0092        xp1=ym1*(jh-j)+i

0093        if(j.eq.jh)xp1=xp1+ix

0095        xp2=ym2*(jh-j0)+i0

0096        if(j0.eq.jh)xp2=xp2+ix

0098        jn=ifix(amin1(xp1,xp2))

0099        jx=ifix(amax1(xp1,xp2))

0100        do 265 ii=jn,jx

0101        ih=ii+1

0102        if(ii.lt.1.or.ih.gt.nx)goto 265

0104        if((x(ih,jh)-aint(x(ih,jh)))+(x(ii,jh)-aint(x(ii,jh))).eq.0)

1goto 265

0106        eq1=i.eq.ih.or.i.eq.ii

0107        if(eq1.and.eq2)goto 265

0109        s1=(yf-yl)/(xf-xl)

0110        s2=(y(ii,jh)-y(ih,jh))/(aint(x(ii,jh))-aint(x(ih,jh)))

0111        if(s1.eq.s2)goto 265

0113        xi=(y(ih,jh)-yl+xl*s1-aint(x(ih,jh))*s2)/(s1-s2)

0114        if(abs(aint(xi+pt5)-.5*(xl+xf)).gt..5*abs(xf-xl))goto 265

0116        xmid=.5*aint(x(ii,jh)+x(ih,jh))

0117        xdif=.5*abs(aint(x(ii,jh))-aint(x(ih,jh)))

DEVICE INDEPENDENT ROUTINES                                            PAGE A-32

0118        if(abs(aint(xi+pt5)-xmid).gt.xdif)goto 265

0120        ih=(ih+ii+ix*(ii-ih))/2

0121        yi=s1*(xi-xl)+yl

0122        if(xint*(y(ih+ix,jh)-y(ih,jh))*ix*iy.gt.

1yint*(aint(x(ih+ix,jh))-aint(x(ih,jh)))*ix*iy)goto 265

0124        xf=aint(xi+pt5)

0125        yf=aint(yi+pt5)

0126        insct=1

0127        if(xf.eq.xl)goto 270

0129  265   continue

0130        goto 260

0131  270   if(insct.eq.1)goto 280

0133        if(l.eq.0.and.i.eq.1.or.i.eq.nx)return

0135        if(l.eq.1.and.j.eq.1.or.j.eq.ny)return

0137        call plot(ifix(xl),ifix(yl),0)

0138        return

0139  280   ix=ifix(xf)

0140        iy=ifix(yf)

0141        call plot(ix,iy,iplot0)

0142        if(xf.eq.xl)return

0144  290   if(iplot.eq.0)return

0146        ix=ifix(x(i1,j1))

0147        iy=ifix(y(i1,j1))

0148        call plot(ix,iy,1)

0149        return

0150        end

APPENDIX B

HIPLOT ROUTINES

0001        SUBROUTINE INIPLT(iunit, xsize, ysize)

c

c     this routine initializes the plot. xsize and ysize denote the

c     size of the total plotting surface in inches. no plotting is

c     permitted outside this area. iunit is the logical unit

c     number of the plot. for iunit = 6, the plot is sent directly

c     to the plotter.

c

0002        common /zgraph/ lunplt, iascr, ibscr, icscr, idscr,

1 xm, ym, dx, dy, dxb, dyb, ixcur, iycur, xmid, ymid,

1 xslope, yslope, xconst, yconst, scrx, scry, ichar(5),

1 ixorig, iyorig

c

0003        common /pagsiz/ xbond, ybond

c

0004        common /world/ xmin, xmax, ymin, ymax

c

0005        lunplt = iunit

0006        if ( iunit .eq. 7 ) call plton

0008        write(lunplt, 1)

0009  1     format(' ;: HOAUL0 ')

c

0010        scrx = 200.

0011        scry = 200.

c

0012        xbond = xsize

0013        ybond = ysize

0014        ixbond = ifix(scrx * xsize)

0015        iybond = ifix(scry * ysize)

0016        dxb = .5 * ixbond

0017        dyb = .5 * iybond

0018        xmid = dxb

0019        ymid = dyb

c

c     set default values

c

0020        ixorig = 0

0021        iyorig = 0

0022        iascr = 0

0023        ibscr = ixbond

HIPLOT ROUTINES                                                         PAGE B-2

0024        icscr = 0

0025        idscr = iybond

c

0026        xslope = ixbond

0027        xconst = 0.

0028        yslope = iybond

0029        yconst = 0.

c

0030        xmin = 0.

0031        xmax = 1.

0032        ymin = 0.

0033        ymax = 1.

c

0034        do 10 i = 1 , 5

0035        ichar(i) = 6 * 2 ** i

0036  10    continue

c

0037        return

0038        end

HIPLOT ROUTINES                                                         PAGE B-3

0001        SUBROUTINE ENDPLT

c

c     this routine ends plotting by deselecting the plotter and

c     dumping the buffer

c

0002        common /zgraph/ lunplt, iascr, ibscr, icscr, idscr,

1 xm, ym, dx, dy, dxb, dyb, ixcur, iycur, xmid, ymid,

1 xslope, yslope, xconst, yconst, scrx, scry, ichar(5),

1 ixorig, iyorig

c

0003        call dmpplt

0004        if ( lunplt .eq. 7 ) call pltoff

0005  c

0006        return

0007        end

HIPLOT ROUTINES                                                         PAGE B-4

0001        SUBROUTINE DMPPLT

c

c     this routine ends plotting by deselecting the plotter and

c     dumping the buffer

c

0002        common /zgraph/ lunplt, iascr, ibscr, icscr, idscr,

1 xm, ym, dx, dy, dxb, dyb, ixcur, iycur, xmid, ymid,

1 xslope, yslope, xconst, yconst, scrx, scry, ichar(5),

1 ixorig, iyorig

c

0003        write(lunplt, 1)

0004        write(lunplt, 2)

0005        write(lunplt, 2)

0006        write(lunplt, 3)

c

0007        return

0008  1     format(1x, 'HUP0 @', 122x)

0009  2     format(129x)

0010  3     format(123x, ';: HU ')

0011        end

HIPLOT ROUTINES                                                         PAGE B-5

0001        SUBROUTINE PENDWN

c

c     this routine puts the pen down so it will draw

c

0002        common /zgraph/ lunplt, iascr, ibscr, icscr, idscr,

1 xm, ym, dx, dy, dxb, dyb, ixcur, iycur, xmid, ymid,

1 xslope, yslope, xconst, yconst, scrx, scry, ichar(5),

1 ixorig, iyorig

c

0003        write (lunplt, 1)

0004  1     format (1x, ' D ')

0005        return

0006        end

HIPLOT ROUTINES                                                         PAGE B-6

0001        SUBROUTINE PENUP

c

c     this routine puts the pen up

c

0002        common /zgraph/ lunplt, iascr, ibscr, icscr, idscr,

1 xm, ym, dx, dy, dxb, dyb, ixcur, iycur, xmid, ymid,

1 xslope, yslope, xconst, yconst, scrx, scry, ichar(5),

1 ixorig, iyorig

c

0003        write (lunplt, 1)

0004  1     format (1x, ' U ')

0005        return

0006        end

HIPLOT ROUTINES                                                         PAGE B-7

0001        SUBROUTINE WRTSTR(str, len, irot, isize)

c

c     subroutine "write string"

c     len is the length of the string

c     irot determines the rotation of the string (1 - 4), 1 is

c     right side up, 2 is rotated 90 degrees clockwise, etc.

c     string is checked to see if in is in bounds

c

0002        common /zgraph/ lunplt, iascr, ibscr, icscr, idscr,

1 xm, ym, dx, dy, dxb, dyb, ixcur, iycur, xmid, ymid,

1 xslope, yslope, xconst, yconst, scrx, scry, ichar(5),

1 ixorig, iyorig

c

0003        byte str(len)

c

c     check to see if in bounds

c

0004        isz = max0(isize, 1)

0005        isz = min0(isz, 5)

0006        ix = 2 ** (isz - 1)

0007        iy = 14 * ix

0008        ix = 12 * ix * len

0009        irt = mod(irot - 1, 4) + 1

0010        isx = iabs(irt - 3) - 1

0011        isy = iabs(irt - 2) - 1

0012        ixl = isx * ix - isy * iy + ixcur

0013        iyl = isx * iy + isy * ix + iycur

0014        if( abs(ixl - xmid) .gt. dxb )goto 100

0016        if( abs(iyl - ymid) .gt. dyb )goto 100

0017  c

c     plot string

c

0018        write (lunplt, 3) irt, isz, (str(i), i = 1, len)

0019        write (lunplt, 4)

0020        return

c

c     string is out of bounds

c

0021  100   if ( lunplt .eq. 7 ) call pltoff

0023        type 1, (str(i), i = 1, len)

0024        type 2

0025        if ( lunplt .eq. 7 ) call plton

0026  c

0027        return

0028  1     format('$WARNING  string ', 100a1)

0029  2     format('  is out of bounds and will not be plotted')

0030  3     format (1x, 'S', 2i1, 1x, 100a1, $)

0031  4     format(1x, '')

0032        end

HIPLOT ROUTINES                                                         PAGE B-8

0001        SUBROUTINE MARKER(mrknum, isize)

c

c     places a marker at the current coordinates

c

0002        common /zgraph/ lunplt, iascr, ibscr, icscr, idscr,

1 xm, ym, dx, dy, dxb, dyb, ixcur, iycur, xmid, ymid,

1 xslope, yslope, xconst, yconst, scrx, scry, ichar(5),

1 ixorig, iyorig

c

0003        isz = min0(isize, 5)

0004        isz = max0(isz, 1)

0005        mrk = max0(mrknum, 0)

0006        mrk = min0(mrk, 5)

0007        write (lunplt, 1) isz, mrk

0008  1     format(1x, ' M', 2i1, 1x)

0009        return

0010        end

HIPLOT ROUTINES                                                         PAGE B-9

0001        SUBROUTINE COLTYP (icol)

c

c     this subroutine sets the line color

c

0002        common /zgraph/ lunplt, iascr, ibscr, icscr, idscr,

1 xm, ym, dx, dy, dxb, dyb, ixcur, iycur, xmid, ymid,

1 xslope, yslope, xconst, yconst, scrx, scry, ichar(5),

1 ixorig, iyorig

c

0003        itype = icol + 1

0004        itype = min0(itype, 8)

0005        itype = max0(itype, 1)

0006        write (lunplt, 1) itype

0007  1     format (1x, ' P', i1)

0008        return

0009        end

HIPLOT ROUTINES                                                        PAGE B-10

0001        SUBROUTINE PLOT(ixo, iyo, i)

c

c     this routine moves to the screen coordinates ix, iy.

c     if i = 0, the pen is put up before moving, if i = 1, the

c     pen is put down before moving. checks to see if within

c     surface boundary.

c

0002        common /zgraph/ lunplt, iascr, ibscr, icscr, idscr,

1 xm, ym, dx, dy, dxb, dyb, ixcur, iycur, xmid, ymid,

1 xslope, yslope, xconst, yconst, scrx, scry, ichar(5),

1 ixorig, iyorig

c

0003        if(i .eq. 0)call penup

0005        if(i .ne. 0)call pendwn

0007        ix = ixo

0008        iy = iyo

c

c     check to see if in bounds

c

0009        delx = ix - xmid

0010        if(abs(delx) .le. dxb)goto 100

0012        xsc = xmid + sign(dxb, delx)

0013        xsl = 1.

0014        if ( ix .ne. ixcur ) xsl = ( xsc - ixcur ) / float(ix - ixcur)

0016        xsl = (iy - iycur) * xsl

0017        ix = xsc

0018        iy = xsl + iycur

c

0019  100   dely = iy - ymid

0020        if(abs(dely) .le. dyb)goto 200

0022        ysc = ymid + sign(dyb, dely)

0023        ysl = 1.

0024        if ( iy .ne. iycur ) ysl = ( ysc - iycur ) / float(iy - iycur)

0026        ysl = (ix - ixcur) * ysl

0027        ix = ysl + ixcur

0028        iy = ysc

c

c     plot point

c

0029  200   write(lunplt, 1)ix, iy

c

c     save current coordinates

c

0030        ixcur = ix

0031        iycur = iy

c

0032  1     format(1x, 2(i4, 1x))

0033        return

0034        end

HIPLOT ROUTINES                                                        PAGE B-11

0001        SUBROUTINE PLOTIN(x, y, i)

c

c     plots the point x, y where x and y are given in inches.

c     if i = 0, pen is put up before plotting, else pen is put down.

c

0002        ix = ifix(200 * x)

0003        iy = ifix(200 * y)

0004        call plot(ix, iy, i)

0005        return

0006        end

HIPLOT ROUTINES                                                        PAGE B-12

0001        SUBROUTINE ERASE

c

c     this is a do-nothing subroutine supplied for compatibility with

c     the GRINELL version of the software.

0002        return

0003        end

HIPLOT ROUTINES                                                        PAGE B-13

0001        SUBROUTINE PLTON

c

0002        common /zgraph/ iunit, iascr, ibscr, icscr, idscr,

1 xm, ym, dx, dy, dxb, dyb, ixcur, iycur, xmid, ymid,

1 xslope, yslope, xconst, yconst, scrx, scry, ichar(5),

1 ixorig, iyorig

c

0003        byte esc

0004        esc = 27

0005        write(iunit, 1) esc

0006  1     format ( 1x, a1, '[?7i' )

0007        return

0008        end

HIPLOT ROUTINES                                                        PAGE B-14

0001        SUBROUTINE PLTOFF

c

0002        common /zgraph/ iunit, iascr, ibscr, icscr, idscr,

1 xm, ym, dx, dy, dxb, dyb, ixcur, iycur, xmid, ymid,

1 xslope, yslope, xconst, yconst, scrx, scry, ichar(5),

1 ixorig, iyorig

c

0003        byte esc

0004        esc = 27

0005        write(iunit, 1) esc

0006  1     format( 1x, a1, '[?6i' )

0007        return

0008        end

APPENDIX C

GRINNELL ROUTINES

0001        SUBROUTINE INIPLT(iunit,x,y)

0002        integer data(8)

0003        common /zgraph/ lunplt, iascr, ibscr, icscr, idscr,

1 xm, ym, dx, dy, dxb, dyb, ixcur, iycur, xmid, ymid,

1 xslope, yslope, xconst, yconst, scrx, scry, ichar(5),

1 ixorig, iyorig

c

0004        common /pagsiz/ xbond, ybond

c

0005        common /world/ xmin, xmax, ymin, ymax

c

0006        common/grbufr/ibufl,ibuf(1000)

0007        data data,ibufl/512,4095,15,111,255,240,3840,2309,1000/

c

0008        call grsini

0009        call grfer("7777,"7777,0)

0010        call grlwr(1,data,0,8,0)

c

0011        scrx = 50.

0012        scry = 50.

c

0013        xbond = x

0014        ybond = y

0015        ixbond = scrx * x

0016        iybond = scry * y

0017        dxb = .5 * ixbond

0018        dyb = .5 * iybond

0019        xmid = dxb

0020        ymid = dyb

c

c     set default values

c

0021        ixorig = 0

0022        iyorig = 0

0023        iascr = 0

0024        ibscr = ixbond

0025        icscr = 0

0026        idscr = iybond

c

0027        xslope = ixbond

GRINNELL ROUTINES                                                       PAGE C-2

0028        xconst = 0

0029        yslope = iybond

0030        yconst = 0

c

0031        xmin = 0.

0032        xmax = 1.

0033        ymin = 0.

0034        ymax = 1.

c

0035        ichar(1) = 7

0036        ichar(2) = 7

0037        ichar(3) = 14

0038        ichar(4) = 14

0039        ichar(5) = 14

c

0040        return

0041        end

GRINNELL ROUTINES                                                       PAGE C-3

0001        SUBROUTINE ENDPLT

0002        call grsend

0003        return

0004        end

GRINNELL ROUTINES                                                       PAGE C-4

0001        SUBROUTINE DMPPLT

0002        call grsbfd

0003        return

0004        end

GRINNELL ROUTINES                                                       PAGE C-5

0001        SUBROUTINE WRTSTR(str, len, irot, isize)

c

c     subroutine "write string"

c     len is the length of the string

c     irot determines the rotation of the string (1 - 4), 1 is

c     right side up, 2 is rotated 90 degrees clockwise, etc.

c     string is checked to see if in is in bounds

c

0002        common /zgraph/ lunplt, iascr, ibscr, icscr, idscr,

1 xm, ym, dx, dy, dxb, dyb, ixcur, iycur, xmid, ymid,

1 xslope, yslope, xconst, yconst, scrx, scry, ichar(5),

1 ixorig, iyorig

c

0003        byte str(len)

0004        byte charb(886)

0005        integer char(223), length(223), matrix(8)

0006        data charb/

1 "370,"220,"160," 20,"340,"210,"210,"210,"370,"210,"210,"160,

1 "360,"210,"210,"360,"200,"200,"200,"360,"210,"210,"360,"210,

1 "210,"360,"170,"200,"200,"200,"170,"160,"210,"200,"200,"200,

1 "210,"160,"170,"210,"210,"170," 10," 10," 10,"360,"210,"210,

1 "210,"210,"210,"360,"170,"200,"370,"210,"160,"370,"200,"200,

1 "360,"200,"200,"370,"100,"100,"100,"340,"100,"110," 60,"200,

1 "200,"200,"360,"200,"200,"370,"160,"210," 10,"170,"210,"210,

1 "170,"160,"210,"270,"200,"200,"210,"160,"210,"210,"210,"360,

1 "200,"200,"200,"210,"210,"210,"370,"210,"210,"210,"160," 40,

1 " 40,"140," 00," 40,"370," 40," 40," 40," 40," 40,"370,"140,

1 "220," 20," 20," 20," 20," 00," 20,"140,"220," 20," 20," 20,

1 " 20," 70,"210,"220,"340,"240,"220,"200,"200,"210,"220,"240,

1 "300,"240,"220,"210,"340,"100,"100,"100,"100,"100,"300,"370,

1 "200,"200,"200,"200,"200,"200,"210,"250,"250,"370,"220,"210,

1 "210,"210,"250,"250,"330,"210,"210,"210,"210,"310,"260,"210,

1 "230,"230,"250,"310,"310,"210,"160,"210,"210,"210,"160,"160,

1 "210,"210,"210,"210,"210,"160,"200,"200,"200,"360,"210,"210,

1 "360,"200,"200,"200,"360,"210,"210,"360," 10," 10," 10,"170,

1 "210,"210,"170,"150,"220,"250,"210,"210,"210,"160,"200,"200,

1 "200,"310,"260,"210,"220,"240,"360,"210,"210,"360,"360," 10,

1 "160,"200,"170,"160,"210," 10,"160,"200,"210,"160," 60,"110,

1 "100,"100,"340,"100,"100," 40," 40," 40," 40," 40," 40,"370,

1 "170,"210,"210,"210,"210,"160,"210,"210,"210,"210,"210,"210,

1 " 40,"120,"210,"210,"210," 40," 40,"120,"120,"210,"210,"210,

1 "210,"330,"250,"210,"210,"210,"330,"250,"250,"210,"210,"210,

1 "210,"120," 40,"120,"210,"210,"210,"120," 40,"120,"210,"210,

1 "160,"210," 10,"170,"210,"210,"210," 40," 40," 40," 40,"120,

1 "210,"210,"370,"100," 40," 20,"370,"370,"200,"100," 40," 20,

1 " 10,"370,"370,"210,"210,"120,"120," 40," 40," 40," 00," 40,

1 " 40," 40," 40," 40,"120,"120,"120,"120,"120,"370,"120,"370,

1 "120,"120," 40,"360," 50,"160,"240,"170," 40,"210,"200,"100,

1 " 40," 20," 10,"210,"170,"220,"250,"140,"120,"120," 40,"100,

1 " 40," 40," 40,"100,"100,"100,"100,"100,"100,"100," 40,"100,

1 " 40," 40," 40," 40," 40," 40," 40,"100,"120," 40,"370," 40,

1 "120," 40," 40,"370," 40," 40,"100," 40," 40,"370," 40,"200,

1 "100," 40," 20," 10,"160,"210,"310,"250,"230,"210,"160,"160,

GRINNELL ROUTINES                                                       PAGE C-6

1 " 40," 40," 40," 40,"140," 40,"370,"100," 40," 20," 10,"210,

1 "160,"160,"210," 10," 60," 10,"210,"160," 20," 20,"370,"220,

1 "120," 60," 20,"160,"210," 10," 10,"360,"200,"370,"160,"210,

1 "210,"360,"200,"210,"160,"100,"100,"100," 40," 20," 10,"370,

1 "160,"210,"210,"160,"210,"210,"160,"160,"210," 10,"170,"210,

1 "210,"160," 40," 00," 00," 40,"100," 40," 40," 00," 40," 20,

1 " 40,"100," 40," 20,"370," 00," 00,"370,"100," 40," 20," 40,

1 "100," 40," 00," 40," 20," 10,"210,"160,"170,"200,"270,"250,

1 "270,"210,"160,"160,"100,"100,"100,"100,"100,"100,"100,"160,

1 " 10," 20," 40,"100,"200,"160," 20," 20," 20," 20," 20," 20,

1 " 20,"160,"210,"120," 40,"377," 40,"100,"100," 70," 20,"174,

1 "222,"222,"222,"174," 20," 70,"314,"110,"110,"204,"204,"204,

1 "170,"374,"100," 40," 20," 10," 20," 40,"100,"374,"340,"100,

1 "100,"100,"100,"100,"100,"104,"374,"356,"104,"104," 50," 50,

1 " 20," 20,"204,"374,"204," 00,"170," 00,"204,"374,"204," 70,

1 " 20," 20," 70,"124,"124,"124,"222," 70," 70," 20," 20," 20,

1 " 20," 50," 50,"252,"104,"356,"104,"104,"104,"104,"104,"104,

1 "104,"376,"170,"204,"200,"200,"160,"200,"204,"170,"170,"204,

1 "204,"204,"264,"204,"204,"170,"140,"220,"220," 20," 20," 20,

1 " 20," 20," 20," 20," 20," 20," 20," 20," 20," 20," 20," 20,

1 " 20," 20," 20," 20," 20," 20," 20," 20," 20," 22," 22," 14,

1 " 20," 20," 20," 20," 20," 20," 20," 20," 20,"154,"220,"220,

1 "220,"220,"150,"200,"200,"360,"210,"210,"360,"210,"210,"160,

1 "140," 20,"160,"200,"200,"160,"100,"100," 70,"100,"160,"210,

1 "210,"160," 40,"100,"100," 70,"160,"200,"200,"340,"200,"200,

1 "160,"100,"100,"160,"250,"250,"160," 20," 20," 40," 40," 20,

1 " 20," 30,"224,"144," 04," 04," 04," 44," 44," 44,"244,"130,

1 "140,"220,"200,"100,"100," 40,"104,"110," 50," 60,"250,"144,

1 "210,"210,"110,"120," 60," 40," 40,"100,"200,"200,"350,"220,

1 "220,"220,"220,"220,"300,"240,"220,"110,"110,"310,"140,"220,

1 "210,"210,"110," 60," 40," 50," 50," 50,"250,"170,"140,"220,

1 "220,"360,"220,"220,"140,"200,"200,"200,"340,"220,"220,"220,

1 "140,"160,"210,"210,"210,"210,"174," 40," 40," 40," 40,"240,

1 "170," 30," 44," 44," 44,"244,"144,"154,"272,"222,"222,"202,

1 "104,"114," 52," 20," 20,"250,"144," 40," 40,"160,"250,"250,

1 "250," 40," 40,"140,"220," 20,"140,"200,"200,"100,"170,"200,

1 " 30," 40," 40," 40,"100," 40," 40," 40," 30,"300," 40," 40,

1 " 40," 20," 40," 40," 40,"300," 20,"250,"100,"  0

1 /

0007        data char/

1 30 * 886,

1 886,886,346,353,356,363,370,377,384,387,396,405,410,415,418,

1 419,420,425,432,439,446,453,460,467,474,481,488,495,499,504,

1 509,513,518,525,006,020,032,046,058,072,086,100,113,128,142,

1 156,168,180,192,206,220,232,244,258,270,282,294,306,320,332,

1 532,541,546,555,558,886,001,013,027,039,053,065,079,093,107,

1 120,135,149,163,175,187,199,213,227,239,251,265,277,289,301,

1 313,327,865,685,874,883,886,886,886,886,886,886,886,886,886,

1 15 * 886,

1 886,886,886,886,886,886,886,886,886,886,886,886,603,339,639,

1 562,587,886,886,655,886,596,886,886,886,630,647,886,578,886,

1 621,886,571,886,612,886,886,886,886,886,886,559,694,700,709,

1 719,727,734,742,749,757,886,763,769,777,785,791,797,803,810,

GRINNELL ROUTINES                                                       PAGE C-7

1 818,824,830,886,836,842,848,856,886,886,886,886,886

1 /

0008        data length/

1 4122,4122,4122,-3094,3*4122,-10256,12*4122,-3102,9*4122,

1 4122,4122,4218,4158,4218,4218,4218,4218,4158,4248,

1 4248,4187,4187,4154,4125,4122,4187,4218,4218,4218,

1 4218,4218,4218,4218,4218,4218,4218,4171,4186,4187,

1 4171,4187,4218,4218,4218,4218,4218,4218,4218,4218,

1 4218,4218,4218,4218,4218,4218,4218,4218,4218,4218,

1 4218,4218,4218,4218,4218,4218,4218,4218,4218,4218,

1 4248,4187,4248,4156,4122,4122,4186,4218,4186,4218,

1 4186,4218,4216,4218,4202,4232,4218,4218,4186,4186,

1 4186,4216,4216,4186,4186,4218,4186,4186,4186,4186,

1 4216,4186,4248,4248,4248,4156,4122,4122,4122,4122,

1 4122,4122,4122,4122,4122,4122,4122,4122,4122,4122,

1 20*4122,

1 4122,4122,4250,4218,4234,4250,4250,4122,4122,4576,

1 4122,4218,4122,4122,4122,4250,4234,4122,4250,4122,

1 4250,4122,4218,4122,4250,4122,4122,4122,4122,4122,

1 4122,4158,4202,4248,4264,4234,4218,4234,4218,4232,

1 4202,0026,4202,4234,4232,4202,4202,4202,4218,4232,

1 4202,4202,4202,4122,4202,4202,4234,4248,4122,4122,

1 4122,4122,4122

1 /

c

c     check to see if in bounds

c

0009        isz = max0(isize, 1)

0010        isz = min0(isz, 5)

0011        id = 0

0012        if ( isz .ge. 3 ) id = 1

0014        len1 = len

0015        do 5 i = 1, len

0016        if ( str(i) .eq. 96 ) len1 = len1 - 1

0018        if ( str(i) .le. 31 ) len1 = len1 - 1

0020        if ( str(i) .eq. 8 ) len1 = len1 - 1

0022  5     continue

0023        ix = ichar(isz) * len1

0024        iy = (7 * ichar(isz)) / 6

0025        irt = mod(max0(irot - 1, 0), 4) + 1

0026        isx = iabs(irt - 3) - 1

0027        isy = iabs(irt - 2) - 1

0028        ixl = isx * ix - isy * iy + ixcur

0029        iyl = isx * iy + isy * ix + iycur

0030        if( abs( ixl - xmid ) .gt. dxb )goto 100

0032        if( abs( iyl - ymid ) .gt. dyb )goto 100

0033  c

c     plot string

c

0034        ix = ixcur

0035        iy = iycur

0036        iesc = 0

0037        lnx = mod ( ( id + 1 ) * isx + 1024, 1024 )

GRINNELL ROUTINES                                                       PAGE C-8

0038        lny = mod ( ( id + 1 ) * isy + 1024, 1024 )

0039        do 20 i = 1 , len

0040           nchar = str(i)

0041           nchar = nchar .and. "377

0042           nchar = nchar + iesc

0043           iesc = 96

0044           if ( nchar .eq. 96 ) goto 20

0046           iesc = 0

0047           n = length( nchar )

0048           ioff = ( mod( iabs(n) , 16 ) - 10 ) * ( id + 1 )

0049           idx = n / 16

0050           n = mod( iabs(idx) , 64 )

0051           istr = char( nchar )

0052           jj = 0

0053           ixl = ix - ioff * isy

0054           iyl = iy + ioff * isx

0055           do 15 ii = 1, n

0056        	nchar = charb(ii + istr - 1) .and. "377

0057        	do 10 j = 1, 8

0058        	ntwo = 2 ** ( 8 - j )

0059        	matrix(j) = nchar / ntwo

0060        	nchar = mod ( nchar , ntwo )

0061  10       continue

0062           call grwdw(matrix,ixl,iyl,lnx,lny,8,id,id,1,0)

0063           ixl = ixl - isy * ( id + 1 )

0064           iyl = iyl + isx * ( id + 1 )

0065  15       continue

0066           inc = ( 3 + idx / 64 ) * ( id + 1 )

c

0067           if ( inc .ne. 0 ) goto 17

0069        	ix = ix - ioff * isy

0070        	iy = iy + ioff * isx

c

0071  17       ix = ix + inc * isx

0072           iy = iy + inc * isy

0073  20    continue

0074        return

c

c     string is out of bounds

c

0075  100   type 1, (str(i), i = 1, len)

0076        type 2

c

0077        return

0078  1     format('$WARNING  string ', 100a1)

0079  2     format('  is out of bounds and will not be plotted')

0080        end

GRINNELL ROUTINES                                                       PAGE C-9

0001        SUBROUTINE MARKER(mrknum, isize)

c

c     places a marker at the current coordinates

c

0002        common /zgraph/ lunplt, iascr, ibscr, icscr, idscr,

1 xm, ym, dx, dy, dxb, dyb, ixcur, iycur, xmid, ymid,

1 xslope, yslope, xconst, yconst, scrx, scry, ichar(5),

1 ixorig, iyorig

c

0003        isz = min0(isize, 5)

0004        isz = max0(isz, 1)

0005        mrk = max0(mrknum, 0)

0006        mrk = min0(mrk, 5)

0007        return

0008        end

GRINNELL ROUTINES                                                      PAGE C-10

0001        SUBROUTINE COLTYP (ityp)

c

c     this subroutine sets the line type

c

0002        common /zgraph/ lunplt, iascr, ibscr, icscr, idscr,

1 xm, ym, dx, dy, dxb, dyb, ixcur, iycur, xmid, ymid,

1 xslope, yslope, xconst, yconst, scrx, scry, ichar(5),

1 ixorig, iyorig

c

0003        common/color/isc

c

0004        itype = min0(ityp, 8)

0005        itype = max0(itype, 0)

0006        isc = itype + 1

0007        return

0008        end

GRINNELL ROUTINES                                                      PAGE C-11

0001        SUBROUTINE PLOT(ixo ,iyo ,ip)

0002        common /zgraph/ lunplt, iascr, ibscr, icscr, idscr,

1 xm, ym, dx, dy, dxb, dyb, ixcur, iycur, xmid, ymid,

1 xslope, yslope, xconst, yconst, scrx, scry, ichar(5),

1 ixorig, iyorig

c

0003        common/color/isc

0004        integer idata(4)

0005        ix = ixo

0006        iy = iyo

c

c     check to see if in bounds

c

0007        delx = ix - xmid

0008        if(abs(delx) .le. dxb)goto 100

0010        xsc = xmid + sign(dxb, delx)

0011        xsl = 1.

0012        if ( ix .ne. ixcur ) xsl = ( xsc - ixcur ) / float(ix - ixcur)

0014        xsl = (iy - iycur) * xsl

0015        ix = xsc

0016        iy = xsl + iycur

c

0017  100   dely = iy - ymid

0018        if(abs(dely) .le. dyb)goto 200

0020        ysc = ymid + sign(dyb, dely)

0021        ysl = 1.

0022        if ( iy .ne. iycur ) ysl = ( ysc - iycur ) / float(iy - iycur)

0024        ysl = (ix - ixcur) * ysl

0025        ix = ysl + ixcur

0026        iy = ysc

c

c     save current coordinates

c

0027  200   ixcur = ix

0028        iycur = iy

c

c     plot point

c

0029        idata(3)=ix

0030        idata(4)=iy

0031        if(ip.eq.1)call grfvl(1,isc,0,1,idata,2)

0033        idata(1)=ix

0034        idata(2)=iy

0035        return

0036        end

GRINNELL ROUTINES                                                      PAGE C-12

0001        SUBROUTINE PLOTIN(x, y, i)

c

c     plots the point x, y where x and y are given in inches.

c     if i = 0, pen is put up before plotting, else pen is put down.

c

0002        ix = ifix(50 * x)

0003        iy = ifix(50 * y)

0004        call plot(ix, iy, i)

0005        return

0006        end

GRINNELL ROUTINES                                                      PAGE C-13

0001        SUBROUTINE ERASE

c

c     erases the entire screen and sets it to the background color

c

0002        call grfer("7777, "7777, 0)

0003        return

0004        end

GRINNELL ROUTINES                                                      PAGE C-14

0001        SUBROUTINE PLTON

0002        return

0003        end

GRINNELL ROUTINES                                                      PAGE C-15

0001        SUBROUTINE PLTOFF

0002        return

0003        end

APPENDIX D

VISUAL ROUTINES

0001        SUBROUTINE INIPLT(iunit, xsize, ysize)

c

c     this routine initializes the plot. xsize and ysize denote the

c     size of the total plotting surface in inches. no plotting is

c     permitted outside this area. iunit is the logical unit

c     number of the plot. for iunit = 6, the plot is sent directly

c     to the plotter.

c

0002        common /zgraph/ lunplt, iascr, ibscr, icscr, idscr,

1 xm, ym, dx, dy, dxb, dyb, ixcur, iycur, xmid, ymid,

1 xslope, yslope, xconst, yconst, scrx, scry, ichar(5),

1 ixorig, iyorig

c

0003        common /pagsiz/ xbond, ybond

c

0004        common /world/ xmin, xmax, ymin, ymax

c

0005        byte vtbufr(1024)

0006        integer irec, vtpos, vtstrt, vtbufl

0007        common / vtbuf/ vtbufr, irec, vtpos, vtstrt, vtbufl

c

0008        if ( iunit .eq. 7 ) goto 100

0010        open( unit=iunit, access='DIRECT', recordsize=128, initialsize=32,

1  associatevariable=irec )

0011        irec = 1

0012        vtbufl = 512

0013        vtstrt = 0

0014        vtpos = 0

0015  100   lunplt = iunit

c

c     set terminal to transparent mode TSX command

c

0016        call writch(29)

0017        call writch(77)

0018        call erase

c

0019        scrx = 75.

0020        scry = 75.

c

0021        xbond = xsize

VISUAL ROUTINES                                                         PAGE D-2

0022        ybond = ysize

0023        ixbond = ifix(scrx * xsize)

0024        iybond = ifix(scry * ysize)

0025        dxb = .5 * ixbond

0026        dyb = .5 * iybond

0027        xmid = dxb

0028        ymid = dyb

c

c     set default values

c

0029        ixorig = 0

0030        iyorig = 0

0031        iascr = 0

0032        ibscr = ixbond

0033        icscr = 0

0034        idscr = iybond

c

0035        xslope = ixbond

0036        xconst = 0.

0037        yslope = iybond

0038        yconst = 0.

c

0039        xmin = 0.

0040        xmax = 1.

0041        ymin = 0.

0042        ymax = 1.

c

0043        ichar(1) = 7

0044        ichar(2) = 10

0045        ichar(3) = 19

0046        ichar(4) = 29

0047        ichar(5) = 38

c

0048        return

0049        end

VISUAL ROUTINES                                                         PAGE D-3

0001        SUBROUTINE ENDPLT

c

c     this routine ends plotting by deselecting the plotter and

c     dumping the buffer

c

0002        common /zgraph/ lunplt, iascr, ibscr, icscr, idscr,

1 xm, ym, dx, dy, dxb, dyb, ixcur, iycur, xmid, ymid,

1 xslope, yslope, xconst, yconst, scrx, scry, ichar(5),

1 ixorig, iyorig

c

0003        call pltoff

0004        if ( lunplt .ne. 7 ) call dmpplt

0006        return

0007        end

VISUAL ROUTINES                                                         PAGE D-4

0001        SUBROUTINE PENDWN

c

c     this routine puts the pen down so it will draw

c

0002        common /zgraph/ lunplt, iascr, ibscr, icscr, idscr,

1 xm, ym, dx, dy, dxb, dyb, ixcur, iycur, xmid, ymid,

1 xslope, yslope, xconst, yconst, scrx, scry, ichar(5),

1 ixorig, iyorig

c

c    <ESC>/0d	turns on dots

c

0003        call writch(27)

0004        call writch(47)

0005        call writch(48)

0006        call writch(100)

c

c     GS (])		turns on vector mode

c

0007        call writch(29)

c

0008        return

0009        end

VISUAL ROUTINES                                                         PAGE D-5

0001        SUBROUTINE PENUP

c

c     this routine puts the pen up

c

0002        common /zgraph/ lunplt, iascr, ibscr, icscr, idscr,

1 xm, ym, dx, dy, dxb, dyb, ixcur, iycur, xmid, ymid,

1 xslope, yslope, xconst, yconst, scrx, scry, ichar(5),

1 ixorig, iyorig

c

c    <ESC>/1d	turns off dots

c

0003        call writch(27)

0004        call writch(47)

0005        call writch(49)

0006        call writch(100)

c

c     FS ()		turns on point plot mode

c

0007        call writch(28)

c

0008        return

0009        end

VISUAL ROUTINES                                                         PAGE D-6

0001        SUBROUTINE WRTSTR(str, len, irot, isize)

c

c     subroutine "write string"

c     len is the length of the string

c     irot determines the rotation of the string (1 - 4), 1 is

c     right side up, 2 is rotated 90 degrees clockwise, etc.

c     string is checked to see if in is in bounds

c

0002        common /zgraph/ lunplt, iascr, ibscr, icscr, idscr,

1 xm, ym, dx, dy, dxb, dyb, ixcur, iycur, xmid, ymid,

1 xslope, yslope, xconst, yconst, scrx, scry, ichar(5),

1 ixorig, iyorig

c

0003        byte inum(5)

0004        data inum / ':', '9', '1', '2', '3' /

0005        byte str(len)

c

c     check to see if in bounds

c

0006        isz = max0(isize, 1)

0007        isz = min0(isz, 5 )

0008        ix = 2 ** (isz - 1)

0009        iy = 14 * ix

0010        ix = 12 * ix * len

0011        irt = mod(irot - 1, 4) + 1

0012        irrt = mod( 12 - 2 * irt , 8 )

0013        isx = iabs(irt - 3) - 1

0014        isy = iabs(irt - 2) - 1

0015        ixl = isx * ix - isy * iy + ixcur

0016        iyl = isx * iy + isy * ix + iycur

0017        if( abs(ixl - xmid) .gt. dxb )goto 100

0019        if( abs(iyl - ymid) .gt. dyb )goto 100

0020  c

c     plot string

c    <ESC>/0d	turns on dots

c

0021        call writch(27)

0022        call writch(47)

0023        call writch(48)

0024        call writch(100)

c

c     US ()		turns on alphagraphics mode

c

0025        call writch(31)

c

c     Set character size

c

0026        call writch(27)

0027        call writch(inum(isz))

c

c     set character rotation

c

0028        call writch(27)

VISUAL ROUTINES                                                         PAGE D-7

0029        call writch('/')

0030        call writch(irrt + 48)

0031        call writch('e')

c

c     write string

c

0032        do 999 i = 1, len

0033        call writch(str(i))

0034  999   continue

c

0035        return

c

c     string is out of bounds

c

0036  100   call pltoff

0037        type 1, (str(i), i = 1, len)

0038        type 2

0039        call plton

c

0040        return

0041  1     format('$WARNING  string ', 100a1)

0042  2     format(' is out of bounds and will not be plotted')

0043        end

VISUAL ROUTINES                                                         PAGE D-8

0001        SUBROUTINE MARKER(mrknum, isize)

c

c     places a marker at the current coordinates

c

0002        common /zgraph/ lunplt, iascr, ibscr, icscr, idscr,

1 xm, ym, dx, dy, dxb, dyb, ixcur, iycur, xmid, ymid,

1 xslope, yslope, xconst, yconst, scrx, scry, ichar(5),

1 ixorig, iyorig

c

0003        return

0004        end

VISUAL ROUTINES                                                         PAGE D-9

0001        SUBROUTINE COLTYP (icol)

c

c     this subroutine sets the line color

c

0002        common /zgraph/ lunplt, iascr, ibscr, icscr, idscr,

1 xm, ym, dx, dy, dxb, dyb, ixcur, iycur, xmid, ymid,

1 xslope, yslope, xconst, yconst, scrx, scry, ichar(5),

1 ixorig, iyorig

c

0003        return

0004        end

VISUAL ROUTINES                                                        PAGE D-10

0001        SUBROUTINE PLOT(ixo, iyo, i)

c

c     this routine moves to the screen coordinates ix, iy.

c     if i = 0, the pen is put up before moving, if i = 1, the

c     pen is put down before moving. checks to see if within

c     surface boundary.

c

0002        byte lowy, highy, lowx, highx, yl, yh, xl, xh

0003        data yl, yh, xl, xh / 96, 32, 64, 32 /

0004        common /zgraph/ lunplt, iascr, ibscr, icscr, idscr,

1 xm, ym, dx, dy, dxb, dyb, ixcur, iycur, xmid, ymid,

1 xslope, yslope, xconst, yconst, scrx, scry, ichar(5),

1 ixorig, iyorig

c

0005        ix = ixo

0006        iy = iyo

c

c     check to see if in bounds

c

0007        delx = ix - xmid

0008        if(abs(delx) .le. dxb)goto 100

0010        xsc = xmid + sign(dxb, delx)

0011        xsl = 1.

0012        if ( ix .ne. ixcur ) xsl = ( xsc - ixcur ) / float(ix - ixcur)

0014        xsl = (iy - iycur) * xsl

0015        ix = xsc

0016        iy = xsl + iycur

c

0017  100   dely = iy - ymid

0018        if(abs(dely) .le. dyb)goto 200

0020        ysc = ymid + sign(dyb, dely)

0021        ysl = 1.

0022        if ( iy .ne. iycur ) ysl = ( ysc - iycur ) / float(iy - iycur)

0024        ysl = (ix - ixcur) * ysl

0025        ix = ysl + ixcur

0026        iy = ysc

c

c     plot point

c

0027  200   if(i .eq. 0)call penup

0029        if(i .ne. 0)call pendwn

0030  c

0031        lowy = mod( iycur, 32 ) .or. yl

0032        highy = ( iycur / 32 ) .or. yh

0033        lowx = mod( ixcur, 32 ) .or. xl

0034        highx = ( ixcur / 32 ) .or. xh

c

c     output previous position

c

0035        call writch(highy)

0036        call writch(lowy)

0037        call writch(highx)

0038        call writch(lowx)

VISUAL ROUTINES                                                        PAGE D-11

c

0039        lowy = mod( iy, 32 ) .or. yl

0040        highy = ( iy / 32 ) .or. yh

0041        lowx = mod( ix, 32 ) .or. xl

0042        highx = ( ix / 32 ) .or. xh

c

c     output current position

c

0043        call writch(highy)

0044        call writch(lowy)

0045        call writch(highx)

0046        call writch(lowx)

c

c     save current coordinates

c

0047        ixcur = ix

0048        iycur = iy

c

0049        return

0050        end

VISUAL ROUTINES                                                        PAGE D-12

0001        SUBROUTINE PLOTIN(x, y, i)

c

c     plots the point x, y where x and y are given in inches.

c     if i = 0, pen is put up before plotting, else pen is put down.

c

0002        ix = ifix(75 * x)

0003        iy = ifix(75 * y)

0004        call plot(ix, iy, i)

0005        return

0006        end

VISUAL ROUTINES                                                        PAGE D-13

0001        SUBROUTINE ERASE

c

0002        common /zgraph/ lunplt, iascr, ibscr, icscr, idscr,

1 xm, ym, dx, dy, dxb, dyb, ixcur, iycur, xmid, ymid,

1 xslope, yslope, xconst, yconst, scrx, scry, ichar(5),

1 ixorig, iyorig

c

c    <ESC><FF>	erases graphics page homes cursor

c

0003        call writch(27)

0004        call writch(12)

c

0005        return

0006        end

VISUAL ROUTINES                                                        PAGE D-14

0001        SUBROUTINE PLTON

0002        call writch(31)

0003        return

0004        end

VISUAL ROUTINES                                                        PAGE D-15

0001        SUBROUTINE PLTOFF

0002        call writch(24)

0003        return

0004        end

VISUAL ROUTINES                                                        PAGE D-16

0001        SUBROUTINE WRITCH(ch)

c

0002        common /zgraph/ lunplt, iascr, ibscr, icscr, idscr,

1 xm, ym, dx, dy, dxb, dyb, ixcur, iycur, xmid, ymid,

1 xslope, yslope, xconst, yconst, scrx, scry, ichar(5),

1 ixorig, iyorig

c

0003        byte vtbufr(1024)

0004        integer irec, vtpos, vtstrt, vtbufl

0005        common / vtbuf/ vtbufr, irec, vtpos, vtstrt, vtbufl

c

0006        byte ch

0007        if ( lunplt .eq. 7 ) goto 100

0009        vtpos = vtpos + 1

0010        vtbufr(vtstrt + vtpos) = ch

0011        if ( vtpos .ge. vtbufl ) call dmpplt

0013        return

c

0014  100   call ittour(ch)

0015        return

0016        end

VISUAL ROUTINES                                                        PAGE D-17

0001        SUBROUTINE DMPPLT

c

0002        common /zgraph/ lunplt, iascr, ibscr, icscr, idscr,

1 xm, ym, dx, dy, dxb, dyb, ixcur, iycur, xmid, ymid,

1 xslope, yslope, xconst, yconst, scrx, scry, ichar(5),

1 ixorig, iyorig

c

0003        byte vtbufr(1024)

0004        integer irec, vtpos, vtstrt, vtbufl

0005        common / vtbuf/ vtbufr, irec, vtpos, vtstrt, vtbufl

c

0006        if ( lunplt .eq. 7 ) return

0008        write(lunplt'irec) ( vtbufr(i), i = vtstrt + 1, vtstrt + vtpos )

0009        vtstrt = mod( vtstrt + vtbufl, vtbufl + vtbufl )

0010        vtpos = 0

c

0011        return

0012        end

INDEX

additional routines  . . . . . 14

auxiliary routines . . . . . . 18

AXIS . . . . . . . . . . . . . 3, 4, 7, 8, 11, 15

AXIS, listing  . . . . . . . . A-5

COLTYP . . . . . . . . . . . . 18

COLTYP, listing  . . . . . . . B-9, C-10, D-9

coordinates, global  . . . . . 2

coordinates, screen  . . . . . 2

coordinates, world . . . . . . 2

CURPOS . . . . . . . . . . . . 15

CURPOS, listing  . . . . . . . A-24

DASH . . . . . . . . . . . . . 18

DASH, listing  . . . . . . . . A-16

DASHLN . . . . . . . . . . . . 7, 10, 11, 18

DASHLN, listing  . . . . . . . A-11

device independent routines  . A-1

displaying a disk plot file  . 17

DMPPLT . . . . . . . . . . . . 5, 6

DMPPLT, listing  . . . . . . . B-4, C-4, D-17

ENDPLT . . . . . . . . . . . . 3, 4, 5, 6, 11, 12

ENDPLT, listing  . . . . . . . B-3, C-3, D-3

ERASE  . . . . . . . . . . . . 14

ERASE, listing . . . . . . . . B-12, C-13, D-13

examples . . . . . . . . . . . 3, 11, 12

FRAME  . . . . . . . . . . . . 7, 10, 11

FRAME, listing . . . . . . . . A-1

getting started  . . . . . . . 3

GRINNELL . . . . . . . . . . . 1, 5, 8, 9, 10, 14, 16, 18

GRINNELL routines  . . . . . . C-1

GRLIB  . . . . . . . . . . . . 16

GRPLOT . . . . . . . . . . . . 16

GRX  . . . . . . . . . . . . . 16

HIPLOT . . . . . . . . . . . . 1, 5, 6, 8, 9, 10, 14, 16, 17, 18

HIPLOT routines  . . . . . . . B-1

HIX  . . . . . . . . . . . . . 16

INIPLT . . . . . . . . . . . . 3, 4, 5, 6, 7, 10, 11, 12, 14

INDEX                                               PAGE I-2

INIPLT, listing  . . . . . . . B-1, C-1, D-1

initializing plots . . . . . . 5

INSECT . . . . . . . . . . . . 18

INSECT, listing  . . . . . . . A-30

ISCRX  . . . . . . . . . . . . 14

ISCRX, listing . . . . . . . . A-20

ISCRY  . . . . . . . . . . . . 14

ISCRY, listing . . . . . . . . A-21

LINE . . . . . . . . . . . . . 3, 4, 7, 9

LINE, listing  . . . . . . . . A-10

MARKER . . . . . . . . . . . . 18

MARKER, listing  . . . . . . . B-8, C-9, D-8

MOV1ST . . . . . . . . . . . . 18

MOV1ST, listing  . . . . . . . A-15

MOVETO . . . . . . . . . . . . 14

MOVETO, listing  . . . . . . . A-12

nominal inch . . . . . . . . . 2

PENDWN . . . . . . . . . . . . 18

PENDWN, listing  . . . . . . . B-5, D-4

PENUP  . . . . . . . . . . . . 18

PENUP, listing . . . . . . . . B-6, D-5

PLOT . . . . . . . . . . . . . 18

PLOT, listing  . . . . . . . . B-10, C-11, D-10

PLOT3D . . . . . . . . . . . . 12

PLOT3D, listing  . . . . . . . A-25

PLOTIN . . . . . . . . . . . . 19

PLOTIN, listing  . . . . . . . B-11, C-12, D-12

plotting frame . . . . . . . . 2

plotting page  . . . . . . . . 2

plotting window  . . . . . . . 2

PLTOFF . . . . . . . . . . . . 14

PLTOFF, listing  . . . . . . . B-14, C-15, D-15

PLTON  . . . . . . . . . . . . 14

PLTON, listing . . . . . . . . B-13, C-14, D-14

PLTSTR . . . . . . . . . . . . 15

PLTSTR, listing  . . . . . . . A-18

running a program  . . . . . . 16

terminating plots  . . . . . . 5

terminology  . . . . . . . . . 2

three dimensional plots  . . . 12

TRIML  . . . . . . . . . . . . 19

TRIML, listing . . . . . . . . A-19

two dimensional plots  . . . . 7

VIPLOT . . . . . . . . . . . . 16

VISUAL . . . . . . . . . . . . 1, 5, 6, 8, 9, 10, 14, 16, 17, 18

VISUAL routines  . . . . . . . D-1

INDEX                                               PAGE I-3

VIX  . . . . . . . . . . . . . 16

VUPORT . . . . . . . . . . . . 3, 4, 7, 8, 11, 14

VUPORT, listing  . . . . . . . A-4

WINDOW . . . . . . . . . . . . 3, 4, 7, 8, 10, 11, 14

WINDOW, listing  . . . . . . . A-3

WRITCH . . . . . . . . . . . . 19

WRITCH, listing  . . . . . . . D-16

WRTSTR . . . . . . . . . . . . 19

WRTSTR, listing  . . . . . . . B-7, C-5, D-6

XWORLD . . . . . . . . . . . . 14

XWORLD, listing  . . . . . . . A-22

YWORLD . . . . . . . . . . . . 15

YWORLD, listing  . . . . . . . A-23