VT125 HGRAPH

General Information

-------------------

In the VT125 version of HGRAPH the VT125 screen is  defined  to

have  the  same number of pixels per nominal inch as the Visual 550.

A consequence of this is that the size of the VT125 plotting page is

reduced  to  10.2  by  6.3 nominal inches.  These are the dimensions

that should be used when calling INIPLT if it is desired to  utilize

the entire screen.

The code that drives the VT125 is a slightly modified  copy  of

the 550 code.  Thus the logical unit number of the VT125 is the same

as the logical unit number of  the  550  (logical  unit  number  7).

Additional modifications not documented here will have to be made to

create an HGRAPH library that supports both the VT125 and the 550.

Since HGRAPH was designed to work with a  variety  of  physical

plotting  devices,  its  subprogram  modules are divided into device

dependent modules and device independent modules.  It should only be

necessary  to modify the device dependent routines to provide HGRAPH

support for the VT125.

Additional modifications may be necessary  when using an  older

model  DM/PL  single  pen   HIPLOT  plotter.    All  the  executable

statements in subroutine COLOR should be removed,  and all P, E, and

F  commands contained in plotter messages in the  INIPLT and  ENDPLT

routines should be removed.  The command 'UH @' is recommended as the

command to be sent to the plotter in subroutine ENDPLT.

VT125 HGRAPH                        PAGE 2

The Visual 550  and  VT125  also  appear  to  have  differences

regarding  the  ability  to  independently display text and graphics

planes.  The VT125 seems to always display both text  and  graphics.

If  this is undesirable, it is possible to control this to a certain

extent from inside the program using the modified  HGRAPH  routines.

Subroutine  erase  will  clear the graphics plane.  Subroutine plton

will, in addition to entering graphics mode, clear the  text  plane.

If  it  is  desireable  to  clear the text screen and remain in text

mode, this can be accomplished by calling plton followed immediately

by  calling  pltoff.   The file CLEAR.DAT has been provided to clear

the graphics screen from outside the control  of  the  program.   To

clear the graphics screen simply enter TYPE CLEAR.DAT.

A couple of problems commonly occur when using HGRAPH.  When  a

program  is  linked  with the HGRAPH library it is common to see the

linker error message LIBRARY LIST OVERFLOW INCREASE  SIZE  WITH  /P.

The  run  time error message STACK OVERFLOW may also be encountered.

The solution to these problems is to use the following commands when

linking programs with HGRAPH:

r link

<.sav file name>/P:400./B:1400=<list of object file names>

<CNTRL/C>

If the errors persist, the values 400 and 1400 can be increased.

VT125 HGRAPH                        PAGE 3

Since the VT125 does not  recognize  graphics  commands  unless

they  occur  between the two commands to enter and exit REGIS (plton

and pltoff), and since these commands may be  used  to  control  the

text  plane,  if calls to HGRAPH routines result in a lot of garbage

characters being printed on the terminal it will be due to the  fact

that plotting was disabled and not reenabled before calling plotting

routines.

The routines that implement cursor control,  character  reading

routines,   and  interactive  legend  developing  routines  are  not

supported.  Do not attempt to use these routines.  They will not  be

included in the library built using this version of HGRAPH.

An indirect command file is  provided  to  compile  the  HGRAPH

source  and  build  the  library  HGRAPH.OBJ.   To create the HGRAPH

object library enter the command @HGRAPH.BLD.

VT125 HGRAPH                        PAGE 4

Device Dependent Routine Modifications

--------------------------------------

CIRCLE :  The subroutine circle is implemented by ENCODEing a  REGIS

command  to  move  to  the  location  specified  by the

parameters IX and IY.  The y coordinate is adjusted  to

account  for  the difference in screen origin locations

between the two terminals.  A circle is then  drawn  by

ENCODEing  the  circle  radius  into  the  regis circle

command  and  sending  this  message.   Since  the  550

version  of  HGRAPH  used  routines  implemented in the

terminal itself to draw circles, it  was  necessary  to

use the REGIS circle drawing command also.  A result of

this is that circles drawn using the VT125  version  of

HGRAPH  will  be  of  lower  quality than circles drawn

using  the  550.   In  addition,  since  the   terminal

implements  the  actual circle drawing, circles will be

plotted without regard to defined  pages,  frames,  and

windows.

VT125 HGRAPH                        PAGE 5

ERASE :  Graphics mode characters to clear  the  VT125  screen  have

been substituted for 550 control characters.  S(E)

INIPLT :  Iniplt was changed to call plton to enable graphics.  This

call   replaced   outputting  specific  550  characters

intended to cause  a  particular  type  of  entry  into

alphagraphics mode.

PENDWN :  Modified to only save up/down status of  pen.   The  REGIS

equivalent of a pen position is determined by a command

that accompanies the graphics coordinates.  This status

is looked at to determine the proper type of command to

accompany the coordinates when a screen position change

is made.

PENUP :  Modified to only save updown  status  of  pen.   See  notes

accompanying PENDWN.

VT125 HGRAPH                        PAGE 6

PLOT :  The modifications  to  suboutine  plot  represent  the  only

significant changes to the HGRAPH source where the code

was actually changed instead of  replaced  by  a  REGIS

equivalent.   The changes reflect the fact that the 550

interprets  graphics  coordinates   relative   to   the

particular  state  that  it  is currently in (ie vector

mode, point-plot mode, etc.).  Regis commands,  on  the

other  hand, contain a command character along with the

graphics coordinates that indicate what action is to be

taken  (ie draw a vector, plot a point, etc.).  Another

difference is that the origin  of  the  REGIS  plotting

page  is  at  the  upper left-hand corner while the 550

origin  is  in  the  lower  left-hand   corner.    Plot

calculates  the proper point in the same manner for the

VT125 as for the 550.  When the screen coordinates have

been calculated the y coordinate is subtracted from 479

to  account  for  the  difference  in   screen   origin

locations.  The coordinate values are then ENCODEd into

the proper type of REGIS message based on  the  up/down

status  of  the  pen  and  output to the terminal.  The

current screen  location  is  saved  as  a  550  screen

coordinate.   This routine should be the first place to

look for  correcting  bugs  in  the  VT125  version  of

HGRAPH.

VT125 HGRAPH                        PAGE 7

PLTOFF :  Pltoff was modified to output the REGIS  graphics  command

terminator  ESC\ to exit graphics mode.  When pltoff is

called the cursor is also  homed  and  lowered  to  the

bottom  line of the screen.  This is done in an attempt

to keep possible subsequent  program  generated  output

from immediately overwriting the plot.

PLTON :  Plton was modified to output the start of a  REGIS  command

string   (ESC   P   p).   This  causes  any  subsequent

characters output to be interpreted as  REGIS  commands

until  the  graphics  string terminator is encountered.

Each time plton is called the text screen is cleared.

WRTSTR :  The subroutine wrtstr  in  the  hgraph  flecs  source  was

replaced  with  the  contents  of  the file wrtst2.flx.

This subroutine generates its own characters instead of

using  terminal  generated  characters.  Including this

routine adds simplicity  to  the  modification  at  the

expense of a certain loss of character quality.

VT125 HGRAPH                        PAGE 8

Routines Not Requiring Modification.

------------------------------------

COLTYP

DEFWIN

DMPPLT

ENDPLT

MARKER

WRITCH

Unsupported Routines

--------------------

CONLGN

CURSON

DEFWIN

FNDCUR

GETSTR

PUTCUR

REDSTR