PN-154

HOST PROGRAM FOR SOURCE FILE TRANSFER BETWEEN VAX AND

OTHER COMPUTERS

R.J. Dosen and V. White

November 16, 1981

The HOST program allows communication  and  ASCII  file  transfer

between the VAX where the program HOST is running and other computers.

The program will allow you to connect to a remote computer at  300  or

1200  baud through the VAX and will transfer files.  This program does

not support any type of error detection or correction  during  a  file

transfer.   Long  distance  calls  may  be made directly.  You will be

charged for long distance phone bills since we  keep  a  log  of  HOST

usage.   Do NOT use UWATS.  Objectionable tones are generated on UWATS

lines which may disconnect you at any time.  Two users at a  time  may

run the HOST program since we have two dialout modems.

HOST program for File Transfer                                  Page 2

1.0  DEFINING HOST

To define the HOST command, you should put the line

$ HOST :== $SYS$SYSTEM:HOST.EXE

in your LOGIN.COM command procedure file.  The host  program  normally

selects any available dial-out modem (currently TTA3, TTA4).  You may,

however in case of trouble specify one e.g.

$ DEFINE  HOST_MODEM  TTA4

2.0  RUNNING HOST

To run the HOST program,  type  the  following  command  at  your

terminal.

HOST

When HOST is  run,  it  will  ask  you  several  self-explanatory

questions,  the  answers  to  which  are  explained  in  the following

section.  After answering them, your terminal will be connected to the

dial-out modem.  Type a control-E and carriage return to get the modem

intelligence's attention it will respond with

HELLO I'M READY

*

You may now type in single letter commands (terminated by return)

to  the  dial-out modem.  See HELP HOST MODEM for complete information

on commands.  You can get-by with the  following  command  D  (return)

selects  Keyboard  Dial Mode (what you type will NOT be displayed) you

will be prompted

NUMBER?

Type the number as it would be dialed (you may include dashes for

readability or 'K' for a 5 second delay) it will NOT echo as you type.

The number will then display for  your  approval.   You  must  type  a

return to tell it to actually dial the number.  it will then display

DIALING:

and dial the far computer.  When connection is  made,  the  modem

will print

ON LINE

and turn off its intelligence, allowing you to talk  directly  to

the far computer.  ( you may need to press return one or more times to

get the attention of the far computer).

HOST program for File Transfer                                  Page 3

You will are now connected to the remote system.

When you have established the connection to  your  remote  system

all  subsequent  terminal  input,  with  only  one  exception, is sent

directly to the remote system.  Login on the remote system and do what

you  like.   The  exception  is when a NUL (ASCII code 0) character is

typed (this is a control-space-bar on a VT-100 or  ADM-3  and  also  a

control-@  on  an  ADM-3  or  Z-19  terminal  ).   This  character  is

intercepted by HOST and allows  you  to  enter  commands  to  HOST  to

transfer files or perform various other functions.

Type HELP HOST COMMANDS for a list of the HOST commands.

Occasionally you may find that you have a connection to a  remote

computer  already  established.   (The  automatic  disconnect  of  the

previous HOST user  has  failed).   You  should  Exit  from  the  HOST

program, wait 30 secs and try again.

If you get the message

ERROR ON ALLOC MODEM

then someone else is using the HOST program.

When you have established the connection to  your  remote  system

all  subsequent  terminal  input,  with  only  one  exception, is sent

directly to the remote system.  Login on the remote system and do what

you  like.   The  exception  is when a NUL (ASCII code 0) character is

typed (this is a control-space-bar on a VT-52 or VT-100 or  ADM-3  and

also  a  control-@  on  an  ADM-3 terminal and on Z-19 CTRL-SHIFT-2 ).

This character is intercepted by HOST and allows you to enter commands

to HOST to transfer files or perform various other functions.

3.0  QUESTIONS AND ANSWERS

Some parameters need to be set up before the connection to the  remote

system  can  be  made.   HOST  must  know through which port the modem

connection is to be made, which type of  remote  system  you  wish  to

communicate  with,  at which baud rate, etc.  Some of these parameters

can be predefined, system wide, and some can be predefined locally  by

you.

For all questions, default answers are provided.   These  default

values  will  be  used whenever a RETURN key is pressed in answer to a

question.

Normally, the first question you will be  asked  by  HOST  is  to

enter  the Baud Rate.  The Baud rate may be selected to be either 1200

or 300.

You will be asked which type of remote system you wish to connect

to.   Three  types  of  remote system are supported.  By this is meant

that suitable values for the line DUPLEX and PARITY are provided,  and

HOST program for File Transfer                                  Page 4

when  file transfer to or from the remote system is requested, all the

commands necessary to set up suitable terminal characteristics on  the

remote system and make the file transfer, are issued automatically.

The remote system type should be entered as a  character  string.

The supported types of system are:

VAX

CYBER

PDP11

For any other system, or if the user wishes to issue all the file

transfer commands to the remote system himself, the system type

OTHER

must be used.

4.0  PRE-DEFINED PARAMETERS

The ports through which HOST will establish  a  connection  should  be

predefined  on  each system, using the System Logical Name HOST_MODEM.

If for some reason the system manager or you  have  not  set  up  this

logical  name  then HOST will ask you to enter the port name.  Current

assignments are:

TTA3 and TTA4   dial-out modems

In addition, the default values for  Baud  Rate,  Remote  System,

line duplex and parity may be set up using the logical names

HOST_BAUD_RATE

HOST_SYSTEM

HOST_PARITY

HOST_DUPLEX

If these names have been  defined  then  they  are  used  as  the

parameters  for  the  HOST program, without prompting the user for the

information

For example:  if you  nearly  always  use  the  HOST  program  to

communicate  with the CYBER at 1200 baud then you might like to put in

your login command file

DEFINE HOST_BAUD_RATE "1200"

DEFINE HOST_SYSTEM "CYBER"

This will avoid all prompts for parameters  and  go  directly  to

port  selector connection.  Of course, if you then wish to communicate

with another system you will have to use the HOST command  CHANGE,  to

HOST program for File Transfer                                  Page 5

change SYSTEM.

5.0  MODEM COMMANDS

This section remains to be completed.  Complain to LIBRARY.

ctrl-E return              get attention of modem intelligence

D return   dial    dial from keyboard

S return   store   store number

M return   memory  display stored numbers

1 return           dial first stored number

.                   .

.                   .

.                   .

9 return           dial ninth stored number

R return   redial  dial number repeatedly

P return   print   display menu of these commands (i.e.  HELP)

I return   idle    turns off modem intelligence

6.0  HOST COMMANDS

Host normally sends all terminal input  directly  to  the  remote

system.  However, if you input a NUL character (control-@ for ADM-3 or

control-space for VT-100 and VT-52 or CTRL-SHIFT-2 on  Z-19)  the  NUL

character  is not sent to the remote system.  Instead HOST prompts you

for a command.  The legal commands are:

1.  HELP - This command displays help on the modem commands.

2.  EXIT  -  This  command  causes  the  HOST  program  to  exit,

returning  you  to  VAX command level.  This command does not

log you off  the  remote  system.   The  connection  will  be

broken, it takes about 30 secs.  You should logoff the remote

system before EXITing HOST.

3.  VAX - This command causes the HOST program to exit, returning

you  to  VAX  command level.  In this case, the connection to

the remote system is not broken.  Use of this command is  not

HOST program for File Transfer                                  Page 6

recommended,  because  the modem line to the port selector is

still owned by you, thus preventing anyone  else  from  using

the HOST program.

4.  DELAY - The SEND command normally  sends  characters  to  the

remote  computer as fast as possible.  Occasionally it may be

necessary to send characters to  the  remote  computer  at  a

slower  rate.  The DELAY command allows you to set this delay

interval between characters.  You will be  prompted  for  the

delay time.  The delay applies to all SEND commands until you

EXIT HOST or  issue  a  new  DELAY  command.   For  supported

machines it is not necessary to set this parameter.

5.  CHANGE  -  This  command  allows  you  to  change  individual

characteristics  of  the  line  without exiting and rerunning

HOST.   If  you  change  SYSTEM  then  only  HOST's  internal

information  about  which  remote system you are connected to

will change.   To  disconnect  from  one  remote  system  and

re-connect  to  another  you  must  EXIT  from  HOST to force

disconnect.

6.  SEND and RECEIVE - These commands allow file transfer to  and

from the remote system.  See separate section below

7.0  FILE TRANSFER TO AND FROM REMOTE SYSTEMS.

ASCII files may be SENT from the VAX to a remote system,  or  RECEIVED

by  the  VAX  from a remote system.  This file transfer is achieved by

typing the file out to the terminal on the sending  system  and  using

this  as terminal input on the receiving system, which must be running

a program to create a file from terminal input.  For all the supported

systems  both  ends  of  this procedure are handled automatically, for

both SENDS and RECEIVES.

If  the  source  file  to  be  transferred   contains   lowercase

characters  then you must be sure that the terminal characteristics on

both the host VAX and the remote system are correctly set.

The following should be set

SET TERM/LOWER           on the VAX (normally already set)

SET /LOWER=TI:           on the PDP

ASCII                    on the CYBER

If a source file is to be SENT to the CYBER, with records  longer

than  72  characters, you must set up the page width on the CYBER to a

suitable value before starting the file transfer

escape PW=n              where n is the page width in

characters

HOST program for File Transfer                                  Page 7

7.1  RECEIVE

This command allows a file transfer to the VAX from the remote system.

You must be logged in on the remote system.

For all supported systems you will be prompted for a  destination             _________ _______

file  name  on  the  local  VAX and the source file name on the remote

machine.  The file will be typed out at the terminal as  it  is  being

transferred.  Typing any character on the terminal during the transfer

will terminate the transfer prematurely.  When the file has been typed

out completely you should then press the RETURN key, or any other key,

and this will terminate the transfer and close the received file.

To receive a file from some other unsupported system you will  be                                       ___________ ______

prompted  for  the  destination  file name on the local VAX.  You must

then issue commands to the remote system to type the source  file  out

on  the  terminal.   The  file will be transferred to the VAX as it is

typed out on the terminal.  To  terminate  the  transfer  prematurely,

type  whatever  you normally type on the remote system to stop typeout

and return to command level.

7.2  SEND

This command will transfer a file from the VAX  to  a  remote  system.

You must be logged in on the remote system.

For supported systems you will be prompted for  the  source  file         _________ _______

name  on  the  local  VAX  and the destination file name on the remote

system.  Optionally, the file may be typed out on the terminal  as  it

is  being transferred.  The file transfer will terminate automatically

at end of file and the file on the remote system will be closed.

For unsupported systems you will be prompted for the source  file         ___________ _______

name  on  the local VAX.  However you must then issue a command to the

remote system to copy terminal input to a file.  This must be a single

command which terminates with the first carriage return, at which time

HOST will begin sending the file.  You must then issue  a  command  to

the  remote system to close the file (if necessary).  The HOST program

sends each record of a file, followed by a carriage return.  The  next

record  is sent when the remote system has responded with a line feed.

If this synchronization is not appropriate for the remote  system  and

data  is  sent too quickly, use of the DELAY command might help.  (See

section on HOST COMMANDS)

To terminate the file transfer  prematurely,  type  whatever  you

normally  type  on  the remote system to stop accepting terminal input

and return to command level.

HOST program for File Transfer                                  Page 8

8.0  INSTALLATION OF THE HOST PROGRAM

8.1  Building And Installing

There is a command file which will build the  HOST  program  and  give

installation information.

HOST should  be  installed  in  the  system  with  the  following

privileges

CMKRNL,CMEXEC,PHY_IO

8.2  Modem Line Characteristics

The port to be used for the modem  connection  to  the  port  selector

should   be   set   up   with   the   following   permanent   terminal

characteristics:

/NOBROADCAST/NOECHO/NOINTERACTIVE/NOTYPE_AHEAD

This helps eliminate dialogues between the far computer and the  LOGIN

process on the VAX.

8.3  Logical Names

The port to be used for the modem connection should be  defined  as  a

four character equivalence name string for the logical name

HOST_MODEM

in the system logical name table

e.g. TTA3  (note no : on end)

but DEFINE  HOST_MODEM  " "    will allow the program to choose.

9.0  WARNINGS

9.1  Integrity

As mentioned above, there is no  error  checking  performed  on  files

transferred between systems.  If you transfer a file you might wish to

sometimes transfer it twice  and  compare  the  two  resultant  files.

Utilities for file comparison exist on PDP, VAX and CYBER

DIFFER on VAX   (for help type HELP DIFFER)

CMP    on PDP   (for help type HELP COMPARE)

Since the program is used to  transfer  data  using  a  modem  and  an

external  telephone  line  to  a remote system great caution should be

exercised, and probably the program is  totally  inadequate  since  it

HOST program for File Transfer                                  Page 9

does NO error checking.

9.2  Tabs

Tabs on the VAX are transmitted to the remote system using  the  ASCII

HT  character.   On  the  FERMILAB  CYBER  there  are  two  procedures

available in PROCLIB, to process the tab characters.  These procedures

are VAXTABS, which assumes tabs set every 8 columns starting at column

8;  and FTNTABS, which assumes tabs set every 3  columns  starting  at

column 7.

9.3  Speed

The speed of transmission is rather slow.  Firstly 1200 baud is  quite

slow  for  file  transfer of large files and secondly the HOST program

sends or receives each character to/from  the  remote  system  with  a

separate I/O request

9.4  Non Source Files

The  file  transfer  is  for  ASCII  source  files  only.    Non-ASCII

characters  may  be interpreted as control information or errors.  NUL

characters may not be transferred.

9.5  Carriage Returns And Blank Lines

If for  any  reason,  your  file  contains  embedded  Carriage  return

characters,  as  the first character of a record, they will be removed

in the process of receiving the file from a remote computer  onto  the

VAX.   From  some  remote  systems received files may contain an extra

blank line at the end.

10.0  ACKNOWLEDGEMENTS

The  HOST  program  was  orignally  written   by   W.Tanenbaum,Harvard

University  High  Energy  Physics Laboratory.  It has been modified at

FNAL to provide  better  support  for  the  FNAL  computers.   It  was

re-modified by J.James Belonis II for the UWPhysics VAX.