LASER is a user-written print symbiont that is tailored specifically to the

Apple LaserWriter.

Submitted by:

Rick Watson

Computation Center

University Of Texas At Austin

Austin, TX  78712

This version of the symbiont is in use at several sites, but should

still be considered to be in "BETA" testing.  It has only been tested

with version 23 of the LaserWriter, but should work with later versions.

The symbiont as distributed is configured to print normal text files in

a variety of formats on the laser printer.  Support is also included for

printing PostScript files, MacPaint files, and Zeta plot files.  See

LASER.C for additional capabilities of the symbiont.

This distribution consists of the following files:

File		Destination		Purpose

---------------------------------------------------------------------------

LASER.EXE	SYS$SYSTEM:LASER.EXE	The symbiont executable.

LASER.TLB	SYS$LIBRARY:LASER.TLB	Library of PostScript header files.

LASER.C					Symbiont source.

LASER_ERR.MSG				Error message source.

LASER.OPT				Link options file.

QUIDEF.H				Source header files.

SMBDEF.H

LBRDEF.H

LASER.HLP	SYS$HELP:HELPLIB.HLB	Description of forms supported.

SYST.COM	SYS$MANAGER:SYSTARTUP.COM Queue startup commands.

Installation.

Installation of the files can be accomplished with the following set of

commands:

$ COPY LASER.EXE SYS$SYSTEM:/PROT=W:RE

$ COPY LASER.TLB SYS$LIBRARY:/PROT=W:RE

$ LIBRARY /HELP SYS$HELP:HELPLIB LASER		! Optional

Select a terminal port to attach the printer to, and edit SYST.COM to

reflect your change.  SYST.COM may be invoked by your system startup

procedures.  The printer may be connected with a cable that has only

transmit, receive and ground pins connected.  Transmit and receive

should (I think) be swapped. The switch on the LaserWriter should be

switched to the "9600" position.  If you are sharing your LaserWriter

with a Macintosh connected via AppleTalk, use the following procedure

when switching the printer:

To switch the printer to the VAX:

1. Insure that no AppleTalk jobs are in progress.

2. Switch switch on LaserWriter to "9600".

3. $START/QUEUE ALW

To switch the printer to the MAC:

1. $STOP/QUEUE/NEXT ALW

2. Allow any jobs in progress to complete.

3. Switch switch on LaserWriter to "AppleTalk".

$ CREATE SYS$SYSDEVICE:[LASER].  This area is used for LASER log files

for spooled files, or when the laser log cannot be created in the user's

directory.  The location and name of the directory may be changed by

changing the definition for LASER$LOG in SYST.COM.  Log files are only

created when there is an error in the job, or when the job sends back

information (i.e. using the PostScript PRINT command).

To build the symbiont:

$ CC LASER

$ MESSAGE LASER_ERR

$ LINK LASER.OPT/OPT

PostScript is a trademark of Adobe Inc.

LaserWriter and MacPaint are trademarks of Apple Computer.