ETHERNET MANAGEMENT II - DATA GATHERING AND MONITORING

OVERVIEW OF PROGRAMS

Frames to be monitored

======================

The programs described will listen for certain Ethernet frames, so you must

have a clear understanding of why the frames are on the Ethernet.

The MOP frames that we will listen for are the SYSTEM ID messages.  Every

Digital Node on the Ethernet sends a periodic SYSTEM ID message every 8 to

12 minutes.  Also, if a Digital node receives a REQUEST ID frame, it will

respond immediately with a SYSTEM ID frame.  The SYSTEM ID frame contains

information about the sending node; including the type of Ethernet

controller, the hardware address, and the type of system.  The LAT and LAVC

programs supplied will transmit REQUEST ID frames to get information about

a particular node; specifically the type of Ethernet controller.  The HWA

program listens for the periodic MOP SYSID messages.

The LAT frames that we will listen for are the service announcement frames

which are transmitted periodically by every node running LAT.  The default

is that they are sent once every minute.  These frames are transmitted to a

specific multicast address.

The LAVC frames that we will listen for are the hello frames which are

transmitted every three seconds by every node in an Ethernet based cluster.

These frames are transmitted to a multicast address.  Each cluster uses a

different multicast address.

ETHERNET MANAGEMENT II - DATA GATHERING AND MONITORING

OVERVIEW OF PROGRAMS

Program description

===================

The programs supplied are written in MACRO-32.  Each program will open and

start channels directly to the Ethernet driver and listen for the

appropriate frames.

Since the programs start the Ethernet channels in exclusive mode, the

programs can only be run on systems that are not using the protocol being

monitored.  For example, the LAVC monitoring program can only be run on a

system that is not a member of an Ethernet based cluster.  And the LAT

monitoring program can only be run on a system that is not running LAT.

Many products use the MOP protocol (e.g. TSM, NCP>CONNECT), and cannot be

running if you wish to use any of the programs.

The programs do not require privileges to be run.

ETHERNET MANAGEMENT II - DATA GATHERING AND MONITORING

LAT PROGRAM

Initial menu

============

$RUN MONLAT

MONLAT - Ethernet LAT monitoring program.  Version 1.1

Monitor requests available:

1) Monitor service announcements from all nodes

2) Monitor service announcements for a specific group code

3) Monitor service announcement from a specific note

any other input will stop the program

Which monitor request would you like to make:

ETHERNET MANAGEMENT II - DATA GATHERING AND MONITORING

LAT PROGRAM

LAT monitor request 1

=====================

Use LAT monitor request 1 to gather information about all nodes sending

service announcements.  Since each node typically sends their service

announcement only once per minute, it's suggested that you allow the

program to monitor for at least two minutes.

Only one service announcement per node will be saved by the program.

After the program has run for the amount of time requested, it will display

all the information it gathered.  The information will be displayed in two

sections.  The first section is all nodes that are using a DECnet address.

The second section is for all nodes that are not using a DECnet address.

In the sample session, there were three nodes found using a DECnet address

and one node found not using a DECnet address.

ETHERNET MANAGEMENT II - DATA GATHERING AND MONITORING

LAT PROGRAM

LAT monitor request 1 - sample session

======================================

Which monitor request would you like to make: 1

How many hours   would you like to monitor: 0

How many minutes would you like to monitor: 2

How many seconds would you like to monitor: 0

Address: AA-00-04-00-3B-08; Controller 23=DEBNA; DECnet node:  2.59

Group code bit mask:

(063-000) 10 00 00 00 00 00 00 21

(127-064) 00 00 00 00 00 00 00 40

(191-128) 00 00 00 00 00 00 00 08

(255-192) 80 00 03 99 E0 00 00 00

Address: AA-00-04-00-21-0A; Controller  5=DEQNA; DECnet node:  2.545

Group code bit mask:

(063-000) 00 00 00 00 00 00 00 01

(127-064) 00 00 00 00 00 00 10 80

(191-128) 00 00 00 00 00 00 00 00

(255-192) 00 00 00 00 00 00 00 00

Address: AA-00-04-00-34-4F; Controller 23=DEBNA; DECnet node: 19.820

Group code bit mask:

(063-000) 10 00 00 00 00 10 00 04

(127-064) 5E 00 00 00 00 00 00 40

(191-128) 00 00 00 00 00 00 00 38

(255-192) 00 00 00 00 00 00 00 00

Address: 08-00-2B-03-3B-2F; Controller  5 DEQNA; DECnet node:  2.59

Group code bit mask:

(063-000) 10 00 00 00 00 00 00 21

(127-064) 00 00 00 00 00 00 00 40

(191-128) 00 00 00 00 00 00 00 08

(255-192) 80 00 03 99 E0 00 00 00

ETHERNET MANAGEMENT II - DATA GATHERING AND MONITORING

LAT PROGRAM

LAT monitor request 2

=====================

Use LAT monitor request 2 to gather information about all nodes sending

service announcements for a specific group code.  Since each node typically

sends their service announcement only once per minute, it's suggested that

you allow the program to monitor for at least two minutes.

Only one service announcement per node will be saved by the program.

After the program has run for the amount of time requested, it will display

all the information it gathered.  The information will be displayed in two

sections.  The first section is all nodes that are using a DECnet address.

The second section is for all nodes that are not using a DECnet address.

In the sample session, there were nine nodes found using a DECnet address

and no nodes found not using a DECnet address.

ETHERNET MANAGEMENT II - DATA GATHERING AND MONITORING

LAT PROGRAM

LAT monitor request 2 - sample session

======================================

Which monitor request would you like to make: 2

How many hours   would you like to monitor: 0

How many minutes would you like to monitor: 2

How many seconds would you like to monitor: 0

Which group code would you like to monitor: 90

Group Code # 90

Address: AA-00-04-00-E5-09; Controller 39=DESVA; DECnet node:  2.485

Address: AA-00-04-00-31-4C; Controller 37=DELQA; DECnet node: 19.49

Address: AA-00-04-00-75-4C; Controller 23=DEBNA; DECnet node: 19.117

Address: AA-00-04-00-7A-4C; Controller 11=DELUA; DECnet node: 19.122

Address: AA-00-04-00-7F-4C; Controller 23=DEBNA; DECnet node: 19.127

Address: AA-00-04-00-8C-4C; Controller 23=DEBNA; DECnet node: 19.140

Address: AA-00-04-00-04-4D; Controller 23=DEBNA; DECnet node: 19.260

Address: AA-00-04-00-A9-4D; Controller 23=DEBNA; DECnet node: 19.425

Address: AA-00-04-00-AB-4D; Controller 37=DELQA; DECnet node: 19.427

ETHERNET MANAGEMENT II - DATA GATHERING AND MONITORING

LAT PROGRAM

LAT monitor request 3

=====================

Use LAT monitor request 3 to gather information about a specific node sending

service announcements.  Since each node typically

sends their service announcement only once per minute, it's suggested that

you allow the program to monitor for at least two minutes.

After the program has run for the amount of time requested, it will display

all the information it gathered.  Note that more information about the node

is displayed for this request than for the other requests.

ETHERNET MANAGEMENT II - DATA GATHERING AND MONITORING

LAT PROGRAM

LAT monitor request 3 - sample session

======================================

Which monitor request would you like to make: 3

How many hours   would you like to monitor: 0

How many minutes would you like to monitor: 2

How many seconds would you like to monitor: 0

Enter the address of the node (AA-00-04-00-75-4C): AA-00-04-17-0A

Address: AA-00-04-17-0A

DECnet Node:  19.535

Controller:   23 = DEBNA

Node Name:    DIMOND

Node desc:    KARMA - A cluster for all purposes

Service name: DIMOND

Service desc: DIMOND - VAX 8800, VMS V5.0

Service name: KARMA

Service desc: KARMA - A cluster for all purposes

Group Code Set:

90

96

232

ETHERNET MANAGEMENT II - DATA GATHERING AND MONITORING

LAVC PROGRAM

Initial menu

============

$RUN MONLAV

MONLAV - Ethernet LAVC monitoring program.  Version 2.0

Monitor requests available:

1) Monitor a single cluster node

2) Monitor all nodes in a specific cluster

3) Monitor all nodes in all cluster

any other input will stop the program

Which monitor request would you like to make:

ETHERNET MANAGEMENT II - DATA GATHERING AND MONITORING

LAVC PROGRAM

LAVC monitor request 1

======================

Use LAVC monitor request 1 to gather information about a specific node in a

cluster.  Since each node sends their hello message once every three

seconds, it's suggested that you allow the program to monitor for at least

seven seconds.

After the program has run for the amount of time requested, it will display

all the information it gathered.

ETHERNET MANAGEMENT II - DATA GATHERING AND MONITORING

LAVC PROGRAM

LAVC monitor request 1 - sample session

=======================================

Which monitor request would you like to make: 1

How many hours   would you like to monitor: 0

How many minutes would you like to monitor: 0

How many seconds would you like to monitor: 7

Enter the address of the node (AA-00-04-00-75-4C): AA-00-04-0A-08

Monitor output from a single cluster node

-----------------------------------------

Node name:                 SLUGGO

Node number:               2.10

Node address:              AA-00-04-0A-08

Cluster Group Number:      65240

Cluster Multicast Address: AB-00-04-01-D8-FF

Identifier:                A hackers paradise

Ethernet controller:       23 = DEBNA

Hardware Address:          08-00-2B-00-7F-9C

ETHERNET MANAGEMENT II - DATA GATHERING AND MONITORING

LAVC PROGRAM

Identifier input file

=====================

The MONLAV program will display an Identifier for each cluster group code

that is monitored, and that is defined in the file "MONLAV.DAT".  When

MONLAV begins to execute, it attempts to open the file MONLAV.DAT and read

the identifier strings from that file.  The user creates the MONLAV.DAT

file using a text editor.  For each cluster group code, the user can

specify a text string that will be displayed by MONLAV with any other

information output about that cluster/node.

Each line of the MONLAV.DAT file contains a cluster group code number, and

an Identifier String.  The cluster group code number is a decimal string of

up to 5 characters in the range 0 to 65535.  The Identifier String is a

text string containing any alphanumeric characters desired.  The Identifier

String cannot contain carriage returns, Line feeds, or other control

characters.  A series of spaces (at least one) is used to separate the

cluster group code number from the Identifier String.

Here is an example of the MONLAV.DAT file used in all the MONLAV examples

that appear in this handout.

65240 A hackers paradise

57    The last resort for old programmers

[EOB]

ETHERNET MANAGEMENT II - DATA GATHERING AND MONITORING

LAVC PROGRAM

LAVC monitor request 2

======================

Use LAVC monitor request 2 to gather information about all nodes in a

specific cluster.  Since each node typically sends their hello message once

every three seconds, it's suggested that you allow the program to monitor

for at least thirty seconds.

Only one hello message per node will be saved by the program.

After the program has run for the amount of time requested, it will display

all the information it gathered.

ETHERNET MANAGEMENT II - DATA GATHERING AND MONITORING

LAVC PROGRAM

LAVC monitor request 2 - sample session

=======================================

Which monitor request would you like to make: 2

How many hours   would you like to monitor: 0

How many minutes would you like to monitor: 0

How many seconds would you like to monitor: 30

Which Cluster Group Code: 65240

Monitor output from a single cluster node

-----------------------------------------

==========================================================================

1) Cluster Group Number: 65240 (AB-00-04-01-D8-FF)

Identification: A hackers paradise

Name      Node       Address             Device      Hardware Address

------    -------    -----------------   --------    -----------------

1) MFOX      2.13       AA-00-04-00-0D-08   39=DESVA    08-00-2B-00-79-C3

2) ROW       2.31       AA-00-04-00-1F-08   37=DELQA    08-00-2B-00-38-66

3) RJ        2.63       AA-00-04-00-3F-08   39=DESVA    08-00-2B-00-79-C5

4) HAWKI     2.83       AA-00-04-00-53-08   39=DELQA    08-00-2B-00-38-62

5) SLUGGO    2.10       AA-00-04-00-0A-08   23=DEBNA    08-00-2B-00-7F-9C

6) HELENA    2.117      AA-00-04-00-75-08   23=DEBNA    08-00-2B-00-0F-D3

7) GALAXY    2.129      AA-00-04-00-81-08   23=DEBNA    08-00-2B-00-0F-CF

8) VX2000    2.158      AA-00-04-00-9E-08   39=DESVA    08-00-2B-00-79-C3

9) SULVAX    2.248      AA-00-04-00-F8-08   39=DESVA    08-00-2B-00-79-BE

10) KATYA     2.344      AA-00-04-00-58-09   39=DESVA    08-00-2B-00-42-FC

11) DIMOND    2.535      AA-00-04-00-17-0A   01=DEUNA    08-00-2B-00-DE-D1

12) XTASY     2.845      AA-00-04-00-4D-0B   39=DESVA    08-00-2B-00-79-C2

13) ADRIC     2.944      AA-00-04-00-B0-0B   39=DESVA    08-00-2B-00-79-BA

ETHERNET MANAGEMENT II - DATA GATHERING AND MONITORING

LAVC PROGRAM

LAVC monitor request 3

======================

Use LAVC monitor request 3 to gather information about all nodes in all

clusters.  Since each node typically sends their hello message once every

three seconds, it's suggested that you allow the program to monitor for at

least forty seconds.  This is because when a new node is found, the program

sends a MOP REQUEST ID message to the new node and waits for the response.

When this is all done, the program resumes listening for cluster hello

messages.

Only one hello message per node will be saved by the program.

After the program has run for the amount of time requested, it will display

all the information it gathered.  The information will be displayed in

multiple sections.  Each section will represent a cluster.  The clusters

will be displayed in ascending Cluster Group Codes.  Within each cluster

display, nodes will be listed in ascending address.

In the sample session, there were two clusters found.

ETHERNET MANAGEMENT II - DATA GATHERING AND MONITORING

LAVC PROGRAM

LAVC monitor request 3 - sample session

=======================================

Which monitor request would you like to make: 3

How many hours   would you like to monitor: 0

How many minutes would you like to monitor: 0

How many seconds would you like to monitor: 40

Monitor output from all nodes in all clusters

---------------------------------------------

==========================================================================

1) Cluster Group Number: 57 (AB-00-04-01-39-01)

Identifier: The last resort for old programmers

Name      Node       Address             Device      Hardware Address

------    -------    -----------------   --------    -----------------

1) GOBGOB    19.187     AA-00-04-00-BB-4C   37=DELQA    08-00-2B-00-32-D4

2) SATIRE    62.101     AA-00-04-00-65-F8   39=DESVA    08-00-2B-00-79-D6

3) PLAY      62.268     AA-00-04-00-0C-F9   37=DELQA    08-00-2B-00-32-E3

4) SPORT     62.276     AA-00-04-00-14-F9   05=DEQNA    08-00-2B-00-8F-09

5) VAX333    63.267     AA-00-04-00-0B-FD   05=DEQNA    08-00-2B-00-8F-43

==========================================================================

2) Cluster Group Number: 3992 (AB-00-04-01-98-10)

Name      Node       Address             Device      Hardware Address

------    -------    -----------------   --------    -----------------

1) VAX1      19.77      AA-00-04-00-4D-4C   39=DESVA    08-00-2B-00-00-01

2) VAX2      19.78      AA-00-04-00-4E-4C   39=DESVA    08-00-2B-00-00-02

==========================================================================

Total of 2 clusters and 7 cluster nodes found

ETHERNET MANAGEMENT II - DATA GATHERING AND MONITORING

HWA  PROGRAM

Description

===========

The Hardware Address (HWA) monitoring program (MONHWA) can be used to

monitor all nodes on a given Ethernet.  All Phase IV nodes are required to

send out a MOP SYSTEM ID (SYSID) message periodically.  The information in

the SYSID messages can be useful in determining more about the nodes that

are part of the LAN.  You can also use this program as a security check, to

see if unauthorized nodes are on the LAN.

The MONHWA program listens to the Ethernet, and collects all the SYSID

messages it receives and stores one per node.  After the monitoring time

has elapsed, the information gathered is displayed.  The information

displayed includes the Hardware Address, the Ethernet controller device

type, and if available, the DECnet Address.

Since SYSID messages are sent out every 8-12 minutes, it is suggested that

you let MONHWA run for at least 30 minutes.

ETHERNET MANAGEMENT II - DATA GATHERING AND MONITORING

HWA  PROGRAM

Sample session

==============

$RUN MONHWA

MONHWA - Ethernet HWA monitoring program. Version 1.0

How many hours   would you like to monitor: 0

How many minutes would you like to monitor: 30

How many seconds would you like to monitor: 0

Monitor output from all periodic SYSID messages

-----------------------------------------------

Hardware Address      Device      DECnet Address       Node #

-----------------    --------    -----------------    --------

------------------!-----------!---------------------!---------

0100002B0008         39=DESVA    AA-00-04-00-4D-4C     19.77

098F002B0008         05-DEQNA

6238002B0008         37=DELQA    AA-00-04-00-53-08     2.83

B979002B0008         39=DESVA    AA-00-04-00-9E-08     2.158

BA79002B0008         39=DESVA    AA-00-04-00-B0-0B     2.944

C379002B0008         39=DESVA

C579002B0008         39=DESVA    AA-00-04-00-3F-08     2.63

C679002B0008         39=DESVA    AA-00-04-00-65-F8     62.101

CF0F002B0008         23=DEBNA    AA-00-04-00-81-08     2.129

D1DE002B0008         01=DEUNA    AA-00-04-00-17-0A     2.535

D432002B0008         37=DELQA