Check Files for Tampering

Michael N. LeVine

Code 3514

Naval Weapons Center

China Lake

Ca 93555

(619) 939-3970

avn 437-3970

LEVINE%FIDLER.DECNET@NWC.ARPA

In various discussions in the PAGESWAPPER, and in various trade

publications, one of the tools mentioned for the detection of possible break-in

is the un-documented VAX/VMS utility CHECKSUM. But in most discussions it is

also recognized that CHECKSUM can be easily fooled and that it can be patched

to give wrong answers. In looking at this problem, I thought that it would be

much more difficult for a hacker to do an undetected tamper of critical system

files if more than one type of check was made on the target files. As a first

cut, I wrote the following program with its driving command file. CHECK will,

for each file specified, calculate the following information about the file:

Checksum	8 bit

Checksum	16 bit

Checksum	32 bit

CRC		CRC-16

CRC		CCITT

CRC		AUTODIN-II

CRC		CRC-12

and then output it with the file name on a single line.

The program can be executed by either

$ RUN CHECK

or

$ CHECK := $'F$LOGICAL("SYS$DISK")''F$DIRECTORY()'CHECK

$ CHECK  file name

or

$ CHECK := $'F$LOGICAL("SYS$DISK")''F$DIRECTORY()'CHECK

$ CHECK

the program will repeatedly prompt for file names until an empty line or EOF is

encountered. The file names specified can contain wild cards, and all matching

files will be checked before the program prompts for a new file.

$ RUN CHECK

File name ? sys$library:*.exe;*

File name ? sys$system:link.exe;*

As an example of how a system check would be done, I have provided a

command file CHECK.COM that would do the required checks. The command file

assumes that a file containing the valid check information exists in the

invoking directory using the name CHECKSUMS.DAT, if this is not true such as

the case of the first time the program has been run on the system or there has

been an upgrade or a new layered product has been installed then invoke the

command file

$ @CHECK FIRST

otherwise to do a normal check just do

$ @CHECK

the command file does the following:

.

.

Completely rebuild the program from sources so any tampering has to be done to

the source where a quick visual check can be made.

.

.

$ mac check

$ link/notrace check

.

.

Then the checks are run on the critical files-redirecting the results to a

temporary file. The files checked includes itself.

.

.

$ define/usermode sys$output checksums.acc

$ run check

sys$system:checksum.exe;*

sys$system:diff.exe;*

.

.

sys$library:*.tlb;*

check.*

.

.

Then a comparison is done checking the results against the data file containing

the correct results. Any differences could mean tampering.

.

.

$ diff checksums.acc checksums.dat

.

.

One last check is made of the master data file letting the output go to the

terminal where the user can then check the master data file's integrity against

a hard copy value.

.

.

$ run check

checksums.dat

.

.

When completed, the command file cleans up including deleting the .EXE and

.OBJ.

In writing this program, one objective I had was to keep it simple so

that a user checking the source could easily spot any changes that had been

made or so that the users could make mods that they felt necessary.

No special privileges are needed to run CHECK, only read access to the

files to be checked. This permits the user to keep and run this program from

any account.

If you have any feedback on this program, I can be contacted at the

address, phone or E-Mail address show above (E-Mail preferred).