+   Ethernet.Doc                                                             +

Ethernet is a utility to display information  about the  packets

of data being transmitted on a local area  network.  PHY_IO privilege

is required.

Usage:

$ RUN ETHERNET

Description:

Places ethernet controller into 'promiscuous'  mode in  order to

examine  all packets transmitted on the  cable; checks each packet in

order to perform one or more of the three  functions  (Trace, Address

collection,   Protocol  collection).    The   user  must  supply  the

function(s)  desired, the number  of  packets to observe  (default is

1000), and optionally a filtering protocol.  If the function includes

'trace', then user must specify how many bytes of each packet to dump

(0 is allowed, 1500 is maximum).

Prompts and  status messages are  written to "FOR$TYPE:", actual

output to "FOR$PRINT:"  (both of which default to "SYS$OUTPUT:").

For  systems  with  more than one  ethernet  controller or whose

controller is not  on  a  Unibus  or  Qbus,  define the logical  name

NI$DEVICE  as the  desired  device.

$ ASSIGN XEB0: NI$DEVICE

or

$ ASSIGN ETA0: NI$DEVICE

Alternatively, define XEA0 or XQA0 to point to the actual device.

Functions:

1) TRACE -- show source  and destination addresses,  protocol type,

packet  size,  and optionally  the  packet  contents  (in

hexadecimal format).

2) ADDRESSES -- collect  all ethernet addresses  observed (for both

source  and destination)  and then list them with  packet

counts.   Addresses corresponding to known  objects (such

as DECnet nodes) are identified by name.

3) PROTOCOLS -- collect all packet protocol types observed and then

list them with packet  counts,  multicast packet  counts,

and  percentage  of  overall  packets; protocols of known

type are identified by name.

4) SUMMARY -- ADDRESSES plus PROTOCOLS.

Restrictions:

PHY_IO privilege  is required in order to  place  the controller

into  promiscuous  mode.  There  can be at  most one promiscuous mode

channel at a time, so only one user can run this program (or  similar

program such as  MONITOR  ETHERNET  [patch  required])  at any  given

moment.   Tracing and dumping  every  packet is likely  to  cause the

program to to  be unable to keep up with the data--excess packets are

dropped silently.   Note:  execution of this  or  similar program can

cause significant interrupt-stack load on the system.

+                                                                     [pr]   +