From:	IN%"POSTMASTER@UWAV1"  "General Delivery" 26-OCT-1989 12:27

To:	DEREK@RITA.ACS.WASHINGTON.EDU

Subj:	Addendum to the TAPE99 description

Received: from JNET-DAEMON by RITA.ACS.WASHINGTON.EDU; Thu, 26 Oct 89 12:27 PDT

Received: From UWAV1(POSTMAST) by RITA with Jnet id 5017 for DEREK@RITA; Thu,

26 Oct 89 12:27 PDT

Date: Thu, 26 Oct 89 12:24 PDT

From: General Delivery <POSTMASTER@UWAV1>

Subject: Addendum to the TAPE99 description

To: DEREK@RITA.ACS.WASHINGTON.EDU

Comments: This is gatewayed mail.  Warning:  Mail may not necessarily be

returnable through this path.

Original_From:  IN%"larryg@BLAKE.ACS.WASHINGTON.EDU"(Larry Gales)

Original_To:  derek@UWAV1.ACS.WASHINGTON.EDU

Received: from blake.acs.washington.edu by UWAV1.ACS.WASHINGTON.EDU; Thu, 26

Oct 89 12:23 PDT

Received: by blake.acs.washington.edu (5.61/UW-NDC Revision: 2.1 ) id AA14930;

Thu, 26 Oct 89 12:24:21 -0700

Date: Thu, 26 Oct 89 12:24:21 -0700

From: Larry Gales <larryg@BLAKE.ACS.WASHINGTON.EDU>

Subject: Addendum to the TAPE99 description

To: derek@UWAV1.ACS.WASHINGTON.EDU

Message-Id: <8910261924.AA14930@blake.acs.washington.edu>

TAPE99 files may contain either single or multiple plots.  If all

of the X-coordinates for a set of plots lie within the current

page size, which is set by the WIDTH parameter, then all of the

plots are printed on one page.  Plots whose X-coordinates stray

outside the current page limits are printed on subsequent pages.

Plot scaling is controlled by a combination of the HEIGHT and WIDTH

parameters, which determine page size, and the X/Y-coordinates

of the plots in TAPE99.  If the X/Y-coordinates of the largest plot

in TAPE99 fit within a single page, then all plots are unscaled and

the exact sizes and aspect ratios of all plots are preserved.

In addition, the OFFSET parameters are ignored.  If, however,

the coordinates of at least one plot in the file exceed the page

size, then all plots are scaled so that the largest plot exactly

fits the area specified by HEIGHT and WIDTH.  Note that scaling may

distort the aspect ratio (height/width ratio) of the plots.  The

OFFSET parameters are honored if scaling is in effect.

For example, the command:

PLOT,TAPE99/CODE=TAPE99,EQUIP=XEROX,HEIGHT=7.5,WIDTH=3.5,

PEN1=(,,0.1),PEN2=(,,0.8)

sends a TAPE99 file to the Xerox such that all lines made with "pen 1"

are very fine (0.1 mm), lines made with "pen 2" are very dark (about

0.8 mm), and lines made with "pen 3" are medium weight (0.35 mm) by

default.  If every individual plot fits within a 3.5 by 7.5 inch

window, then all plots are unscaled, otherwise, the largest plot is

scaled exactly to fit 3.5 by 7.5 inches,

and all other plots are proportionately

reduced.  Any neighboring plots whose X-coordinates fit on the

same page are physically plotted on the same page.