Converting a Kernel Mode
Driver to WDM

Walter Oney

Copyright © 1998 by Walter Oney
All rights reserved

Revision History

| - Date | - Remarks |
| --- | --- |
| Nov. 30, 1998 | Original revision |
| Dec. 1, 1998 | Added footnote re device naming |
| Dec. 3, 1998 | Added code for no-resource driver |

Converting a Kernel Mode Driver To WDM
by Walter Oney

This white paper describes a straightforward way to convert an existing NT Kernel Model Device Driver (KMD) to the Win32 Driver Model (WDM) that Windows 98 and Windows 2000 use. The basic difference between an existing KMD and a WDM driver has to do with how a driver detects and configures hardware. Prior to WDM, a driver was completely responsible for locating the hardware it owned, assigning I/O resources like IRQ lines, and configuring itself and the hardware. With the advent of WDM, the driver has only the last of these three responsibilities. The operating system now detects hardware and assigns I/O resources; a WDM driver can operate in a purely passive role.

### Applicability

The technique described here applies to a device with these characteristics:

- The device cannot be removed from the computer, disabled, or reconfigured while power is on. This doesn't mean the device can't have removable media, just that the device itself will always be present and using a fixed set of I/O resources.

- The device always operates at full power while the computer is on.

These assumptions greatly simplify writing a WDM driver. Fully supporting Plug and Play and Power Management requires, among many other things, that you implement your own queues of I/O requests with full cancel and cleanup logic, that you have a way of stalling I/O requests that arrive while the system is reassigning I/O resources, and that you provide for the possibility that your device might be removed (either by surprise or at the behest of the end user) at any time, even while you're deciding whether or not to accept a new IRP. A great deal of intricate code is required. If these complications apply to you, don't follow the approach outlined in this paper. Instead, enroll in a seminar and/or wait until 1999's crop of WDM driver books and learn how to do WDM for real.

### Logo Requirements

Microsoft says that "minimum" requirements for a Windows 2000 driver are that "[t]he device can be added and removed dynamically without rebooting the system" and that "[t]he driver [must] properly handle Suspend and Resume messages". See http://www.microsoft.com/hwdev/wdm/ontdrvr.htm. It would be presumptuous of me to evaluate a driver's compliance with Microsoft's detailed logo programs, but I have two comments about these requirements.

The requirements I quoted make an explicit exception for "system devices and [the] primary disk drive." There are many other devices besides these that can't be removed from a running system, however. By providing full PNP support in your driver, you could still (perhaps) allow someone to temporarily disable your device for the purpose of resolving some sort of resource conflict. Then again, maybe your device isn't capable of being disabled dynamically, in which case full PNP support wouldn't do anyone any good anyway.

Suspend and Resume refer to power management features that aren't available on all platforms. Lacking an ACPI BIOS, for example, none of my desktop systems supports a low-power state. On these systems, power management support in a WDM driver is actually pretty pointless.

You might wonder why you should even bother converting your driver to an "almost" WDM driver. One advantage will be that a single driver binary might then run both on Windows 2000 and Windows 98. (There are many incompatibilities between the two implementations of WDM, but the ones I'm aware of pertain to the very features you're going to avoid using if you follow this cookbook.) Additionally, Microsoft might decide that your legacy device with its new WDM driver meets its requirements after all and give you a logo despite less than perfect compliance. Finally, you need to learn the WDM technology somehow, and the approach I'll outline gives you a start at that.

### Tools Required

A WDM driver uses the same executable file format as an existing KMD. You therefore use the same development tools for building and debugging a WDM driver as you've always used.

A WDM driver uses a new set of header files and libraries. You will need either the Windows 2000 beta-2 Device Driver Kit or the Windows 98 Device Driver Kit to obtain these headers. The DDKs also include documentation of many of the new features of WDM. Since the NT version of the kit is now newer than the Windows 98 version (and will continue to evolve over time), you should prefer to use it.

### Overview

To convert an existing driver to WDM, you simply need to rewrite your DriverEntry and DriverUnload routines and write three new IRP dispatch functions (two of which are trivial). In a current driver, DriverEntry and DriverUnload have several jobs to perform, as indicated in Table 1-1. The table also indicates where (and whether) a WDM driver would perform these same tasks.

| - Task | - Where performed in WDM |
| --- | --- |
| Initialize Driver Object | DriverEntry (no change, but slightly different steps required) |
| Enumerate hardware | By the OS; driver has no role here anymore |
| Create device object(s) | AddDevice function |
| Assign I/O resources | By the OS; driver has no role here anymore |
| Translate I/O resources | By the OS; driver needn't do this anymore |
| Allocate additional kernel objects, connect interrupt, etc. | Handler for IRP_MJ_PNP (IRP_MN_START_DEVICE) IRP |
| Disconnect interrupt, release kernel objects | Handler for IRP_MJ_PNP (IRP_MN_STOP_DEVICE) IRP |
| Delete device objects | Handler for IPR_MJ_PNP (IRP_MN_REMOVE_DEVICE) IRP |
| Global cleanup | DriverUnload (no change, but much less work) |

Table 1-1

Reallocation of DriverEntry and DriverUnload Responsibilities

In summary:

- Your current DriverEntry does some global initialization (perhaps) and initializes various fields in the Driver Object, including most especially the MajorFunction table. A WDM DriverEntry does the same things, by and large, but there are three new types of I/O Request Packet for which you need to supply dispatch functions.

- Detection of hardware is now done by the operating system. Your existing DriverEntry may be calling the HAL to look at, say, configuration space entries on the PCI bus. You can simply delete that code from your driver.

- Your DriverEntry probably creates and initializes a device object for each instance of the hardware you find. Your driver will continue to do this, but it will happen in a new subroutine you'll write called AddDevice. The operating system calls AddDevice once for each hardware instance. Each invocation will create just one device object.

- For each device object, your current DriverEntry probably goes through more or less heroic efforts to allocate and report IRQ, port, memory, and DMA channel assignments. Allocating these I/O resources is now a job for the operating system. You can simply remove the code you now have that performs this task.

- When you reserve I/O resources in an existing KMD, you also use routines like HalTranslateBusAddress to translate bus-relative values to system-relative values. The operating system now does this step automatically too.

- Your driver probably does some additional work to prepare to manage the I/O resources. This additional work includes steps like mapping memory, creating controller and adapter objects, connecting an interrupt, etc. A WDM driver does these steps when it receives an IRP with the major function code IRP_MJ_PNP and a minor function code of IRP_MN_START_DEVICE.

- Your current DriverUnload routine probably deconfigures your devices and deletes the device objects. A full-blown WDM driver would also do these things in response to various Plug and Play requests, but we're assuming here that your device isn't capable of being removed or temporarily disabled. So you don't have to worry about those details.

- Your current DriverUnload routine also undoes any global initialization that DriverEntry performed. A WDM driver's DriverUnload routine would do pretty much the same thing. In the scheme I'm describing in this paper, though, the DriverUnload function will never be called because it won't be possible to unload the driver in the first place.

The three new types of IRP you need to handle are these ones:

- IRP_MJ_PNP describes Plug and Play events. You'll be handling a few of the minor functions for this type of IRP. You'll also be explicitly passing these requests down to lower levels of the driver hierarchy.

- IRP_MJ_POWER describes power management events. Since your device is assumed to always operate at full power, all you need to do with these IRPs is pass them down to lower levels of the driver hierarchy.

- IRP_MJ_SYSTEM_CONTROL encapsulates Windows Management Instrumentation (WMI) commands and requests. Since I'm assuming your goal is simply to port an existing driver without adding additional functionality, all you need to do with these IRPs is pass them down to lower levels of the driver hierarchy.

# Skeleton of a WDM Driver

In this section, I'll describe the basics of the code you need to write to transform your stodgy old KMD into a shiny new WDM driver. The code fragments I'm discussing here are taken from the skeleton driver that accompanies this paper. (The skeleton includes a Visual C++ 6.0 project that you will need to customize as described in the README.TXT file. There's also a standard SOURCES file that you can use to build the skeleton with the normal BUILD utility.)

## DriverEntry and DriverEntry

A WDM DriverEntry routine need do nothing more than fill in function pointers within the driver object. If there's any other global initialization that needs to be performed, DriverEntry would do that too. Here's a complete DriverEntry function for a simple device:

extern "C" NTSTATUS DriverEntry(IN PDRIVER_OBJECT DriverObject,
  IN PUNICODE_STRING RegistryPath)
  {
  DriverObject->DriverUnload = DriverUnload;
  DriverObject->DriverExtension->AddDevice = AddDevice;

  DriverObject->MajorFunction[IRP_MJ_POWER] = DispatchPower;
  DriverObject->MajorFunction[IRP_MJ_SYSTEM_CONTROL] = 
    DispatchWmi;
  DriverObject->MajorFunction[IRP_MJ_PNP] = DispatchPnp;

  return STATUS_SUCCESS;
  }

The AddDevice function is brand new with WDM. That's the function the system will call to alert you when there's a hardware device for you to manage. The three types of IRP that are mentioned here are also brand new.

What might you need to add to this? First of all, your driver probably handles more types of IRP than just PNP, POWER, and SYSTEM_CONTROL. At a minimum, you're probably handling READ, WRITE, CREATE, CLOSE, and CLEANUP requests too. So you would need to specify those dispatch functions in the major function table.

In addition, you might be using what I call the "standard model" for IRP queuing, wherein your dispatch routines call IoStartPacket and your DPC routine calls IoStartNextPacket. In that case, you would need to set the DeviceStartIo pointer in your device object here.

Notice: no calls to the HAL, no loop to detect hardware, no creation of device objects, and so on.

Since DriverEntry doesn't allocate any memory or do any other initialization that has a side effect that needs cleaning up, your DriverUnload function can be empty:

VOID DriverUnload(PDRIVER_OBJECT DriverObject)
  {
  }

But, since the driver I'm describing in this paper will never be unloaded, the DriverUnload function will never be called.

## AddDevice

For each instance of hardware belonging to you, the system calls the AddDevice function you registered in DriverEntry. The basic job of a WDM AddDevice function is to create a device object. Period. It doesn't allocate resources, connect interrupts, or anything. Here's an annotated AddDevice function:

NTSTATUS AddDevice(PDRIVER_OBJECT DriverObject,
  PDEVICE_OBJECT pdo)
    {
    NTSTATUS status;
    PDEVICE_OBJECT fdo;
   
1   UNICODE_STRING devname;
    WCHAR namebuf[32];
    static LONG devcount = -1;
    _snwprintf(namebuf, arraysize(namebuf),
      L"\\Device\\WDMSKEL_%d",
      InterlockedIncrement(&devcount));
    RtlInitUnicodeString(&devname, namebuf);

2   status = IoCreateDevice(DriverObject,
        sizeof(DEVICE_EXTENSION), &devname,
        FILE_DEVICE_UNKNOWN, 0, FALSE, &fdo);
    if (!NT_SUCCESS(status))
        return status;

    PDEVICE_EXTENSION pdx = 
      (PDEVICE_EXTENSION) fdo->DeviceExtension;
    pdx->DeviceObject = fdo;
    pdx->Pdo = pdo;

3   fdo->Flags |= DO_POWER_PAGABLE;
4   pdx->LowerDeviceObject =
      IoAttachDeviceToDeviceStack(fdo, pdo);
5   fdo->Flags &= ~DO_DEVICE_INITIALIZING;
    }

6   return STATUS_SUCCESS;
    }

- However you were giving your device object a name previously is okay in WDM. In this example, I'm calling the first device \Device\WDMSKEL_0, etc., which follows a common convention for naming device objects. By the way, it's really okay to call _snwprintf here -- Microsoft told me so.

- We create one device object per call to AddDevice. The result of a successful call ends up in the local variable called fdo. That stands for Functional Device Object -- the WDM name for the device object managed by the main, or "function", device driver for a piece of hardware.

- Think of POWER_PAGABLE as being a magic incantation for now. Technically speaking, it means your dispatch routine for IRP_MJ_PNP is in paged memory and therefore runs at PASSIVE_LEVEL. The system makes additional decisions based on this flag, though, so you want to set it unless you know of a good reason not to. At the level of functionality we're addressing in this paper, you should leave this flag off if you're writing a disk driver. (If your device ends up holding a paging file, a hibernate file, or a crash-dump file, you need a non-paged power handler. I'm not going to tell you in this short paper how you'd ever find out that the system was going to use your device for one of these purposes.)

- This line of code is new with WDM. The pdo argument to the function points to what's called the Physical Device Object. Think of the PDO as being a placeholder to which the system can attach a bus driver that manages the actual connection between the system and your device. This call to IoAttachDeviceToDeviceStack adds one more link in a potentially long chain of device objects that are all associated with the single piece of hardware you're going to manage. The return value is the address of the device object immediately below yours. It might be the PDO, or it might be an object created by a lower filter driver. Whatever it actually represents, it's the device object to which you're going to forward those IRPs that you're obliged to pass down the driver stack. You need to save this pointer in some piece of nonpaged memory that you can easily find starting from the device object; your device extension is the obvious place.

- The INITIALIZING flag is something you have to clear. You might not have worried about this flag in your existing driver because the system automatically clears it in every device object you create during DriverEntry. Since we're creating our device object after DriverEntry returns, we're responsible.

- You return STATUS_SUCCESS from AddDevice to indicate successful creation of the device object.

What might you need to add to this? This would be the right time to initialize the DPC object that you get "for free" with a device object and any spin locks, queues, and so on, that you declare in your device extension structure. This would also be the right time to create a symbolic link (in the \DosDevices directory, say) to allow an application to open a handle to your device. Finally, you want to declare your preference for buffered or direct user-mode buffer handling by setting one or the other of DO_BUFFERED_IO or DO_DIRECT_IO in the device object flags. You can't change your mind later about the buffering method because of the way WDM filter drivers have to implement their own AddDevice functions.

What you definitely would not add to this AddDevice function is code to allocate I/O resources or otherwise configure your device. That comes later.

## Handling Plug and Play Requests

Handling IRP_MJ_PNP requests in all their wonderful complexity is gigantically complicated. So much so that Microsoft probably should have figured out another way for driver writers to participate in the configuration process. We'll just have to learn to cope, not least because a PNP IRP is how we get our configuration in WDM.

For a device like we're considering here that can't be removed or disabled dynamically, there's a simple way to fit into the architecture. First, you can have the following fairly simple dispatch function for IRP_MJ_PNP:

NTSTATUS DispatchPnp(IN PDEVICE_OBJECT fdo, IN PIRP Irp)
    {
    PIO_STACK_LOCATION stack = 
      IoGetCurrentIrpStackLocation(Irp);
    ULONG fcn = stack->MinorFunction;
    PDEVICE_EXTENSION pdx = 
        (PDEVICE_EXTENSION) fdo->DeviceExtension;

    switch (fcn)
        {

1   case IRP_MN_START_DEVICE:
        {
2       NTSTATUS status = ForwardAndWait(fdo, Irp);
        if (!NT_SUCCESS(status))
3           return CompleteRequest(Irp, status, 
              Irp->IoStatus.Information);

4       status = StartDevice(fdo, 
            &stack->Parameters.StartDevice.
              AllocatedResources
              ->List[0].PartialResourceList,
            &stack->Parameters.StartDevice.
              AllocatedResourcesTranslated
              ->List[0].PartialResourceList);

        return CompleteRequest(Irp, status, 0);
        }

5   case IRP_MN_QUERY_STOP_DEVICE:
    case IRP_MN_QUERY_REMOVE_DEVICE:
        return CompleteRequest(Irp, STATUS_UNSUCCESSFUL, 0);

    default:
6       IoSkipCurrentIrpStackLocation(Irp);
        return IoCallDriver(pdx->LowerDeviceObject, Irp);
        }
    }

- IRP_MN_START_DEVICE is the subfunction that tells us our configuration and instructs us to prepare our device for operation.

- ForwardAndWait is a helper function (see the DriverEntry.cpp file in the WDMSKEL sample) that sends an IRP to the next lower device object and waits for it to finish. We need to forward the START_DEVICE request in this way so we can do our other configuration work at PASSIVE_LEVEL.

- CompleteRequest is another helper function (also in DriverEntry.cpp) I use to perform the mechanics of calling IoCompleteRequest. It returns the same status code as you supply in the second argument.

- StartDevice is yet another helper function that performs the configuration and startup steps for your device. I'll show you a skeleton a bit further on.

- These are the other two minor functions of IRP_MJ_PNP that this legacy driver needs to handle. The system sends QUERY_STOP to ask, "would you mind releasing your I/O resources momentarily so I can rebalance resource assignments?" It sends QUERY_REMOVE to ask, "would you mind being removed from the system?" We answer both queries with a firm "No!", as befits the assumptions I made earlier about this device. If you were to answer "Yes" to either of these queries, you'd then need an amazing amount of additional logic in your driver.

- This driver forwards all other Plug and Play requests to the next driver down. IoSkipCurrentIrpStackLocation is new with the WDM versions of the DDK. All it really does is retard the IRP stack pointer by one position in anticipation that IoCallDriver will immediately advance the pointer. The net effect is that the next driver gets the same stack location as we got. Using this macro saves doing a memory copy.

The StartDevice helper contains code that you'll be porting from your current DriverEntry routine. Right now, you have code that starts with knowledge about which I/O ports and memory addresses you're using, which DMA channels, which IRQ, and so on. It does things like calling MmMapIoSpace, HalGetAdapter, IoConnectInterrupt, and so on. The StartDevice function basically picks up at the same starting point. Structurally, it will look like this:

NTSTATUS StartDevice(PDEVICE_OBJECT fdo, 
  PCM_PARTIAL_RESOURCE_LIST raw, 
  PCM_PARTIAL_RESOURCE_LIST translated)
    {

    NTSTATUS status;
    PDEVICE_EXTENSION pdx = 
        (PDEVICE_EXTENSION) fdo->DeviceExtension;
    
    PCM_PARTIAL_RESOURCE_DESCRIPTOR resource =
        translated->PartialDescriptors;
    ULONG nres = translated->Count;

    <local variable declarations>

    for (ULONG i = 0; i < nres; ++i, ++resource)
        {
        switch (resource->Type)
            {

        case CmResourceTypePort:
            <save info about port resource>
            break;
    
        case CmResourceTypeInterrupt:
            <save info about interrupt resource>
            break;
        
        case CmResourceTypeMemory:
           <save info about memory resource>
            break;

        case CmResourceTypeDma:
            <save info about DMA resource>
            break;

        default:
            break;
            }
        }

    <code to begin using assigned resources>

    return STATUS_SUCCESS;
    }

This peculiar looking routine requires explanation. When you get the IRP_MN_START_DEVICE request, the Parameters union in the I/O stack points (pretty obliquely, but it still "points") to a list of the I/O resources you've been assigned. I showed you code that sends two lists to StartDevice in the form of the raw and translated arguments. The "raw" resources are the bus-relative values you're probably used to figuring out on your own. The "translated" resources are the system-relative values you're probably used to deriving via a translation process.

Reduced, to essentials, what you're getting is two parallel arrays of CM_PARTIAL_RESOURCE_DESCRIPTOR structures. You can't know a priori what order the system is going to decide to hand them to you, however. If you're expecting an IRQ and a port address, you might find the IRQ description as the [0] element of the array and the port description as the [1] element. Or they might be in the other order. That's why I recommend the approach shown just above. First, you declare a bunch of local variables that will end up holding all the relevant things about your I/O resources. Inside the loop, you copy values from the resource descriptor structures into those local variables. After the loop, you then go ahead with your calls to MmMapIoSpace, IoConnectInterrupt, etc. Except for one detail, those calls will be the same as you're currently making.

By the way, if you were to build and install a device with no I/O resources (pretty unusual) and use just the code in the preceding fragment, you'd crash because of invalid "raw" and "translated" pointers. The WDMSKEL sample contains a NULL pointer check that prevents the crash for those readers who want to just build and install the (useless) skeleton to get a feel for things.

At this point in their learning, most people who've written old-style KMDs breath a big sigh of relief because they didn't have to find their own device and allocate their own resources.

## Handling Power and WMI Requests

As I mentioned earlier, you also need dispatch routines for two other types of IRP: IRP_MJ_POWER and IRP_MJ_SYSTEM_CONTROL. For our purposes, these routines can be very simple:

NTSTATUS DispatchPower(PDEVICE_OBJECT fdo, PIRP Irp)
    {
    PoStartNextPowerIrp(Irp);
    IoSkipCurrentIrpStackLocation(Irp);
    PDEVICE_EXTENSION pdx = 
        (PDEVICE_EXTENSION) fdo->DeviceExtension;
    return PoCallDriver(pdx->LowerDeviceObject, Irp);
    }

NTSTATUS DispatchWmi(PDEVICE_OBJECT fdo, PIRP Irp)
    {
    IoSkipCurrentIrpStackLocation(Irp);
    PDEVICE_EXTENSION pdx = 
        (PDEVICE_EXTENSION) fdo->DeviceExtension;
    return IoCallDriver(pdx->LowerDeviceObject, Irp);
    }

The power requests need to be handled using the two service functions highlighted in bold face. Other than that, all you need to do with these IRPs is send them down the stack as the example shows.

# Installing a WDM Driver

Installing a WDM driver involves creating a dual-personality INF file. One personality is for Windows 98, the other is for Windows 2000. The NT DDK has a documentation file (SETUP.CHM) that describes the INF format in detail. Rather than try to recapitulate all of that, let me just show you a dirt-simple INF file that you can adapt to get you started:

[Version]
Signature=$CHICAGO$
Class=UNKNOWN
Provider="Walter Oney Software"

[Manufacturer]
"Walter Oney Software"=WCO

[DestinationDirs]
DefaultDestDir=10,System32\Drivers

[SourceDiskFiles]
WdmSkel.sys=1

[SourceDiskNames]
1="Install disk",disk1

[WCO]
"WDM Skeleton Driver"=WdmSkelInstall,*WCO1234

[WdmSkelInstall]
AddReg=WdmSkel.AddReg
CopyFiles=WdmSkel.CopyFiles
 
[WdmSkel.AddReg]
HKR,,DevLoader,,*ntkern
HKR,,NTMPDriver,,WdmSkel.sys

[WdmSkel.CopyFiles]
WdmSkel.sys,,,2

[WdmSkelInstall.NT]
CopyFiles=WdmSkel.CopyFiles
 
[WdmSkelInstall.NT.Services]
AddService=WdmSkel,2,WdmSkelService

[WdmSkelService]
ServiceType=1
StartType=3
ErrorControl=1
ServiceBinary=%10%\system32\drivers\WdmSkel.sys

If you launch the hardware wizard in either Windows 98 or Windows 2000 and use the "Have Disk" button to point to the directory containing your INF file, your driver will get installed.

I left one part out of this INF file that you'll need to read up on. If you have a legacy device that can't electronically announce its I/O resource requirements, you'll need to add a LogConfig section. The two install sections would have pointers to the LogConfig. That's how the system will know what your device's requirements are, and it's the key to the entire automatic configuration process that Plug and Play enables. By supplying a logical configuration, you allow a legacy device to fit well into the architecture of the operating system, so it's a good thing to do.

Brian Catlin has pointed out that, as the DDK documentation says, giving a device a name can allow security violations to occur. That is, unless you take steps to validate security when someone opens a handle using the name you supply, you may end up performing unauthorized operations. Using IoRegisterDeviceInterface avoids the security problem, but also leads to fair difficulty on the application side. See my two-part series of Microsoft Systems Journal articles for Nov/Dec 1997 for an example of how an application can open a handle for a registered interface.

Basically, I think that if you have a legacy device, and if you're writing the only applications that will ever use it (which is often true of instrumentation devices, for example), there's no particular reason to use the device interface scheme. In fact, you might as well give the device object a name in the \DosDevices branch of the name space (instead of \Device) so you don't have to create a separate symbolic link.

This is the way you'd call StartDevice if your device has I/O resources. If you examine the WDMSKEL version of this call, you'll note that I check the two parameter pointers for NULL beforehand. Without those checks, the skeleton driver would crash on NT.

The exception is for DMA. You should call IoGetDmaAdapter instead of HalGetAdapter.