JOYSTICK DRIVER FOR WINDOWS NT

Version 1.0

Written by Gary T. Desrosiers and Kenneth W. Clapp

Installation instructions

The driver is shipped with the following files

\cpl		Control panel applet source files

\dll		Win32 multimedia subsystem driver source files

\driver		Kernel mode driver source files

\doc		Documentation

\install\checked	Checked (debug) version of drivers

\install\free	Release version of drivers

Copy either the checked build files or the free build files (recommended)

from the \install directory to a diskette or subdirectory. The following are

the files needed for installation;

JOYSTICK.CPL - Control panel applet

JOYSTICK.DLL - User mode Win32 subsystem driver

JOYIO.SYS - Kernel mode joystick driver

OEMSETUP.INF - installation script

For Windows NT 4.0, open the control panel and select “Multimedia”.

Click on the “Devices” tab and press the “Add” button.

Select the “Unlisted or Updated Driver” and click OK.

Put the path of where you copied the four distribution files and click

OK. You will be asked to restart you’re system. After this is done,

you’re joystick will need to be calibrated. When the system restarts

go into the control panel and select “Joystick calibration”. Move the

joystick as far as possible in all directions until the green crosshairs

are basically centered. Click OK to write the new calibration info

to the registry. You are now ready to use the joystick.

For Windows NT 3.5.1, open the control panel and select “Drivers”.

Press the “Add” button. Select the “Unlisted or Updated Driver”

and click OK. Put the path of where you copied the four distribution

files and click OK. You will be asked to restart you’re system.

After this is done, you’re joystick will need to be calibrated. When

the system restarts go into the control panel and select “Joystick calibration”.

Move the joystick as far as possible in all directions until the green crosshairs

are basically centered. Click OK to write the new calibration info

to the registry. You are now ready to use the joystick.

Overview

The driver(s) implements the functionality for the following Win32 Multimedia joystick

API calls;

joyGetDevCaps - Queries a joystick to determine its capabilities

joyGetNumDevs - Queries the joystick driver for the number of joysticks it supports.

joyGetPos - Queries a joystick for its position and button status

joySetThreshold - Sets the movement threshold of a joystick.

joyGetThreshold - Queries a joystick for its current movement threshold.

joySetCapture - Captures a joystick my causing its messages to be sent to the specified window.

joyReleaseCapture - Release the specified captured joystick.

Dual joysticks each with two buttons are supported from a single Y split joystick port.

Module descriptions

JOYSTICK.DLL - Win32 subsystem driver that is called by WINMM.DLL and implements the required functionality of the joystick API. This module calls JOYIO.SYS to read the joystick values through the CreateFile/ReadFile API using a symbolic link (\device\joyio).

JOYSTICK.CPL - Control panel applet is used to calibrate the joystick and write the calibration values to the registry for the Win32 subsystems driver’s use. Joysticks return different values based on the hardware implemented, these physical values have to be transformed into logical (and consistent) values that joyGetPos() returns.

JOYIO.SYS - Kernel mode driver that is called by the Win32 based driver to provide I/O to the joystick port. JOYIO.SYS establishes a symbolic link so that it can be called via CreateFile/ReadFile calls. The driver polls the port to determine the resistive value for both the X and Y axis. It then checks the button states and returns all the information in a JOYINFO structure to the caller.

Rebuilding the drivers

- Install the Microsoft DDK

- Startup a Checked or Free build window

- cd \ddk\src\mmedia\drvlib

- Issue the command; build

- Go to one of the joystick driver directorys, either \dll or \driver

- Issue the command; build

- The build process links the module in the

\ddk\lib\i386\checked or

\ddk\lib\i386\free libraries. You should copy

you’re new module from here to the install

library that contains the OEMSETUP.INF and

reinstall the driver.

Functional description

The “Add driver” dialog in the control panel reads the OEMSETUP.INF file in the directory specified and copies JOYSTICK.DLL and JOYSTICK.CPL to the \system32 directory and JOYIO.SYS to the \system32\drivers directory. It then loads the JOYSTICK.DLL module and sends it a DRV_INSTALL message. JOYSTICK.DLL then sets up the registry entries for JOYIO to start on the next system reboot and also sets the entries that informs WINMM what the joystick win32 subsystem driver is called. When the system is restarted, JOYIO is started and the DriverEntry entry point in JOYIO.SYS is called which sets up the functions that the driver support (ReadFile), creates the device “\device\joyio’ and the symbolic link. When Windows finally gets around to loading WINMM, it goes through it’s registry info to determine which drivers to load. It will now find that there is a joystick driver to load and does so. The load of JOYSTICK.DLL causes a DRV_LOAD message to be issued and the driver configuration to be read from the registry.

The control panel applet JOYSTICK.CPL calibrates the joystick by determining the extreme ranges that the joystick is capable of. Based on this info, it calculates a X/Y base and X/Y delta that is then written to the registry (HKEY_CURRENT_USER\Software\Joystick) and used in the Win32 subsystem driver to compute logical return values for joyGetPos(). In order to inform the Win32 Subsystem driver that the calibration information has changed, the control panel applet opens the driver and sends it the message JDD_CONFIGCHANGED which informs the user mode driver to reload the information from the registry. Since the control panel applet must read the actual values of the joystick in order to calculate the base/delta values, it calls the kernel mode driver directly instead of going through WINMM.

When you make a call to joyGetPos() (for example) control is transferred to WINMM.DLL which sends a message (via SendDriverMessage) to the JOYSTICK.DLL driver via SendDriverMessage(). JOYSTICK.DLL receives control in it’s DrvProc() with a message JDD_GETPOS. It then calls the kernel mode driver to read the current joystick values and convert these physical values to logical values based on the calibration configuration information that the control panel applet has setup and placed into the registry.