MICOM switch model 600 control program for VAX.

With these programs you can communicate to the MICOM command console port

from any VAX terminal.  Temporary and permanent configuration changes can

be run for a command procedure (I find this useful for saving small

changes until I get around to blasting another CPROM and use the feature

to force permanent connects of remote spooled printers).  You can also

save $$ on the MICOM PC software/hardware.

The current version requires:

o INTERLINE STRING -- <LF><CR>                          that's 2 characters

o LAST LINE STRING -- NULL NULL NULL <LF><CR>           that's 5 characters

As shipped the interline and last line strings are ridiculous

(carriage return, 6 nulls, line feed, and 6 nulls).  Modifying

the CPROM to more "normal" (RMS) carriage control simplifies

operation (and programming).

o CONSOLE SPEED -- 9600

Our MICOM 600 has no problem keeping up with 9600 baud communications

despite the absence of XON-XOFF flow control.  The factory setting

is 1200 baud!

o Detached process -- MICOM_CONTROL

A detached process communicates with the MICOM command console and

the user process through mailboxes.  The detached process is started

with the MICOM_START command procedure.  When debugging the detached

process I suggest compiling with D_LINES and running the process

interactively.

o User process -- MICOM

The MICOM program reads keyboard input with prompt, send the MICOM

command through to the detached process, and echoes the switches

reply.  HELP facilities and in-line comments are useful features.

o Special thanks -- Jim Belonis

Jim critiqued the program and made suggestions for improvement.

His feedback was greatly appreciated.

R. Scott Peabody

Dept. of Chemistry, BG-10

University of Washington

Seattle, WA 98195

(206)543-1699

PEABODY@UWACHEM via BITNET